SA WG2 Meeting #S2-152e(e-meeting)	S2-2206865r04

17 - 26 August 2022, Electronic Meeting	(revision of S2-22xxxxx)

Source:	Qualcomm Inc.

Title:	Conclusion proposal for KI#1

Document for:	Approval

Agenda Item:	9.8

Work Item / Release:	FS_RedCap_Ph2/Rel-18

Abstract of the contribution: Introduction of conclusion to support CN handling of MT data/signalling during long eDRX in RRC Inactive.

# 2. Proposal

This paper proposes the following updates to TR 23.700-68 clause 8.

* Start of change *

# 8	Conclusions

Editor's note:	This clause will capture conclusions from the study.

## 8.1	Conclusions for Key Issue #1: Enabling long eDRX cycle support in RRC_INACTIVE state

The following aspects are concluded as principles for the normative work:

-	It is agreed that RAN decides when to apply MT data/signalling handling in CN when the UE is unreachable due to long extended DRX in RRC inactive.

-	To initiate the MT data/signalling handling in CN during unreachable time approach, the gNB sends a request to CN and provides unreachability information (e.g., eDRX values negotiated between UE and gNB for RRC_INACTIVE state) to CN (e.g., when UE entering RRC_INACTIVE state with long eDRX). When CN handles the MT data, CN applies the HLCOM functions based gNB provided unreachability information (as described in solution #6).

-	Based on the deployment scenario (UPF collocated with NG-RAN, CU-UP split, etc.), the traffic model, mobility patterns, the gNB may send the request for MT data/signalling handling in CN with unreachability information, e.g. when the UE enters RRC_INACTIVE state, or when the gNB receives MT data/signalling, or when the gNB cannot buffer any further MT data.

NOTE: RAN buffering capability enhancement to support long eDRX cycle is not supported.

-	If CN handling of MT data/MT signalling apply then the gNB notifies also CN with RRC State transition to RRC_CONNECTED.

- 	RAN paging triggered by CN in RRC_INACTIVE state may be escalated to entire Registration Area if RAN failed to page the UE in the RNA. What parameters RAN provides to the AMF in the request will be decided during the normative phase.

NOTE: further synch with RAN WGs is needed.

* End of changes *