SA WG2 Meeting #S2-152E	S2-2206906r02

17 – 26 August , 2022, E-meeting

Source:	Xiaomi

Title:	conclusion for Key Issue#3

Document for:	Approval

Agenda Item:	9.16

Work Item / Release:	5G_PIN /Rel-18

Abstract of the contribution: This paper proposes the conclusion for key Issue #3.  This paper merged S2-2206015, S2-2206483, S2-2206906 and S2-2205743

*** Start of changes ***

8	Conclusions

Editor's note:	This clause will list conclusions that have been agreed during the course of the study item activities.

## 8.3	Conclusion on Key Issue #3

The following principles are concluded for Key Issue #3 "Management of PIN and PIN Elements":

1)	PEMC and PIN AF are responsible for PINE management (e.g., add/remove the PINEs, etc.) and PIN management (e.g. create/modify/delete/activate/deactivate a PIN, etc.).

3)	PIN AF, PEMC, and UDR store information for PIN management and PINE management (e.g., PIN ID/name, PIN Element list, profile of PINE, PIN configuration).

4)	UE’s subscription data should contain PIN related information when it needs to act as PEMC or PEGC.

NOTE 1: The specific PIN related information will be decided in normative work.

5)	PIN AF may provide necessary information to 5GC to enable the PIN management.

NOTE 2: The specific information to enable the PIN management needs to be determined in conclusion of KI#6.

8)	A PEMC shall subscribe in the UDM to act as PEMC.

9)	A PEMC or a PEGC shall link with a trust member group, which includes at least one PEMC and one or more PEGCs.

10)	To support management of multiple PEMCs for a PIN, a trust member group may include multiple PEMCs.

11)	In order to make sure PEMCs and PEGCs using same PIN AF, or a PEMC using same PIN AF after recovery, the PIN AF registers itself into UDR via UDM or NEF for a trust member group.

*** End of changes ***