SA WG2 Meeting #152e	S2-2206943r07

# Aug. 17th – 26th , 2022; Elbonia

Source: 	Nokia, Nokia Shanghai Bell

Title: 	FS_eLCS_Ph3 TR Solution to KI#9

Document for: 	Approval

Agenda Item: 	9.10

Work Item / Release:	FS_eLCS_Ph3 / Rel-18

Abstract of the contribution: FS_eLCS_Ph3 TR KI#9

# 1	Discussion

The FS_eLCS_Ph3 SID objectives reads

Key Issue #9: Support of Positioning Requirements Related to Satellite Access

-	What kind of location information can represent the UE location that meets required accuracy in NR satellite access (e.g. doing verification based on the location information to meet regulatory requirements)?

-	How to further enhance the LCS to verify location services related requirements defined in TS 22.261 [8];

In Rel-17, when a UE is using NR satellite access, in order to ensure to meet the regulatory requirements, the network will verify whether the PLMN selected by the UE is allowed to operate in the country of the UE location based on the UE location information.

It is proposed to deploy an assistant UE device realized by target UE capability to solve the KI.

# 2 Proposal

It is proposed to update TR 23.700-71 as follows

## *** First change ***

## 6.0	Mapping Solutions to Key Issues

Table 6.0-1: Mapping Solutions to Key Issues

|  | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| Solutions | #9 |  |  |  |
| X | X |  |  |  |

### *** Next change ***

### 6.x	Solution #x: Use assistant UE device to support Positioning Verification Related to Satellite Access

### 6.X.1	Introduction

In Rel.17, for registration over NTN in regulatory cases, NI-LR procedure is triggered to support satellite access. The same procedures can be enhanced to perform position verification for emergency cases of the KI.

This solution defines an assistant UE device realized by target UE capability in this KI#9 with its known location can perform positioning measurements (e.g., RSTD, RSRP, UE Rx-Tx Time Difference measurements, etc.) and can report these measurements to a location server through LCS procedures defined in TS 23.273.

NOTE: This solution uses such assistant UE device realized by UE in KI#9 with known location, with same or superset of radio (NTN) and LCS capabilities.

Such assistant UE device can be deployed by operators and their measurements and location can be trusted in the bordering area.

This solution is independent from Solutions defined to resolve KI#7.

Note: Whether such assistant UE device can operate as proposed in this solution is FFS.

### 6.X.2	Functional Description

These assistant UE device can be used to verify the relative position of the target UE for the verification purpose:

Deployed assistance device instances on the border

Target area (TAI) reported by UE to be verified (to the west of deployed devices)

As above diagram shows, as long as target UE can be verified that it is located west of the deployed the assistant UE device, its reported position can also be considered verified.

Satellite

LMF can performs verification with measurements from assistant UE device  & UE

Particularly, for absolute area verification of target UE, this area can be enclosed with these assistant UE devices with appropriate density.

When UE reports its location within the area surrounded by these assistant UE device instances, verification can be carried out that all UL/DL measurements of the UE should be within the measurements’ ranges of surrounding these assistant UE device instances.

For example, when LMF instructs gNB, these assistant UE devices and target UE to perform RTT measurements at (roughly) same time, the RTT measurements of target UE should be within the measurements of surrounding assistant UE devices.

NOTE: detailed deployment density or topology of such assistant UE devices depends on the shape of the area or extend of the border, it is implementation specific.

It is reasonable to treat the area where UE reports to reside in should be smaller/within the NTN satellite serving area. Otherwise, cell coverage from ephemeris of NTN satellite is already enough for the verification.

assistant device used to verify UE in its reported area should be in same NTN satellite coverage

The ephermeris of NTN satellites and areas which require verification are static or periodically repeated information. The assistant UE device(s) used to verify UE location to distinguish the different areas of emergency or lawful interception could use the same NTN satellite for comparable positioning measurements.

### 6.X.3	Procedures

LMF takes verification request and responds the result

For either NI-LR of emergency cases or MT-LR of lawful interception, Nlmf_Location_DetermineLocation can take the UE reported location and TAI as input parameter and performs the verification.

Verification result can be used by AMF in NI-LR or used by LI Client through AMF/GMLC/NEF.

Step 3.1 and 3.3 may not occur in the order shown in the diagram. For instance, when LMF prefers to derive the RSTD window assistance for target UE from nearby assistant UE device(s) measurements, step 3.3 may occur before step 3.1.

Editor’s Note: whether steps 3.1, 3.2 and 3.3. can be supported are FFS.

Step 3.1 and 3.3 may also occur concurrently in mixed mode. For instance, LMF can align the capability and position methods of such assistant UE device(s) and target UE, then request the location information or measurements (like LPP RequestLocationInformation) at the same time. It can ensure the NTN satellite in roughly same position when such assistant UE device and target UE are performing distance related measurements.

NOTE:	The Target UE uses NTN and assume to operate using GNSS capability.

Editor’s Note: How the LMF aligns the capability and positioning methods of the assistant UE devices and target UE is FFS. How the LMF synchronizes, and at which accuracy, the request for location information and measurements is FFS.

NOTE: such assistant UE devices could also have GNSS capability, GNSS assistance of search window (like codePhaseSearchWindow) of target UE can also be derived from these assistant UE devices’ measurements in step 3.3 before step 3.1.

Step 3.4: LMF performs the verification based on measurements reported by these assistant UE device (s) and target UE.

Editor’s Note: sequence of steps with detail explanation of each step are FFS.

### 6.X.4	Impacts on services, entities, and interfaces

Editor’s Note: Detailed impacts are FFS

# *** End of changes ***