3GPP TSG-WG SA2 Meeting #154-AH-e 	S2-2300642

Elbonia, January 16 – 20, 2023	(revision of S2-220xxxx)

Source:	Huawei, HiSilicon

Title:	KI#2, update conclusion on KI#2 about TNGF selection

Document for:	Approval

Agenda Item:	9.20.1

Work Item / Release:	FS_5WWC_Ph2 / Rel-18

Abstract: The conclusion on KI#2 in terms of trusted N3GPP access is updated according to the latest revised WID of 5WWC_Ph2.

1. Introduction/Discussion

There is an Editor’s Note in the conclusion on KI#2.

Editor's note:	It is FFS whether TNGF ID is provided as part of TNAN information and the UE needs to use it to build the target NAI.

Based on the revised WID of 5WWC_Ph2 (SP-221347) approved in SP#98e, the conclusion of TNGF selection shall be updated correspondingly to capture different deployment options for trusted non-3GPP access network. Therefore, the EN can be deleted based on the updated conclusion.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-17.

# * * * * First change * * * *

8.2.2	How to select a TNGF that supports the S-NSSAI(s) needed by the UE

Selection of a TNGF that supports the S-NSSAIs needed by the UE is enabled based on following assumptions:

-	the following deployments are supported

-	A TNGF can be reach over one single SSID or multiple SSIDs where the TNGF may support a different set of slices

-	An SSID may provide access to one or more TNGF(s), where each of these TNGF(s) may support a different set of slices

-	UE(s) may use WLAN SP policy to select a SSID allowing to access to a TNGF that supports the slices the UE intends to use and then leverage this information to try registering onto 5GC via a specific TNGF that supports those slices. UE may indicate that a specific TNGF is to be used by including a TNGF ID in the NAI's realm.

-	The WLAN SP policy is extended with information on which SSID to use to reach a specific TNGF identified by TNGF ID.

-	ANDSP is extended with information which TNGF ID to use for which slices.

NOTE:	Which part of ANDSP to extend to include information as to which TNGF ID to use for which slices will be coordinated with CT1.

-	The AMF may trigger the UE Policy Association Establishment procedure to provide the UE with updated WLANSP/ANDSP if the selected TNGF does not support the slices requested by UE. The AMF requests the PCF to receive a notification when the PCF has completed the WLANSP/ANDSP update; once the AMF has received this notification from the PCF, the AMF can issue a registration reject.

-	AMF may provide target TNAN information (SSIDs, TNGF ID) associated with the Requested NSSAI to UE within Registration Reject message.

-	The UE may select an appropriate SSID associated with the Requested NSSAI and build the realm of NAI taking the TNGF ID into account (if available).

Target TNAN information in a Registration Reject message is valid only for one immediate subsequent attempt of the UE to select an SSID and access the TNGF.

No solution using the AMF redirection of the UE with a registration accept will be pursued in normative specifications.

# * * * * End of changes * * * *