3GPP TSG SA WG2 Meeting #156-E	S2-2304108r03

# Electronic meeting, April 17-21, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 4211 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to handling of LADN area per DNN and S-NSSAI |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, CATT, Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | GMEC |  |  |  |  |  | Date: |  |  | 2023-04-07 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Part of the text in 5.20b.2 is overlapping with text in 5.6.5a. It seems better to keep the AMF requirements related to LADN per DNN and S-NSSAI in one place and it is proposed to capture that fully in 5.6.5a. There are also two open issues: It is indicated that if a group is provisioned with a LADN service area, the SMF configures the DNN of the group as a LADN DNN. There is also a related Editor’s note: Editor’s note: The SMF does not need to know whether there is a LADN Service Area for a DNN/S-NSSAI, SMF does not need to know the content of the LADN Service Area. Whether this should be reflected in the data provided by UDM is FFS. The existing text does not seem fully correct: - Some subscribers of a DNN may be configured with a LADN area while other subscribers of the DNN are not. This means that the DNN cannot be configured as a LADN DNN for all users. It may also happen that the same DNN may have different LADN services area for two different groups. This depends on the Group IDs and the DNNs that the users are subscribed to. - As indicated in the EN, the SMF does not need to know the content of the LADN service area. The SMF only needs to know that a DNN/S-NSSAI is to be treated as for LADN. - The solution also needs to work if the AMF is configured with LADN area and no LADN area is received from UDM. It is therefore more suitable that AMF indicates to SMF whether the PDU Session is to be treated as for LADN. There is also an Editor’s note related to overlaps of service area: Editor’s Note: If there is no overlapping between the LADN service area configured in AMF and service area received from UDM, how to handle this case is FFS. It is not clear how AMF should act if the UE does not support LADN service area per DNN and S-NSSAI while there is a LADN area in AMF per DNN and S-NSSAI. One option is that AMF sends the intersection (smallest common denominator) of all LADN areas for a DNN and different S-NSSAIs to such a legacy UE. However, it is not clear whether there is a reasonable use case for this. It is also a problem if there is no overlap between the LADN areas (as stated by the EN). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that AMF indicates to SMF whether the PDU Session is to be treated as a LADN PDU Session. Clarify how AMF determines LADN service area. Clarify how AMF handles non-supporting UEs. Remove Editor’s note. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear requirements on AMF and SMF. Remaining Editor’s notes. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.6.5a, 5.20b.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** First Change ****

5.6.5a	Supporting LADN per DNN and S-NSSAI

The 5GS may support LADN per DNN and S-NSSAI, and the functions specified in clause 5.6.5 are used (with the extension to apply per DNN and S-NSSAI whenever applicable) with the following enhancements:

-	The UE indicates the support of LADN per DNN and S-NSSAI in the UE MM Core Network Capability of the Registration Request message.

-	The LADN service area can be provisioned for a group (e.g. 5G VN group) or an individual subscriber using UDM/NEF parameter provisioning service triggered by AF request as described in clause 4.15.6 of TS 23.502 [3].

- 	LADN service area per DNN and S-NSSAI may be configured in the AMF. The LADN service area may also be provided to AMF as part of the subscription data from UDM.

-	The LADN Service Area Information provided to the UE is determined by AMF, based on the Registration Area that the AMF assigns to the UE and either the local configured LADN service area or the subscribed LADN service area received from UDM. In case there is both a configured LADN service area and a LADN service area received from UDM, the AMF decides based on operator configuration which one takes precedence.

-	If the UE indicates support of LADN per DNN and S-NSSAI, the AMF provides the LADN Service Area Information per LADN DNN and S-NSSAI to the UE.

- 	If the UE does not indicate support of LADN per DNN and S-NSSAI but indicate support of LADN per DNN as described in clause 5.6.5, and the AMF has a LADN service area for the DNN as well as a LADN service area for the DNN and S-NSSAI, the AMF may determine the LADN Service Area Information per LADN DNN sent to UE using the LADN service area for the DNN as described in clause 5.6.5.

- 	If the UE does not indicate support of LADN per DNN and S-NSSAI but indicate support of LADN per DNN as described in clause 5.6.5, and the AMF has no LADN service area for the DNN but there is a LADN service area for the DNN and S-NSSAI, then the AMF may determine the LADN Service Area Information per LADN DNN sent to UE using this LADN service area for the DNN and S-NSSAI as the LADN service area for that DNN as described in clause 5.6.5 if the UE is not subscribed to the same DNN for multiple S-NSSAI(s) (i.e. the UE is subscribed to the DNN for a single S-NSSAI only).

NOTE: 	In order to serve the UEs that do not support LADN per DNN and S-NSSAIs but indicate support of LADN per DNN as described in clause 5.6.5, the operator can avoid using the same DNN for multiple S-NSSAIs if LADN service area is configured per DNN and S-NSSAI.

-    If the UE neither indicates support of LADN per DNN and S-NSSAI nor indicates support of LADN per DNN, or the AMF neither has a LADN service area for the DNN nor has a LADN service area for the DNN and S-NSSAI, the AMF shall not provide any LADN Information to the UE.

-	The AMF enforces the LADN Service Area per LADN DNN and S-NSSAI in the same way as is described in clause 5.6.5 with the difference that the LADN service area is defined per DNN and S-NSSAI.

-	The UE enforces the LADN Service Area per LADN DNN and S-NSSAI, if received from the AMF, in the same way as is described in clause 5.6.5 with the difference that the LADN service area is defined per DNN and S-NSSAI.

- 	If the AMF enforces the LADN Service Area per LADN DNN and S-NSSAI for the UE, the AMF indicates to the SMF during PDU Session Establishment that the PDU Session is subject to LADN per LADN DNN and S-NSSAI. If indicated by AMF at PDU Session Establishment that PDU Session is subject to LADN per LADN DNN and S-NSSAI, the SMF configures the DNN of PDU Session as LADN DNN and subscribe to "UE mobility event notification" for reporting UE presence in Area of Interest by providing LADN DNN and S-NSSAI to the AMF as described in clause 5.6.11 and 5.3.4.4.

**** Next Change ****

5.20b.2	Support LADN service area for a group

The procedure as defined in clause 4.15.6.2 of TS 23.502 [3] is applicable for provisioning of LADN service area for a group with the following clarifications and enhancements:

-	The AF request additionally contains the LADN service area as part of DNN and S-NSSAI specific Group Parameters, and the LADN service area is stored in UDR as subscription data and delivered to AMF. If the AMF receives the LADN service area for a group, the AMF configures the DNN of the group as LADN DNN.

-	If the AF provides the LADN service area in the form of geographical information, the NEF maps the geographical information to a list of TAs before sending the service area to the UDM.

LADN per DNN and S-NSSAI as defined in clause 5.6.5a is applicable for enforcement of LADN service area.

**** End of Changes ****