3GPP TSG-SA2 Meeting #156E	S2-2304373r04

# 17th – 21th April, 2023, Elbonia													   revision of S2-xxxxxxx

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 4287 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | PIN identifiers |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | PIN |  |  |  |  |  | Date: |  |  | 2023-04-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As concluded in TR 23.700-88, PIN ID needs to be supported by 5G. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a new clause to describe identifers for PIN. The 5G System already supports group identification and group membership management through External Group ID and Internal Group ID. A PIN also has exactly the same requirements that can be already fullfilled using this group management feature. It is proposed that a PIN is identified externally by a PIN ID. It is also proposed that External Group ID and Internal Group ID is used to identify and manage the PEGC UEs that are part of a PIN. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is not clear how the PIN is identified and how the PEGCs that are part of a PIN are managed in 5GC |  |  |  |  |  |  |  |  |
| Clauses affected: |  | new 5.44.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * 1st change (all text are new) * * * *

## 5.44	Support for Personal IoT Networks

### 5.44.x	PIN Identifiers

A PIN is identified by a PIN ID in the URSP rules. The PEGCs in a PIN may be managed dynamically by AF . The AF for PIN may use the External Group ID to identiy PIN to which the list of PEGC(s) that are part of.

The External Group ID is mapped to an Internal Group ID which is used to identify the PIN and it is unique in the PLMN.

# * * * * End of change * * * *