SA WG2 Meeting #156E (e-meeting)	S2-2304697r04

April 17 – 21, 2023	(revision of S2-230xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 4057 | rev | x | Current version: | 18.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | PDTQ policy renegotiation, PDTQ storage and parameter enhancement in PDTQ create request |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Vivo, OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIMLsys |  |  |  |  |  | Date: |  |  | 2023-04-03 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | This paper focuses on to solve the following issues. During the PDTQ policy create procedure, the H-PCF queries or/and subscribes to the NWDAF as defined in clause 6.6.4 or clause 6.14.4 of TS 23.288 [50] to request the Network Performance analytics and in case of non-GBR traffic type, the DN Performance analytics can be requested as well. But the output analytics in NWDAF for example, the DN performance, can not provide the analytic per ASP ID. Only the analytic statistics is per DNN, S-NSSAI and Application Identifier. So, one possible way to solve this problem is, add the DNN, S-NSSAI and Application Identifier in PDTQ policy create procedure and the PCF can use these parameters to consume the analytic from NWDAF. Only provide ASP ID is not possible to obtain the analytic from NWDAF. As for the storage of PDTQ policy in UDR, according to the section 6.1.2.7, the PDTQ policy is stored in UDR as Data Set "Policy Data" and Data Subset "PQDT data". But in TS 23.502, there is no such information storage in UDR. In order to retrieve the PDTQ policy from the UDR, that the data key should be specified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the DNN, S-NSSAI and Application Identifier in PDTQ policy create procedure and the PCF can use these parameters to consume the analytic from NWDAF. The PDTQ policy is stored in UDR with data key = PDTQ reference ID. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is unclear how to renegotiate the existing PDTQ policy, and it is impossible for PCF to request the analytic from NWDAF only by ASP ID. It is unclear how to store the PDTQ policy in UDR. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.16.15.1, 4.16.15.2.1, 5.2.12.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Change 1 * * *

#### 4.16.15.1	General

The intent of this clause is to specify generic service procedures to enable the AF to negotiate viable time window for the planned application data transfer with specific QoS requirements and operational conditions via the support of the NEF.

The PDTQ policies are defined for a specific ASP which include the respective Desired Time Windows, the optional Network Area Information, the Request for Notification if the AF accepts that the PDTQ policy can be re-negotiated using the PDTQ warning notification procedure.

The Network Performance analytics or DN Performance analytics for NWDAF as described in TS 23.288 [50] will be subscribed by the PCF to assist its decision to derive the PDTQ policies for the selected time window(s).

One or more negotiated PDTQ policies could be provided by PCF to AF via NEF together with the PDTQ Reference ID. If the AF receives more than one PDTQ policies from the PCF, the AF will select one of them and inform the PCF about the selected PDTQ policy which will then be stored in the UDR to be applied to the AF session when requested by the AF. Prior to the start of the Desired Time Window for the planned application data transfer, the AF requests the PCF to apply the selected PDTQ policy to the selected target UEs' PDU sessions. The PCF will then determine the appropriate PCC rules according to the negotiated PDTQ policy for the corresponding PDU session.

* * * Start of Change 2 * * *

##### 4.16.15.2.1	Procedures for negotiation of planned data transfer with QoS requirements

This clause describes the PDTQ procedures to negotiate viable time window for the planned application data transfer via the support of the NEF.

Figure 4.16.15.2.1-1: Negotiation for planned data transfer with QoS requirements

Prior to the transport of the Application AI/ML data, the AF negotiates with the 5G Core for the PDTQ policies to provide assistance for its application data transfer. The AF discovers its serving NEF, if it has not done so, using the mechanism as described in clause 6.3.14 of TS 23.501 [2].

1a.	The AF invokes the Nnef_PDTQNegotiation_Create Request (ASP Identifier, Number of UEs, the list of Desired time windows, QoS reference or individual QoS parameters, and optionally the alternative QoS parameters, Network Area Information, Request for notification, Alternative Service Requirements, Application Identifier). The Request for notification is an indication that PDTQ warning notification can be sent to the AF. Note that, only Network Area Information is to be used for the PDTQ negotiation.

NOTE:	Based on AF's internal logic (policy). the AF may determine the Minimum Guaranteed QoS requirements based on the UEs who are to be participated in the PDTQ transfer window and the network input data and the group application data transfer trigger conditions.

1b-1c.	The NEF may authenticate the AF and authorize the PDTQ request from the AF. If the authentication/authorization of the AF's request has failed, the NEF will respond to the AF's request through the Nnef_PDTQNegotiation_Create Response with the failure result, and the following steps are skipped.

The NEF may map the ASP ID into DNN and S-NSSAI to be used in step 2.  The NEF may refer to the Application Identifier in step 1a to be the Application ID that is used in the step 5 for subscription analytic in NWDAF.

2.	Based on an AF request, the NEF may translate the information provided by the AF (e.g. QoS reference or geographical information etc.) based on the local policy and invokes the Npcf_PDTQPolicyControl_Create (ASP Identifier, Number of UEs, the list of Desired time windows, the QoS parameters, and optionally the alternative QoS parameters, Network Area Information, Request for notification, Application Identifier) with the H-PCF to authorize the creation of the policy regarding the PDTQ. If the PCF was provided with Request for notification, then PCF will send PDTQ warning notification to the AF as specified in clause 4.16.15.2.3 to notify the AF when the network performance in the area of interest goes below the criteria set by the operator as described in clause 6.1.2.7 of TS 23.503 [20].

The PCF may be configured to map the ASP identifier to a target DNN and S-NSSAI if the NEF did not provide the DNN, S-NSSAI to the PCF.

3.	H-PCF queries the UDR to retrieve all existing PDTQ polices for the corresponding ASP using Nudr_DM_Query (Policy Data, Planned Data Transfer with QoS) service operation.

4.	The UDR provides all the stored PDTQ policies and corresponding related information (e.g. the Number of UEs, the list of Desired time windows) to the H-PCF.

5.	Based on information provided by the AF and other available information, the H-PCF queries or/and subscribes to the NWDAF as defined in clause 6.6.4 or clause 6.14.4 of TS 23.288 [50] to request the Network Performance analytics and in case of non-GBR traffic type, the DN Performance analytics can be requested as well.

When requesting the network performance or DN performance analytics, if "any UE" is used, then the AoI information is used to identify the target gNB(s) for the prediction of the availability of the network resource.

The DNN, S-NSSAI and Application ID can be consumed by H-PCF as Analytics Filter Information to queries or/and subscribes to the analytic in NWDAF.

6.	By referring to the outcome of the analytics report as described in clause 6.1.2.7 of TS 23.503 [20], H-PCF determines one or more PDTQ policies. Each PDTQ policy includes a recommended time window for the traffic transfer for each of the AF session for each of the UEs involved.

7.	The PCF sends one or more PDTQ policies to NEF in Npcf_PDTQPolicyControl_Create Response including the PDTQ Reference ID.

8.	The NEF sends a Nnef_PDTQNegotiation_Create response to the AF to provide one or more PDTQ policies together with the PDTQ Reference ID.

9.	If more than one PDTQ policies were provided to the AF, the AF selects one of the PDTQ policies and notifies NEF for the selected PDTQ policy via Nnef_PDTQNegotiated_Update together with the PDTQ Reference ID. The AF stores the PDTQ Reference ID and the selected PDTQ policy for the future interaction with the PCF. If the NEF received only one PDTQ policy from the PCF, steps 10-11 are not executed and the flow proceeds to step 12. Otherwise, the flow proceeds to step 10.

10-12.	The NEF notifies H-PCF for the selected PDTQ policy by the AF. The H-PCF acknowledges NEF. The NEF responds to the AF's request through the Nnef_PDTQNegotiation_Update Response.

13.-14.	The H-PCF stores the PDTQ Reference ID together with the new PDTQ policy in the UDR by invoking Nudr_DM_Update (PDTQ Reference id, Policy Data, Planned Data Transfer with QoS). The UDR sends a response to the H-PCF as its acknowledgement.

After the PDTQ policy negotiation, if the AF decides to select an alternative PDTQ policy (e.g. data rate reduction, relaxing delay constraints of planned and event driven traffic etc.), the AF may trigger this procedure to request the corresponding H-PCF via the support of NEF to revise the PDTQ policy. In such a case, in step 1 instead of invoking Nnef_PDTQNegotiation_Create Request the AF will invoke the Nnef_PDTQNegotiation_Update Request together with the PDTQ Reference ID. Consequently, instead of invoking Npcf_PDTQPolicyControl_Create Request the NEF will invoke Npcf_PDTQPolicyControl_Update Request together with the PDTQ Reference ID.

* * * End of Change * * *