3GPP TSG-WG SA2 Meeting #157	S2-2306910

# Berlin, Germany, May 22 – 26, 2023		(revision of S2-2306099)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0773 | rev | 5 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates for Nnwdaf_MLModelTraining service |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | [Vivo, NTT DOCOMO, Ericsson], Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph3 |  |  |  |  |  | Date: |  |  | 2023-05-12 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | First, one EN about FL process is left from last meeting: Editor's note: It is FFS whether and what and how (i.e. using which service) Training Input Data Information is provided from the Client NWDAF to the Server NWDAF. This paper proposes to use Nnwdaf_MLModelTraining_Notify service to address this EN. Second, during the FL process, if the FL client determines to leave the FL process, it is not clear which service to use. This paper clarifies explicitly the Nnwdaf_MLModelTraining_Notify service should be used in this scenario. Rev5: 1. According to the description in step 0 and step 6a of clause 6.2C.2.2, the Accuracy reporting interval and pre-determined status (ML model Accuracy threshold and Total training time) are used to indicate the reporting conditions, so these two parameters should be added to the Training Reporting Information in clause 6.2F.2. 2. The ML model metric should be the input parameter of Nnwdaf_MLModelTraining_Subscribe service operation based on the description in step 0 of clause 6.2C.2.2. In addition, the consumer may decide to stop the training process as described in step 6b of clause 6.2C.2.2, so a Termination Request that indicates the FL Server NWDAF to terminate the ML model training should also be added as the input parameter of Nnwdaf_MLModelTraining_Subscribe service operation. 3. To align with step 4 of clause 6.2C.2.2, the “Accuracy of local model” in the input of Nnwdaf_MLModelTraining_Notify service operation is changed to “local ML model metric (e.g., ML model Accuracy/Precision/Recall)”. 4. It’s unclear what parameter should be included in the Nnwdaf_MLModelTraining_Notify when the FL Client NWDAF is not able to complete the training of the interim local ML model within the maximum response time provided by the FL Server NWDAF. It proposes that the FL Client NWDAF shall send the Delay Event Information to the FL Server NWDAF. The Delay Event Information includes a delay event indication with a cause code (e.g. local ML model training failure, more time necessary for local ML model training) that indicates the FL Client NWDAF is not able to complete the training of the interim local ML model within the maximum response time, and the expected time to complete the training if available. 5. As described in step 6a of clause 6.2C.2.2, the FL Server NWDAF may send a Nnwdaf_MLModelProvision_Notify message to update the global ML model metric to the consumer. Therefore, the global ML model metric that is generated by the FL Server NWDAF should be added to the input of Nnwdaf_MLModelTraining_Notify service operation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Clairy the FL client NWDAF can use Nnwdaf_MLModelTraining_Notify service to report status of FL training; - Clarify the FL client NWDAF can use Nnwdaf_MLModelTraining_Notify service to leave the FL process; Rev5: 1. Add the Accuracy reporting interval and pre-determined status (ML model Accuracy threshold and Total training time) to the Training Reporting Information in clause 6.2F.2. 2. Add ML model metric and Termination Request that indicates the FL Server NWDAF to terminate the ML model training to the input of Nnwdaf_MLModelTraining_Subscribe service operation. 3. Change the “Accuracy of local model” in the input of Nnwdaf_MLModelTraining_Notify service operation to the “local ML model metric (e.g., ML model Accuracy/Precision/Recall)”. 4. Add the Delay Event Information that includes a delay event indication with a cause code and the optional expected time to complete the training to the input of Nnwdaf_MLModelTraining_Notify service operation. 5. Add the global ML model metric that is generated by the FL Server NWDAF to the input of Nnwdaf_MLModelTraining_Notify service operation. 6. Other editorial changes. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is not clear which service is used by the FL client NWDAF to report status of FL training or leave the FL process. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2F.1, 6.2F.2, 7.10.1, 7.10.2, 7.10.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

## 6.2F	Procedure for ML Model Training

### 6.2F.1	ML Model Training Subscribe/Unsubscribe/Notify/Update

The procedure in Figure 6.2F.1-1 is used by an NWDAF service consumer, i.e. an NWDAF containing MTLF to subscribe to another NWDAF, i.e. an NWDAF containing MTLF, for a trained ML model based on the ML model provided by the service consumer NWDAF. The service may be used by an NWDAF containing MTLF to enable e.g. Federated Learning or to update ML model. The service is also used by an NWDAF to request an NWDAF containing MTLF to prepare training ML model or modify existing ML Model training subscription.

Editor's note:	Procedure for Nnwdaf_MLModelTrainingInfo service will be added in the future.

Figure 6.2F.1-1: Procedure for ML Model Training subscribe/unsubscribe/notify/update

1.	The NWDAF service consumer may subscribe or unsubscribe or modify a subscription for training an ML model by invoking the Nnwdaf_MLModelTraining_Subscribe/ Nnwdaf_MLModelTraining_Unsubscribe service operation. The parameters that can be provided by the NWDAF service consumer are listed in clause 6.2F.2.

In order to enable Federated Learning, NWDAF Service consumer act as FL Server NWDAF can subscribe to multiple NWDAFs containing MTLF act as FL Client NWDAFs, which are selected by the FL Server NWDAF.

The FL server NWDAF may use the request to check if an NWDAF can meet the ML model training requirement (e.g. Interoperability information, Analytics ID, Service Area/DNAI, and/or availability of data and time). In such case, the FL server NWDAF includes an ML Preparation Flag. When the ML Preparation Flag presents in the request, the service provider NWDAF only checks if it can meet the ML model training requirement and / or can successfully download the model if the model information is provided.

The FL server NWDAF may use the request to get the Model Accuracy of the global ML Model calculated by the FL Client NWDAFs. In such cases, the service consumer NWDAF includes a Model Accuracy Check Flag. When the Model Accuracy Check Flag is present in the request, the service provider NWDAF uses the local training data as the testing dataset to calculate the Model Accuracy of the ML model provided by the service consumer NWDAF.

Editor's note:	It is FFS for the details of ML model training requirement.

When NWDAF service consumer determine to further update the ML model, NWDAF service consumer modifies the subscription by invoking Nnwdaf_MLModelTraining_Subscribe service operation including Subscription Correlation ID with ML Model Information (i.e. file address (e.g. URL or FQDN) of the ML Model that needs to update).

2.	The NWDAF containing MTLF trains ML model provided at step 2 by collecting new data or re-use the data that it owns.

3.	When the NWDAF containing MTLF completes ML model training, the NWDAF containing MTLF notifies the NWDAF service consumer with ML Model Information (i.e. file address (e.g. URL or FQDN) of updated ML Model) by invoking the Nnwdaf_MLModelTraining_Notify service operation. The parameters that can be provided by the NWDAF containing MTLF as service provider is specified in clause 6.2F.2.

If the NWDAF containing MTLF determines to terminate the ML model training, i.e. NWDAF containing MTLF will not provide further notifications related to this request, then the NWDAF containing MTLF may notify the NWDAF Service consumer a Terminate Request indication with cause code (e.g. NWDAF overload, not available for the FL process anymore, etc.) by invoking the Nnwdaf_MLModelTraining_Notify service operation.

In order to enable Federated Learning, NWDAF containing MTLF act as FL Client NWDAF can notify NWDAF Service consumer act as FL Server NWDAF the local ML model information and status report of FL training including accuracy of local model and Training Input Data Information (e.g. areas covered by the data set, sampling ratio, maximum/minimum of value of each dimension, etc.).

If the Model Accuracy Check Flag is present in the Nnwdaf_MLModelTraining_Subscribe, the service provider NWDAF acting as FL Client NWDAF may notify the NWDAF Service consumer acting as FL Server NWDAF the Model Accuracy of the global ML Model.

# * * * * Next change * * * *

### 6.2F.2	Contents of ML Model Training

The consumers of the ML model training services (i.e. an NWDAF containing MTLF) may provide the input parameters in Nnwdaf_MLModelTraining_Subscribe or Nnwdaf_MLModelTrainingInfo_Request service operations as listed below:

-	Analytics ID: identifies the analytics for the provided ML model is used.

-	ML Model Interoperability Information as defined in clause 6.2A.2.

-	A Notification Target Address (+ Notification Correlation ID) as defined in TS 23.502 [3] clause 4.15.1, allowing to correlate notifications received from the NWDAF containing MTLF with the subscription.

-	[Optional] ML Model Information (address (e.g. URL or FQDN) of Model file).

-	[Optional] ML Model ID: identifies the provided ML model.

-	[Optional] ML Preparation Flag: identifies that the request is for preparing Federated Learning.

Editor’s Note: Whether and how the ML Preparation Flag can indicate the request is for executing Federated Learning is FFS.

-	[Optional] ML Model Accuracy Check Flag: identifies that the request is for using the local training data as the testing dataset to calculate the Model Accuracy of the global ML model provided by the service consumer NWDAF acting as the FL Server NWDAF.

-	[Optional] ML Correlation ID: identifies the Federated Learning procedure for training the ML model. This parameter is included when the service is used for Federated Learning.

-	[Optional] Available data requirement. This is for informing the requirement on available data for the ML model training. e.g. FL Server NWDAF sends the requirement in preparation request to a FL Client NWDAF for selecting the FL Client NWDAF which can meet the available data requirement.

Editor's note:	The details of available data requirement is FFS.

-	[optional] Availability time requirement. This is for informing the requirement on availability time for the ML model training, e.g. FL Server NWDAF sends the requirement in preparation request to FL Client NWDAF for selecting the FL Client NWDAF which is available in the required time for training ML model.

-	[Optional] Training Filter Information: enables to select which data for the ML model training is requested, e.g. S-NSSAI, Area of Interest. Parameter types in the Training Filter Information are the same as or subset of parameter types in the ML Model Filter Information which are defined in procedure 6.2A.1.

-	[Optional] Target of Training Reporting: indicates the object(s) for which data for ML model training is requested, i.e. a group of UEs or any UE (i.e. all UEs).

-	[Optional] Use case context: indicates the context of use of ML model.

-	[Optional] Training Reporting Information with the following parameters:

-	Maximum response time: indicates maximum time for waiting notifications (i.e. training results);

-	Accuracy reporting interval;

-	pre-determined status (ML model Accuracy threshold and Total training time).

Editor's note:	It is FFS whether the number of required notifications is needed.

-	[Optional] Expiry time.

The NWDAF containing MTLF provides to the consumer of the ML model training service operations as described in clause 7.10, the output information in notification as listed below:

-	The Notification Correlation Information.

-	ML Model Information (address (e.g. URL or FQDN) of Model file).

-	[Optional] ML Model ID: identifies the provisioned ML model.

-	[Optional] Model Accuracy: The model accuracy of the global ML model, which is calculate by the FL Client NWDAF using the local training data as the testing dataset.

-  [Optional] Training Input Data Information (e.g., area, sampling ratio, max/min of value of each dimension of data), etc.

NOTE: The parameters in Training Input Data Information are up to the implementation.

-	[Optional] ML Correlation ID. This parameter may be included when the service is used for Federated Learning.

# * * * * Next change * * * *

## 7.10	Nnwdaf_MLModelTraining Service

### 7.10.1	General

Service Description: This service enables the consumer to subscribe/unsubscribe/notify/modify for ML model training.

NOTE:	In this release of the specification, the service provider and consumer are limited to NWDAF containing MTLF.

When used for Federated Learning, this service enables FL server NWDAF to enable Federated Learning while providing global ML model information to FL Client NWDAF and getting local ML model information and status report of FL training as defined in clause 6.2C.2.3 from the FL Client NWDAF.

# * * * * Next change * * * *

7.10.2	Nnwdaf_MLModelTraining_Subscribe service operation

Service operation name: Nnwdaf_MLModelTraining_Subscribe

Description: Subscribes to NWDAF ML model training with specific parameters.

Inputs, Required:

-	Analytics ID as defined in Table 7.1-2;

-	Interoperability information;

-	Notification Target Address (+ Notification Correlation ID);

Inputs, Optional:

-	ML Model ID: identifies the provided ML model.

-	ML Model Information (i.e, file address (e.g. URL or FQDN) of ML Model that needs to update);

-	Subscription Correlation ID (in the case of modification of the ML Model Training subscription);

-	ML Training Information, i.e. data availability requirement, time availability requirement.

-	ML model metric (e.g., ML model Accuracy/Precision/Recall);

-	Termination Request that indicates the FL Server NWDAF to terminate the ML model training;

-	ML Preparation Flag;

-	ML Model Accuracy Check Flag;

-	ML Correlation ID;

-	Training Filter Information;

-	Target of Training Reporting;

-	Training Reporting Information as defined in clause 6.2F.2;

-	Use case context;

-	Expiry time.

Outputs Required: When the request is accepted: Subscription Correlation ID (required for management of this subscription). When the request is not accepted, an error response with cause code (e.g. NWDAF does not meet the ML training requirements).

NOTE:	The detail reasons in the cause code are up to Stage 3.

Outputs, Optional: ML Correlation ID (e.g. confirm of the subscription for this FL procedure).

# * * * * Next change * * * *

### 7.10.4	Nnwdaf_MLModelTraining_Notify service operation

Service operation name: Nnwdaf_MLModelTraining_Notify

Description: NWDAF notifies the consumer instance of the trained ML model that has subscribed to the specific NWDAF service. The NWDAF can also use this service to indicate to consumer it will terminate the ML model training.

Inputs, Required:

-	Notification Correlation Information: this parameter indicates the Notification Correlation ID that has been assigned by the consumer during ML model training;

-	Set of the tuple (Analytics ID, ML model Information (i.e. file address (e.g. URL or FQDN) of updated ML Model).

Inputs, Optional:

-	ML Correlation ID, when for Federated Learning;

-	Corresponding Use case context;

-	Termination Request: this parameter indicates that NWDAF requests to terminate the ML model training, i.e. NWDAF will not provide further notifications related to this request, with cause code (e.g. NWDAF overload, not available for the FL process anymore, etc.);

-	ML Model ID: this parameter identifies the provisioned ML model;

-	ML Model Accuracy: The model accuracy of the global ML model, which is calculate by the FL Client NWDAF using the local training data as the testing dataset;

-	Status report of FL training: local ML model metric (e.g., ML model Accuracy/Precision/Recall) and Training Input Data Information (e.g. areas covered by the data set, sampling ratio, maximum/minimum of value of each dimension of data, etc.), which are generated by the FL Client NWDAF during FL procedure;

-	Delay Event Information with the following parameters:

-	delay event indication with cause code (e.g. local ML model training failure, more time necessary for local ML model training, etc.): this parameter indicates that the FL Client NWDAF is not able to complete the training of the interim local ML model within the maximum response time provided by the consumer.

-	[OPTIONAL] the expected time to complete the training.

-	global ML model metric that is generated by the FL Server NWDAF.

NOTE:	The detail reasons in the cause code are up to stage 3.

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

# * * * * End of changes * * * *