3GPP TSG SA WG2 Meeting #157	S2-2307138

# Berlin, Germany, May 22-26, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1330 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update on service description of IMS AS according to LS S2-2306307 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC |  |  |  |  |  | Date: |  |  | 2023-05-12 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to replying LS (S2-2306307/S4-230672) from SA4, the service definition and description of IMS AS need to support the following scenarios in which the data channels used by multiple IMS data channel applications share a single SDP "m=" line: - Scenario 1: their remote endpoints are different ones, for example, one is the local DCMF, the other is the remote DCMF. - Scenario 2: their remote endpoints are the same one, for example, both are the peer UE. In addition, a typo needs to be corrected. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Update the service operation descritpion of IMS AS. - Update "Media Instruction" parameter to add "data channel instruction set" parameter in it to enable differentiating control on different data channel streams in one "m=" line and remove the related EN. - a typo is corrected. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | AA.2.4.2.2, AA.2.4.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### AA.2.4.2	Nimsas_SessionEventControl

#### AA.2.4.2.1	General

Service description: This service enables the consumer to be notified about session events when served IMS subscribers takes part in IMS sessions.

The following operations are available for this service:

-	Explicit subscription to receive session events. This is not specified in this Release. In this Release an implicit subscription is assumed where the IMS AS notifies a configured or discovered DSCF of a call event through a Notify.

-	Notifying IMS session control events of a specific IMS subscriber to NFs.

This service operation reports the event to the consumer that has subscribed implicitly via configuration in the IMS AS.

#### AA.2.4.2.2	Nimsas_SessionEventControl_Notify service operation

Service operation name: Nimsas_SessionEventControl_Notify

Description: This service operation enables IMS AS to notify consumers of session events related to a specific served IMS subscriber requesting use of IMS data channel media.

Inputs, Required: Session ID, Event ID,

Session ID is the identity of the IMS session for which the event relates to.

Event ID is the event triggered within the IMS session.

Inputs, Optional: Calling ID, Called ID, Session case, Event initiator, Media info list.

Calling ID is the public identity of the calling IMS subscriber. Called ID is the public identity of the called IMS Subscriber. Session case indicates if this is an originating or terminating IMS session. Event initiator indicates initiator of the event, i.e. 'served IMS subscriber' vs 'remote IMS subscriber'. Media info list includes for each media in the list:

-	media ID: uniquely identifies this media item within the list. The identity is allocated by IMS AS,

-	media specification: This depends on media type including relevant media attributes of interest to the consumer. The media specification includes the following media description attributes:

-	Media Type: Data_Channel. The elements below are derived from the SDP received by the IMS AS in a SIP INVITE or a re-INVITE related to an IMS Data Channel and the corresponding DTLS connection. This information is sent to a Data Channel AS in a P2A scenario when the Data Channel AS terminates IMS DC and the DCMF acts as UDP proxy. When the DCMF acts as an HTTP proxy terminating IMS DC, the information is passed to the DCMF.

-	Data Channel Mapping and Configuration Information: This attribute is applicable to Data Channel and includes relevant configuration Information.

-	Maximum Message Size: This attribute defines the maximum size to be expected.

-	Data Channel Port: This attribute identifies the port for the Data Channel.

-	Security Setup: This attribute identifies the security set up of the DTLS connection.

-	Security Certificate Fingerprint: This attribute identifies the security certificate fingerprint.

-	Security Transport Identity: This attribute identifies transport layer identity.

Editor's note:	It is FFS if the Stream ID needs to be added as further Input parameter.

Outputs, Required: Result indication.

Outputs, Optional: None.

The table below presents supported EventIDs and related parameters.

Table AA.2.4.2.2-1: List of events and Related Optional parameters

| EventID | Parameters |
| --- | --- |
| SessionEstablishmentRequestEvent | Calling ID, Called ID, Session case, Media info list |
| SessionEstablishmentProgressEvent | Media info list |
| SessionEstablishmentAlertingEvent | Media info list |
| SessionEstablishmentSuccessEvent | Media info list |
| SessionEstablishmentFailureEvent |  |
| MediaChangeRequestEvent. | Event initiator, Media info list |
| MediaChangeSuccessEvent | Media info list |
| MediaChangeFailureEvent | Media info list |
| SessionTerminationEvent | Session case |

# * * * * Second change * * * *

### AA.2.4.3	Nimsas_MediaControl Service

#### AA.2.4.3.1	General

Service description: This service enables the consumer to control IMS AS handling of media flow within an IMS session. The service can be used by the consumer before responding to a Nimsas_SessionEventControl_Notity request.

#### AA.2.4.3.2	Nimsas_MediaControl_MediaInstruction service operation

Service operation name: Nimsas_MediaControl_MediaInstruction

Description: This operation provides instructions to the IMS AS for all media flows a consumer wants to control based on its policies for the received IMS session event, and that may require resource reservation in media resource e.g. MF or MRF.

For the case when a specific media flow needs to be terminated in DCMF media resource (i.e. termination of a Data Channel media descriptor or a data channel stream in a data channel media descriptor offered by one of the IMS subscribers) or originated by the DCMF or MRF (i.e. origination of a data channel media descriptor or a data channel stream in a data cahnnel media descriptor to be offered towards one of the IMS subscribers), the consumer must provide a complete DCMF media specification including information required by DCMF to know how to terminate or originate the media flow.

Inputs, Required: Session ID, Media instruction set

Session ID specifies the IMS session for which the MediaInstruction operation applies.

Media instruction set includes a set of instructions for each media flow to control. Each instruction includes:

-	media ID: used by the producer (IMS AS) and the consumer for referencing purposes. The consumer reuses the media ID it received from the IMS AS for referencing the same media. This field will be null for instructions related to originating new media.

-	Media resource capability: Identify the Media Resource capabilities the Media instruction is intended for (e.g. DCMF, ARMF).

-	Media instruction: includes instructions to the producer (IMS AS) for handling the media.

It contains Data Channel instruction set, which includes a set of instructions for each data channel stream in the media flow to control. Each item includes:

-	Data Channel stream ID: Identify the data channel stream in the media flow. The consumer reuses the data channel stream ID it received from the IMS AS for referencing the same data channel stream.

-	Data Channel stream instruction: includes instructions to the producer (IMS AS) for handling the data channel stream. The following instructions are supported:

-	"TerminateDCStream": Terminate the data channel stream in the offered media descriptor of the UE in the mediaResource, i.e. this data channel stream will not be exposed to the other UE.

-	"OriginateDCStream": Originate and offer a data channel stream in the media descriptor from the mediaResource to the UE.

-	"TerminateAndOriginateDCStream": Terminate the data channel stream in the offered media flow in the mediaResource from the UE and originate the data channel stream in the same media flow from the mediaResource to the other UE, i.e. the offered data channel stream of the originating UE will be replaced by the mediaResource provided data channel stream, which is sent towards the other UE.

-	"UpdateDCStream": Update a data channel stream in the media flow of the mediaResource previously allocated by the instructions "TerminateDCStream', "OriginateDCStream", and "TerminateAndOriginateDCStream"

-	"DeleteDCStream": Delete a data channel stream in the media flow of the mediaResource previously allocated by the instructions "TerminateDCStream', "OriginateDCStream", and "TerminateAndOriginateDCStream".

-	"RejectDCStream": Remove a data channel stream in the offered media flow, i.e. the data channel stream in the offered media descriptor will be removed both from the offer sent to the remote UE and from the answer returned to the initiator of the offer.

Inputs, Conditional: None.

Inputs, Optional:

Media resource data: is an optional element of each media instruction including information intended for consumption by the media resource. For mediaResourceCapability "DCMF":

-	Media proxy configuration (HTTP or UDP) applicable to the media flow.

-	Remote MDC1/MDC2 media endpoint address.

-	The replacement HTTP URL allocated by the consumer representing the application list offered to the IMS subscriber via the MDC1 interface.

-	Data Channel Mapping and Configuration information when originating/terminating data channel media flows on the Mb interface.

Outputs, Required: operation result indication.

Outputs, Optional: media resource information set:

This set includes entries corresponding to each instruction in the received media instruction set. Each entry in the set includes the media ID (same as received or new for new media), and may include the Media resource capability 'DCMF', which includes information about the allocated resource being of interest for the consumer. Example of such media resource information is the allocated local media address for the MDC2 interface when an offer must be provided to the Data Channel Application Server.

# * * * * End of changes * * * *