3GPP TSG-WG SA2 Meeting #160-Ad Hoc-e	S2-2401707

Online, Jan 22 – 29, 2024

Source:	Huawei, HiSilicon

Title:	New KI: Control of UE's access to 5GS via a wireless access backhaul

Document for:	Approval

Agenda Item:	19.6

Work Item / Release:	FS_VMR_Ph2 / Rel-19

Abstract: One or several new key issue(s) regarding WT#1 is proposed to cover several aspects, including access control and mobility, as highlighted in the approved SID SP-231799.

- Introduction

In SA plenary #102 meeting, the new phase of vehicle-mounted relay, FS_VMR_Ph2, was agreed and in which there were three main working tasks to be studies, as the following table highlighted:

The aim of this study work is to investigate and identify potential architecture and system level enhancements to further evolve the functionalities of base station relays mounted on vehicles. Specifically, the objectives include: - WT-1: Identify gaps for supporting the architecture with a gNB onboard of a relay and the use of a PDU session for the wireless backhauling of the N2/N3 interfaces e.g. including UE access control, including mobility aspects. - WT-2: Identify whether and how to enhance the architecture to enable authorization and configuration of relay - WT-3: Identify how to enhance the architecture to enable support for UE location services (with focus on the identification of the location of the Cell of the VMR) and emergency services

2. Discussion

According to the description of the WT#1, the basic architecture of this study is not the same as the one in Rel-18. In this release, MWAB node holds full functionalities of gNB (including gNB-CU and gNB-DU), and the gNB makes use of PDU session of the co-located UE part to convey the N2/N3 messages for the UE accessing the MWAB node.

In this case, to support the architecture mentioned above, regarding the WT#1, the control of UE's access to 5GS via a wireless access backhaul need to be considered.

The following clause 2 proposes a new key issue of the KI#1 about control of UE's access to 5GS via a wireless access backhaul.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-06.

# * * * * First change* * * *

5.X	Key Issue #X: Control of UE's access to 5GS via a wireless access backhaul

This key issue is to investigate efficient control of UE access to MWAB. In particular, the following aspects should be addressed:

-	Whether and how to enhance the existing CAG mechanism to control and manage the access of a UE via MWAB.

NOTE 1:	Support of legacy UE(s) shall be considered.

NOTE 2:	Aspects related to RAN need to be coordinated with RAN WGs.

# * * * * End of changes * * * *