3GPP SA WG2 Meeting #160-Ad Hoc-e	S2-2401199

E-meeting, January 22 - 29, 2024

Source:	Nokia, Nokia Shanghai Bell, Ericsson, Huawei, HiSilicon

Title:	KI description for WT#1

Document for:	Approval

Agenda Item:	19.11

Work Item / Release:	FS_UPEAS Ph2 / Rel-19

Abstract of the contribution: This paper proposes KI description for WT#1.

# Discussion

The new approved SID titled “Study on UPF enhancement for Exposure And SBA Phase 2” (SP-231795) includes the below WT:

WT#1: Study potential enhancement on UPF selection to support selection of UPF providing a selected user plane functionalities, e.g. NAT, Packet inspection, etc.

A Rel. 19 KI is needed for this WT. The following is proposed.

# Proposal

It is proposed to incorporate the following KI for Rel. 19 UPEAS Ph2 WT#1 in the UPEAS Ph2 TR 23.700-63 v0.0.0.

Start of changes

## X.1	Key Issue #X: Selection of UPF providing a selected user plane functionality

Currently a UPF advertises the features it supports in the NF profile of UPF registered in NRF as specified in e.g. 3GPP TS 29.510 clause 6.1.6.2.13 and/or over PFCP as defined in 3GPP TS 29.244. An SMF leverages the exposed UPF information for proper UPF discovery and selection. A UPF implementation may support more optional functionalities like NAT, Packet Inspection, etc which are not fully specified by 3GPP. The following should be studied:

-	Whether there is a need and how to extend the existing UPF advertising capabilities.

-	Whether and how the input parameter(s) for UPF selection by SMF should be enhanced to support selection of UPF supporting specific user plane functionalities, e.g. NAT, Packet Inspection, etc.

End of changes