3GPP TSG-WG SA2 Meeting #161	S2-2403359

# Athens, Greece, February 26 - March 1, 2024

Source: 	China Mobile

Title: 	New Solution: Edge computing handling by local SMF

Document for: 	Approval

Agenda Item: 	19.8

Work Item / Release:	FS_EDGE_Ph3 / Rel-19

Abstract of the contribution: Propose a solution on KI#1: Enhancements for EAS (re)discovery and UPF (re)selection with reducing impact on central 5GC NFs.

1	Discussion

This paper is to propose a new solution on KI#1.

In this solution, UL CL/BP controlled by local SMF (L-SMF) is used to offload edge computing related traffic to local DN. The central SMF is not involved with EDI management, DNS message handling rules configuration or update, and UL CL/BP or L-PSA insertion, and all the above is handled by L-SMF.

2	Proposal

It is proposed to include the below changes into TR 23.700-49 v0.1.0.

FIRST CHANGE

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

| Solutions | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | ... |
| Solution x: Edge computing handling by local SMF | X |  |  |  |
| … |  |  |  |  |

NEXT CHANGE (all the text is new)

6.x	Solution #X: Edge computing handling by local SMF

6.x.1	Key Issue mapping

This solution is related to the KI#1.

6.x.2	Functional Description

The architecture of this solution is described as Figure 6.x.2-1.

AN

Neasdf

Nsmf

Nsmf

Nnef

Nudm

Npcf

Namf

UDM

EASDF

DN

L-DN

UPF (L-PSA)

PSA UPF

(C-PSA)

NEF

UPF (ULCL/BP)

PCF

L-SMF

SMF

AMF

UE

Local Network

Figure 6.x.2-1: Network Architecture for the L-SMF

In this solution, UL CL/BP controlled by local SMF (L-SMF) can offload edge computing related traffic to local DN. The L-SMF is responsible for EDI management, DNS message handling rules configuration or update, and UL CL/BP or L-PSA insertion, and central SMF is not involved for the above information.

During PDU Session Establishment procedure, SMF decides to trigger the local-SMF selection based on SM subscription data. The SMF sends the indication of L-SMF selection to AMF, as well as the selected PCF ID/address.

The tunnels between AN, ULCL/BP UPF, central PSA UPF are setup by the following: UL N3 tunnel is setup by AMF provides UL CL/BP UPF CN Tunnel Info received from L-SMF to (R)AN, DL N3 tunnel is set up by AMF provides AN Tunnel Info received from (R)AN to UL CL/BP, UL N9 tunnel between UL CL/BP UPF and central PSA UPF is setup by AMF provides PSA CN Tunnel Info received from SMF to L-SMF, DL N9 tunnel between UL CL/BP UPF and central PSA UPF is setup by AMF provides UL CL/BP UPF CN Tunnel Info received from L-SMF to SMF.

6.x.3	Procedures

ULCL/BP

UE

(R)AN

AMF

SMF

UPF

L-SMF

PCF

UDM

1. Steps 1-11 of PDU Session Establishment procedure, clause 4.3.2.2.1 of TS 23.502

2. L-SMF selection

3. Nsmf_PDUSession_CreateSMContext Request

4. SM Policy Association Establishment

5. UPF selection

6. Steps 9-14 of PDU Session Establishment procedure, clause 4.3.2.2.1 of TS 23.502

7. Steps 15-17 of PDU Session Establishment procedure, clause 4.3.2.2.1 of TS 23.502

Figure 6.x.3-1: Procedure of edge computing handling by local SMF

1.

Steps 1-11 of the UE-requested PDU Session Establishment procedure is performed as described in clause 4.3.2.2.1 of TS 23.502 [3], with the following change:

-In Step 11, the central SMF response with the indication of L-SMF selection, and selected PCF ID or PCF address to AMF.

Editor’s NOTE: How the central SMF receives the indication to trigger the L-SMF selection is FFS

2.	AMF selects the L-SMF with considering the UE location..

3.

AMF triggers another PDU session establishment and sends the PCF address or PCF ID to the L-SMF by invoking Nsmf_PDUSession_CreateSMContext Request service.

4.	L-SMF obtains offloading policy from PCF via SM Policy Association Establish procedure as described in clause 4.16.4 of TS 23.502 [3].

5.	L-SMF selects UL CL/BP and optionally L-PSA as described in clause 6.3.3 of TS 23.501 [2].

6.	Steps 9-14 of the UE-requested PDU Session Establishment procedure is performed as described in clause 4.3.2.2.1 of TS 23.502 [3], with the following changes:

- SMF is replaced by L-SMF.

- CN Tunnel Info is UL CL/BP CN Tunnel Info.

In this step, the (R)AN obtains the CN Tunnel Info of the UL CL/BP UPF.

7.	Steps 15-17 of the UE-requested PDU Session Establishment procedure is performed as described in clause 4.3.2.2.1 of TS 23.502 [3], with the following change:

- AN Tunnel Info is replaced by UL CL/BP CN Tunnel Info.

- SMF is replaced by L-SMF.

- In step 15, AMF also sends PSA CN Tunnel Info to the L-SMF.

- In step 16a, L-SMF also provides PSA CN Tunnel Info to the UL CL/BP.

Editor’s NOTE: how to support the DNS message handling in this solution is FFS.

6.x.4	Impacts on existing services, entities and interfaces

AMF:

-	L-SMF selection indicated by DNN of the PDU Session Establishment Request.

-	Providing the PCF address from Central SMF to L-SMF

-	Providing the PSA CN Tunnel Info to L-SMF.

-	Providing the UL CL/BP CN Tunnel Info to SMF.

SMF: Providing PCF address to AMF.

PCF: Providing Offloading policy to L-SMF.

Support the communication with L-SMF.

L-SMF: Support the communication with PCF

End of CHANGE