3GPP TSG-WG2 Meeting #168	S2-2502904

7 - 11 April, 2025, Gothenburg, Sweden

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 5384 | rev | - | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Removing Non-3GPP Device Connection Information from SM policy control create |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UIA_ARC |  |  |  |  |  | Date: |  |  | 2025-03-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Since providing QoS differentiation is only supported by the PDU Session Modification procedure, the Non-3GPP Device Connection Information should not be added in Npcf_SMPolicyControl_Create service. This information is provided to PCF via Npcf_SMPolicyControl_Update service when the PCRT condition “Non-3GPP Device Identifier received” is met. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following changes are proposed: - Non-3GPP Device Connection Information is removed from Npcf_SMPolicyControl_Create service. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Erroneous list of inputs for Npcf_SMPolicyControl_Create service. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.5.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

##### 5.2.5.4.2	Npcf_SMPolicyControl_Create service operation

Service operation name: Npcf_SMPolicyControl_Create.

Description: The NF Service Consumer can request the creation of a SM Policy Association and provides relevant parameters about the PDU Session to the PCF.

Inputs, Required: SUPI (or PEI in the case of emergency PDU Session without SUPI), PDU Session id, DNN, S-NSSAI and RAT Type.

Inputs, Optional: Information provided by the SMF, such as PDU Session Type, Request Type, Access Type, the IPv4 address and/or IPv6 prefix, PEI, GPSI, User Location Information, UE Time Zone, Serving Network identifier (PLMN ID, or PLMN ID and NID, see clause 5.34 of TS 23.501 [2]), Charging Characteristics information, Session-AMBR, subscribed default QoS information (5QI, 5QI Priority Level, ARP), UE support of reflective QoS (see TS 23.501 [2], clause 5.7.5.1), Number of supported packet filters for signalled QoS rules for the PDU Session (see TS 23.501 [2], clause 5.7.1.4), UE's capability to support (S)RTP Multiplexed Media Identification Information in IP Packet Filters (see clause 4.3.2.2.1), 3GPP PS Data Off status, Trace Requirements and Internal Group Identifier (see clause 5.9.7 of TS 23.501 [2]), DN Authorization Profile Index, DN authorized Session AMBR, Framed Route information (as defined in Table 5.2.3.3.1-1), MA PDU Request indication, MA PDU Network-Upgrade Allowed indication, ATSSS capabilities of the MA PDU Session, QoS constraints from the VPLMN (as defined in clause 5.7.1.11 of TS 23.501 [2]), Satellite backhaul category, list of NWDAF instance Ids (used by AMF, SMF, UPF) and corresponding Analytics ID(s), PVS IP address(es) and/or PVS FQDN(s) and Onboarding Indication in the case of ON-SNPN (see clause 5.30.2.10.4.2 of TS 23.501 [2]), URSP rule enforcement that including Connection Capability, PCF binding information (address(es) of PCF for UE, instance id of PCF for UE), HR-SBO support indication (see clause 6.2.1.2 of TS 23.503 [20]), Alternative S-NSSAI (see clause 5.15.19 of TS 23.501 [2]), URSP delivery in EPS support indication, Local Offloading Management indication.

NOTE 1:	If SMF receives the DN authorized Session AMBR from the DN-AAA at PDU Session establishment, it includes the DN authorized Session AMBR within the Session-AMBR, instead of the subscribed Session-AMBR received from the UDM, in the request.

NOTE 2:	It is up to stage 3 to determine whether the corresponding supportedFeature in Npcf_SMPolicyControl can be reused as URSP delivery in EPS support indication.

NOTE 3:	When Local Offloading Management is allowed, SMF provides Local Offloading Management indication to PCF.

W-5GAN specific PDU Session information provided by the SMF is specified in TS 23.316 [53].

Outputs, Required: SM Policy Association ID defined in TS 29.512 [57]. Success or Failure.

Outputs, Optional: Policy information for the PDU Session as defined in TS 23.503 [20] and Policy Control Request Trigger(s) of SM Policy Association as defined in clause 6.1.3.5 of TS 23.503 [20].

See clause 5.8.2.2 of TS 23.501 [2] for allocation of IPv4 address and IPv6 prefix. The IPv6 prefix length is /64, or is shorter than /64 when Prefix Delegation applies.

See clause 4.16.4 for the detail usage of this service operation.

See clauses 4.22.2.1 and 4.22.3 for detailed usage of this service operation for ATSSS.

# * * * * End of changes * * * *