3GPP TSG-WG2 Meeting #168	S2-2502966

# Göteborg, Sweden, April 07-11, 2025

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.256 | CR | 0194 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Instructing a UAV to stop the altitude reporting |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UAS_Ph3, TEI19 |  |  |  |  |  | Date: |  |  | 2025-03-28 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Altitude reporting for UAV UEs can be triggered as the result of in-flight planning procedure or USS changeover procedure. However, it is not clear how the altitude reporting is stopped in cases, for instance, when the UAV UE reaches the destination point or when the 5GC needs to change the derived altitude reporting thresholds. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding new clause for stop of UAV UE altitude reporting |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing the part of the required functionality |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.16.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

>>>> Start of Changes <<<<

### 5.16.X	Stop altitude reporting for aerial UEs

Procedure shown in Figure 5.16.X-1 specifies how the 5GS instructs the aerial UEs to stop their altitude reporting due to a trigger from NG-RAN and/or a request from the USS.

UAS NF/ NEF

USS

NFs

AMF

NG-RAN

UAV

0. UAV UE’s altitude reporting as specified in clause 5.16.2 and 5.16.3

1. An event of failing to provide an altitude report to the NG-RAN or receive by the NG-RAN

Option A: Application layer

2a. Informing the USS about an issue to provide the altitude information reporting as per request from the USS

3a. Updating or instructing the UAV to stop the altitude reporting

4a. Nnef_UAVFlightAssistance_Update/Delete

Option B: Node-level signalling

2b. N2 message

3b. Namf_EventExposure_Notify

4b. Nnef_UAVFlightAssistance_Notify

5b. Nnef_UAVFlightAssistance_Update/Delete

6b. Namf_EventExposure_Unsubscribe

7b. N2 message

8b. Instructing/configuring UEs to stop the altitude reporting

Figure 5.X.1-1: Procedure for instructing aerial UEs to stop the altitude reporting

0.	The UAV UE is requested to perform altitude information reporting as specified in clause 5.16.2, and the UAV UE performs its altitude information reporting as specified in clause 5.16.3.

1.	In case the UAV UE fails to report the altitude information and/or the NG-RAN to receive the altitude information, the UAS NF/NEF or the USS may decide to stop/update the UAV UE’s altitude reporting.

NOTE X1:	The UAS NF/ NEF or the serving USS can also decide to stop UAV UE’s altitude reporting in cases when the UAV UE deviates from the assigned flight trajectory, which can be detected during the in-flight monitoring (clause 5.12.3) or the USS changeover (clause 5.13.2), or when the DAA procedure is triggered to avoid a collision (clause 5.14).

Option A: Application layer.

2a.	The UAV UE informs the USS over the application layer about an issue to provide altitude information reporting as requested/configured earlier.

3a.	The USS decides either to update the altitude reporting configuration at the UAV UE or to cancel the altitude information reporting. The respective request is sent by the USS to the UAV UE over the application layer.

4a. The USS may invoke the Nnef_UAVFlightAssistance_Update/Delete service operation to inform the UAS NF/ NEF about the altitude reporting of the UAV UE and/or to update/delete the UAV UE’s context.

Option B: Node-level signalling.

2b.	The NG-RAN node sends an N2 message to the AMF with information about an issue/event with the configured altitude information reporting.

3b.	The AMF invokes the Namf_EventExposure_Notify service operation towards UAS NF/NEF to inform about an issue/event with the configured altitude information reporting.

4b.	The UAS NF / NEF may decide to invoke the Nnef_UAVAssistance_Notify service operation to inform the USS about the altitude information reporting event. The UAS NF / NEF may also skipp this step in cases the UAS NF / NEF determines there is no longer a need for altitude information reporting from the UAV UE, for instance, when the UAV UE has reached the flight’s destination; in such a case, the procedure proceeds with step 6b.

5b.	Based on the Nnef_UAVAssistance_Notify request from the UAS NF/NEF (Step 4b) or USS’s own decision (otside the 3GPP scope), the USS may request the 5GS to update/stop UAV UE’s altitude reporting.

The USS invokes the Nnef_UAVFlightAssistance_Update/Delete service operation with the indication to update/stop the altitude information reporting for this UAV UE.

6b.	The UAS NF / NEF invokes the Namf_EventExposure_Unsubscribe service operation to the AMF to unsubscribe from the altitude information reports; for this purpose, the UAS NF /NEF may include values of the altitude thresholds and/or periodicity, if applicable, indicating to stop the altitude information reporting.

NOTE X2:	The UAS NF /NEF can indicate the AMF to stop altitude information reporting, for example, by setting the altitude thresolds and/or reporting periodicity to some maximum values; the exact details and attributes is expected to be defined by Stage 3.

If the USS decides to change altitude reporting information (e.g. to request new altitude reporting thresholds and/or reporting periodicity) and includes new altitude reporting thresholds and/or reporting periodicity values in the Nnef_UAVFlightAssistance_Update request in Step 5b, the AMF needs to send a new Namf_EventExposure_Subscribe request, see Steps 3b-5b in clause 5.16.2.

7b-8b.	The AMF sends the corresponding N2 message to the NG-RAN node, and the NG-RAN instructs the UAV UE to stop the altitude reporting using the existing procedures.

NOTE X3:	Aspects related to AMF informing NG-RAN regarding stop of UAV UE’s altitude reporting are specified in TS 38.413 [24].

>>>> End of Changes <<<<