3GPP TSG-SA2 Meeting #168	S2-2503052

# Goteborg, Sweden, 07 – 11April, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0547 | rev | - | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Policy/Parameter provisioning to support PWS for 5G ProSe multi-hop U2N Relay |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph3 |  |  |  |  |  | Date: |  |  | 2025-04-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The description of PWS support in clause 5.18 includes PWS support over multi-hop UE-to-Network Relays. But the PWS parameters are missing in the Policy/Parameter provisioning for 5G ProSe multi-hop UE-to-Network Relay. Additionally, RAN2 has agreed to Re-use UuMessageTransferSidelink as the PC5-RRC message transmitted by the Last relay or by the intermediate relay UE that provides SI to the child UE. Therefore, the SIB 6/7/8 can be forwarded to the intermediate U2N relay UE via PC5-RRC message, and the intermediate U2N relay UE will broadcast the warning message by using a configured Destination Layer-2 ID(s). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add PWS parameters in the Policy/Parameter provisioning for 5G ProSe multi-hop UE-to-Network Relay. And how to use the configured parameters for broadcasting warning messasge is clarified. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | PWS is not supported over multi-hop UE-to-Network Relays. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.4.1a, 5.18 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * 1st change* * * *

5.1.4.1a	Policy/Parameter provisioning for 5G ProSe multi-hop UE-to-Network Relay

The following information is provisioned in the UE in support of the UE assuming the role of a 5G ProSe Layer-3 UE-to-Network Relay supporting 5G ProSe Layer-3 multi-hop UE-to-Network Relay:

1)	Authorisation policy for acting as a 5G ProSe Layer-3 UE-to-Network Relay supporting 5G ProSe Layer-3 multi-hop UE-to-Network Relay when "served by NG-RAN":

-	PLMNs in which the UE is authorized to relay traffic for 5G ProSe Layer-3 Remote UEs supporting 5G ProSe Layer-3 multi-hop UE-to-Network Relay.

The authorisation for a UE to act as a 5G ProSe Layer-3 UE-to-Network Relay supporting 5G ProSe Layer-3 multi-hop UE-to-Network Relay also authorizes the use of 5G ProSe Layer-3 multi-hop UE-to-Network Relay Discovery with Model A and Model B.

NOTE 1:	It is up to UE and application implementation to select a discovery model or whether to perform both models simultaneously.

2)	ProSe Relay Discovery policy/parameters for 5G ProSe Layer-3 UE-to-Network Relay supporting 5G ProSe Layer-3 multi-hop UE-to-Network Relay:

-	Includes the parameters that enable the UE to perform 5G ProSe Layer-3 multi-hop UE-to-Network Relay Discovery when provided by PCF or provisioned in the ME or configured in the UICC:

-	5G ProSe Layer-3 multi-hop UE-to-Network Relay Discovery parameters (User Info ID, Relay Service Code(s), UE-to-Network Relay Layer Indicator per RSC, multi-hop indicator per RSC, maximum number of hops per RSC). The UE-to-Network Relay Layer Indicator indicates whether the associated RSC is offering 5G ProSe Layer-2 or Layer-3 UE-to-Network Relay service. RSC dedicated for emergency service may also be provisioned. The multi-hop indicator indicates whether the associated RSC can be supported for 5G ProSe multi-hop UE-to-Network Relay operation. The maximum number of hops per RSC indicates the maximum number of hops can be supported for the associated RSC.

NOTE 2:	The RSC used for multi-hop service is different from the RSC used for single-hop service.

-	Default Destination Layer-2 ID(s) for sending Relay Discovery Announcement and Relay Discovery Additional Information messages and receiving Relay Discovery Solicitation messages;

-	For 5G ProSe Layer-3 UE-to-Network Relay supporting 5G ProSe Layer-3 multi-hop UE-to-Network Relay, the PDU Session parameters (PDU Session type, DNN, SSC Mode, S-NSSAI, Access Type Preference) to be used for the relayed traffic for each ProSe Relay Service Code;

NOTE 3:	5G ProSe Relay Discovery policy/parameters can be provided from ProSe Application Server to the 5G ProSe UE-to-Network Relay supporting 5G ProSe Layer-3 multi-hop UE-to-Network Relay.

3)	For 5G ProSe Layer-3 UE-to-Network Relay supporting 5G ProSe Layer-3 multi-hop UE-to-Network Relay to relay Ethernet or Unstructured traffic from 5G ProSe Layer-3 Remote UE supporting 5G ProSe Layer-3 multi-hop UE-to-Network Relay by using IP type PDU Session:

-	Mapping of ProSe Service(s) to ProSe Application Server address information (consisting of IP address/FQDN and transport layer port number).

4)	Parameters to broadcast warning messages:

-	Configured Destination Layer-2 ID(s);

-	PC5 QoS parameters defined in clause 5.6.1;

-	the NR Tx Profile based on the configuration as specified in clause 5.1.3.1.

NOTE x:	It is up to inter operator alignment and network configuration whether a common Destination Layer-2 ID or IDs are used for all UEs cross PLMNs in a particular region for Public Warning Notification Relaying.

5)	Validity time indicating the expiration time of the Policy/Parameter for 5G ProSe Layer-3 multi-hop UE-to-Network Relay discovery and communication.

The following information is provisioned in the UE in support of the UE assuming the role of a 5G ProSe Layer-3 Remote UE supporting 5G ProSe Layer-3 multi-hop UE-to-Network Relay and thereby enabling the use of a 5G ProSe Layer-3 UE-to-Network Relay supporting 5G ProSe Layer-3 multi-hop UE-to-Network Relay:

1)	Authorisation policy for using a 5G ProSe Layer-3 UE-to-Network Relay supporting 5G ProSe Layer-3 multi-hop UE-to-Network Relay:

-	For 5G ProSe Layer-3 Remote UE supporting 5G ProSe Layer-3 multi-hop UE-to-Network Relay, indicates whether the UE is authorised to use a 5G ProSe Layer-3 UE-to-Network Relay supporting 5G ProSe Layer-3 multi-hop UE-to-Network Relay.

The authorisation for a UE to act as a 5G ProSe Layer-3 Remote UE supporting 5G ProSe Layer-3 multi-hop UE-to-Network Relay also authorizes the use of 5G ProSe Layer-3 multi-hop UE-to-Network Relay discovery with Model A and Model B.

NOTE 4:	It is up to UE and application implementation to select a discovery model or whether to perform both models simultaneously.

2)	Policy/parameters for 5G ProSe Layer-3 Remote UE supporting 5G ProSe Layer-3 multi-hop UE-to-Network Relay:

-	Includes the parameters for 5G ProSe Relay Discovery and for enabling the UE to connect to the 5G ProSe UE-to-Network Relay supporting 5G ProSe Layer-3 multi-hop UE-to-Network Relay after discovery when provided by PCF or provisioned in the ME or configured in the UICC:

-	5G ProSe Layer-3 multi-hop UE-to-Network Relay Discovery parameters (User Info ID, Relay Service Code(s), UE-to-Network Relay Layer indicator per RSC, multi-hop indicator per RSC, maximum number of hops per RSC). The UE-to-Network Relay Layer Indicator indicates whether the associated RSC is offering 5G ProSe Layer-2 or Layer-3 UE-to-Network Relay service. RSC(s) dedicated for emergency service may be provisioned to enable the support of emergency services for multi-hop UE-to-Network Relaying. The multi-hop indicator indicates whether the associated RSC can be supported for 5G ProSe multi-hop UE-to-Network Relay operation. The maximum number of hops per RSC indicates the maximum number of hops can be supported for the associated RSC.

NOTE 5:	The RSC used for multi-hop service is different from the RSC used for single-hop service.

-	Default Destination Layer-2 ID(s) for sending Relay Discovery Solicitation messages and receiving Relay Discovery Announcement and Relay Discovery Additional Information messages;

-	For 5G ProSe Layer-3 Remote UE supporting 5G ProSe Layer-3 multi-hop UE-to-Network Relay, the PDU Session parameters (PDU Session type, DNN, SSC Mode, S-NSSAI, Access Type Preference) to be used for the relayed traffic without using N3IWF access, or an indication of N3IWF access, for each ProSe Relay Service Code;

-	For 5G ProSe Layer-3 Remote UE supporting 5G ProSe Layer-3 multi-hop UE-to-Network Relay, optionally the ProSe application Traffic Descriptor(s) (as defined in TS 23.503 [9]) to be used for the relayed traffic for each ProSe Relay Service Code;

3)	Policy/parameters for N3IWF selection for 5G ProSe Layer-3 Remote UE supporting 5G ProSe Layer-3 multi-hop UE-to-Network Relay:

-	N3IWF identifier configuration for 5G ProSe Layer-3 Remote UE supporting 5G ProSe Layer-3 multi-hop UE-to-Network Relay (either FQDN or IP address) in the HPLMN.

-	5G ProSe Layer-3 UE-to-Network Relay supporting 5G ProSe Layer-3 multi-hop UE-to-Network Relay access node selection information - a prioritized list of PLMNs for N3IWF selection. It also indicates if selection of an N3IWF in a PLMN should be based on Tracking Area Identity FQDN or on Operator Identifier FQDN.

NOTE 6:	5G ProSe Relay Discovery policy/parameters can be provided from ProSe Application Server to the 5G ProSe Layer-3 Remote UE supporting 5G ProSe Layer-3 multi-hop UE-to-Network Relay.

4)	Parameters to broadcast warning messages:

-	Configured Destination Layer-2 ID(s);

-	PC5 QoS parameters defined in clause 5.6.1;

-	the NR Tx Profile based on the configuration as specified in clause 5.1.3.1.

NOTE y:	It is up to inter operator alignment and network configuration whether a common Destination Layer-2 ID or IDs are used for all UEs cross PLMNs in a particular region for Public Warning Notification Relaying.

5)	Validity time indicating the expiration time of the Policy/Parameter for 5G ProSe Layer-3 multi-hop UE-to-Network Relay discovery and communication.

The following information is provisioned in the UE in support of the UE assuming the role of a 5G ProSe Layer-3 Intermediate UE-to-Network Relay:

1)	Authorisation policy for acting as a 5G ProSe Layer-3 Intermediate UE-to-Network Relay:

-	For 5G ProSe Layer-3 Intermediate UE-to-Network Relay, indicates whether the UE is authorised to act as a 5G ProSe Layer-3 Intermediate UE-to-Network Relay.

The authorisation for a UE to act as a 5G ProSe Layer-3 Intermediate UE-to-Network Relay also authorizes the use of 5G ProSe Layer-3 multi-hop UE-to-Network Relay Discovery with Model A and Model B.

NOTE 7:	It is up to UE and application implementation to select a discovery model or whether to perform both models simultaneously.

2)	ProSe Relay Discovery policy/parameters for 5G ProSe Layer-3 Intermediate UE-to-Network Relay:

-	Includes the parameters that enable the UE to perform 5G ProSe Layer-3 multi-hop UE-to-Network Relay Discovery when provided by PCF or provisioned in the ME or configured in the UICC:

-	5G ProSe Layer-3 multi-hop UE-to-Network Relay Discovery parameters (User Info ID, Relay Service Code(s), UE-to-Network Relay Layer Indicator per RSC, multi-hop indicator per RSC, maximum number of hops per RSC). The UE-to-Network Relay Layer Indicator indicates whether the associated RSC is offering 5G ProSe Layer-2 or Layer-3 UE-to-Network Relay service. RSC dedicated for emergency service may also be provisioned. The multi-hop indicator indicates whether the associated RSC can be supported for 5G ProSe multi-hop UE-to-Network Relay operation. The maximum number of hops per RSC indicates the maximum number of hops can be supported for the associated RSC.

NOTE 8:	The RSC used for multi-hop service is different from the RSC used for single-hop service.

-	Default Destination Layer-2 ID(s) for sending Relay Discovery Announcement and Relay Discovery Additional Information messages and receiving Relay Discovery Solicitation messages;

-	For 5G ProSe Layer-3 Intermediate UE-to-Network Relay, the traffic type (IP, Ethernet, Unstructured) to be used for the relayed traffic for each Relay Service Code;

NOTE 9:	5G ProSe Relay Discovery policy/parameters can be provided from ProSe Application Server to the 5G ProSe Layer-3 Intermediate UE-to-Network Relay.

3)	Parameters to broadcast warning messages:

-	Configured Destination Layer-2 ID(s);

-	PC5 QoS parameters defined in clause 5.6.1;

-	the NR Tx Profile based on the configuration as specified in clause 5.1.3.1.

NOTE z:	It is up to inter operator alignment and network configuration whether a common Destination Layer-2 ID or IDs are used for all UEs cross PLMNs in a particular region for Public Warning Notification Relaying.

4)	Validity time indicating the expiration time of the Policy/Parameter for 5G ProSe Layer-3 Intermediate UE-to-Network Relay discovery and communication.

The following information is provisioned in the UE in support of the UE assuming the role of a 5G ProSe Layer-3 UE-to-Network Relay supporting 5G ProSe Layer-3 multi-hop UE-to-Network Relay, in the UE in support of the UE assuming the role of a 5G ProSe Layer-3 Remote UE supporting 5G ProSe Layer-3 multi-hop UE-to-Network Relay and thereby enabling the use of a 5G ProSe Layer-3 UE-to-Network Relay and in the UE in support of the UE assuming the role of a 5G ProSe Layer-3 Intermediate UE-to-Network Relay:

1)	Radio parameters for 5G ProSe Layer-3 multi-hop UE-to-Network Relay Discovery when the UE is not "served by NG-RAN":

-	Includes the radio parameters NR PC5 with Geographical Area(s) and an indication of whether they are "operator managed" or "non-operator managed". The UE uses the radio parameters to perform 5G ProSe Direct Discovery over PC5 reference point when "not served by NG-RAN" only if the UE can reliably locate itself in the corresponding Geographical Area. Otherwise, the UE is not authorized to transmit.

-	Default PC5 DRX configuration (see TS 38.331 [16]).

2)	Radio parameters for 5G ProSe Layer-3 multi-hop UE-to-Network Relay communication when the UE is not "served by NG-RAN":

-	Includes the radio parameters NR PC5 with Geographical Area(s) and an indication of whether they are "operator managed" or "non-operator managed". The UE uses the radio parameters to perform 5G ProSe Direct Communication over PC5 reference point when "not served by NG-RAN" only if the UE can reliably locate itself in the corresponding Geographical Area. Otherwise, the UE is not authorized to transmit.

NOTE 10:	The validity time of these radio parameters is the same as the validity time of the Policy/Parameter listed above for 5G ProSe Layer-3 UE-to-Network Relay supporting 5G ProSe Layer-3 multi-hop UE-to-Network Relay or 5G ProSe Layer-3 Remote UE supporting 5G ProSe Layer-3 multi-hop UE-to-Network Relay or 5G ProSe Layer-3 Intermediate UE-to-Network Relay.

3)	Policy/parameters related to privacy:

-	A privacy timer value indicating the duration after which the UE shall change each source Layer-2 ID self-assigned by the UE when privacy is required.

# * * * * Next change* * * *

5.18	Support of Public Warning Notification Relaying

As specified in TS 22.268 [33], if UE that is authorised to act as 5G ProSe UE-to-Network Relay receives Public Warning Notification from the network and the UE is configured to perform Public Warning relaying, it shall handle the Public Warning message acting in the role of 5G ProSe UE-to-Network Relay.

The 5G ProSe UE-to-Network Relay broadcasts the warning message i.e. SIB 6/7/8 received from the network to the 5G ProSe Remote UE(s) by using Broadcast mode of 5G ProSe direct communication as specified in clause 5.3.2. The 5G ProSe UE-to-Network Relay uses a configured Destination Layer-2 ID(s) as specified in clause 5.1.4.1 when broadcasting the warning message and the 5G ProSe Remote UE receives warning messages broadcasted over PC5 reference point by using a configured Destination Layer-2 ID(s) as specified in clause 5.1.4.1. The PC5 QoS parameters as specified in clause 5.1.4.1 are used to broadcast and receive the warning message for the 5G ProSe UE-to-Network Relay and the 5G ProSe Remote UE, respectively.

If the UE that is authorized to act as a 5G ProSe Intermediate UE-to-Network Relay receives the broadcasted warning message by using a configured Destination Layer-2 ID(s) as specified in clause 5.1.4.1a and the UE is configured to perform Public Warning relaying, it shall broadcast the message and increment the Hop-Count value, or add a Hop-Count set to 2 into the message if it is not present. The 5G ProSe Intermediate UE-to-Network Relay shall not broadcast the warning message if the Hop-Count value is larger than the stored Hop-Count in its discovery entry. The 5G ProSe Intermediate UE-to-Network Relay uses a configured Destination Layer-2 ID(s) as specified in clause 5.1.4.1a when broadcasting the warning message and the 5G ProSe Remote UE receives warning messages broadcasted over PC5 reference point by using a configured Destination Layer-2 ID(s) as specified in clause 5.1.4.1a. The PC5 QoS parameters as specified in clause 5.1.4.1 are used to broadcast and receive the warning message for the 5G ProSe Intermediate UE-to-Network Relay and the 5G ProSe Remote UE, respectively.

A 5G ProSe Remote UE can receive the broadcasted warning message without establishing a connection to the 5G ProSe UE-to-Network Relay or 5G ProSe Intermediate UE-to-Network Relay.

The 5G ProSe UE-to-Network Relay performs the duplication detection function as specified in TS 23.041 [32] to suppress the received duplicated warning messages over Uu. The 5G ProSe Intermediate UE-to-Network Relay and 5G ProSe UE-to-Network Remote UE performs the duplication detection function as specified in TS 23.041 [32] to detect duplicated warning messages received over PC5 and/or Uu.

NOTE:	The duplication detection function in the 5G ProSe UE-to-Network Relay and 5G ProSe Intermediate UE-to-Network Relay avoids broadcasting the duplicated warning message over PC5.

If the 5G ProSe Layer-2 UE-to-Network Relay or the 5G ProSe Intermediate Layer-2 UE-to-Network Relay does not have Destination Layer-2 ID to broadcast Public Warning messages, the 5G ProSe Layer-2 UE-to-Network Relay or the 5G ProSe Intermediate Layer-2 UE-to-Network Relay forwards the PWS SIBs (i.e. SIB 6/7/8) to a connected 5G ProSe Layer-2 Remote UE over the unicast link as specified in TS 38.300 [12].

The Public Warning System architecture for 5G System is specified in TS 23.041 [32].

# * * * * End of change * * * *