3GPP TSG-WG SA2 Meeting #168	S2-2503140

Gothenburg, Sweden, 7th April – 11th April, 2025	(revision of -)

Source:	Huawei, HiSilicon

Title:	New Solution for UE Data Collection and Transfer over User Plane

Document for:	Approval

Agenda Item:	20.3.1

Work Item / Release:	FS_AIML_CN_Ph2/ Rel-20

Abstract: For training data collection from the UE to the OTT server for UE-side model training, a new solution is proposed to enable the 5G Core to support UE data collection over User Plane.

# 1. Introduction

SID SP-250413 has WT#1 to address RAN requirements:

-	WT#1: Study whether and how to support the standardized transfer of standardized data over UP for UE data collection to meet requirements for AI/ML for NR air interface operation with UE-side model training.

Note 1: The WT#1 scope is based on UP Data Collection for Option2 documented in RAN LSs RP-243316 and RP-242389.

Note 2: The requirements are elaborated in LSs RP-242389 and R2-2411152 which will be considered during the study phase.

Note 3: WT#1 impact on user consent and privacy will be addressed in cooperation with SA3.

The solution includes three aspects for training data collection from the UE to the OTT server for UE-side model training: 1) UEs transfer the training data to the 5G Core; 2) 5G Core exposes the training data to the OTT server; 3) 5G NFs assist UE selection for data collection.

# 2. Text Proposal

It is proposed to capture the following changes to TR 23.700-04.

# * * * * First change * * * *

## 6.X	Solution#X: UE Data Collection and UE Data Exposure over User Plane

### 6.X.1	Description

This solution includes three aspects for training data collection from UE to OTT server for UE-side model training: 1) UEs transfer the training data to the 5G Core; 2) 5G Core exposes the training data to the OTT server; 3) 5G NFs assist UE selection for data collection.

For 5G Core assistance in data transfer from UE to OTT server, we assume that it will be a dedicated NWDAF with UE data exposure capability that collect data from UE and exposes to the OTT server (optionally) via NEF. In this solution, the NWDAF with UE data exposure capability is called Data-Exposure NWDAF (DE-NWDAF).

The data collection and exposure path is shown in Figure 6.X.1-1:

UE Data for UE-side model training

UE Data for UE-side model training

NEF

DE-NWDAF

UE

OTT server

UE Data Transfer over UP connection

Figure 6.X.1-1: data collection and exposure framework

To support UE data exposure, the DE-NWDAF registers to the NRF with information listed below in its NF profile:

-	UE Data Exposure Indication.

For UE data subscription/request, the OTT server may provide the inputs parameters listed below:

-  Data Context ID(s): identifies the use case for which the required data is used for, e.g., Beam management, CSI prediction, Positioning. Data context ID is used to determine what data should be collected.

- 	[Optional] Sub case Info: identifies the specified sub-case for required data, e.g. AI-assisted/direct positioning.

-  Time Window: indicates the time interval within which the data should be collected.

-  Filter Info

-	[Optional] Data Requirement Information

### Editor’s Note: whether other parameters are required are FFS. The details of the filter and data requirement info are FFS.

### 6.X.2 Procedures

#### 6.X.2.1 Procedure of UE Data Exposure from DE-NWDAF to untrusted OTT Server

Figure 6.X.2.1-1: Procedure of UE Data Exposure from DE-NWDAF to untrusted OTT Server

1. DE-NWDAF registers its NF profile including UE Data Exposure Indication into NRF;

2. OTT Server subscribes for UE data from NEF by invoking Nnef_EventExposure_Subscribe service; OTT server includes Data Context ID, optional Sub case Info, Time Window, Filter information and optional Data Quality Requirement in the data request.

3. NEF checks if the OTT Server is allowed to get the requested data.

4. Optionally, If OTT server is not permitted, NEF rejects OTT Server via Nnef_EventExposure_Subscribe Response with a failure indication and optional failure reason. In this case, Step 5-9 will be skipped.

5. NEF discovers the DE-NWDAF supporting UE Data Exposure from NRF.

6. NEF subscribes for UE Data from DE-NWDAF by invoking Nnwdaf_DataManagement_Subscribe Service; NEF provides Data Context ID, optional Case Info, Time Window, Filter information, Data Requirement received in Step 2 to DE-NWDAF.

7. Optionally, if DE-NWDAF doesn’t have the required UE data, DE-NWDAF may trigger UE data collection procedure as described in clause 6.X.2.2. Otherwise, DE-NWDAF can provide the data immediately.

8-9. DE-NWDAF provides the UE training data to OTT server via NEF.

10-11. Optionally, DE-NWDAF provides the newly collected UE training data to OTT server via NEF.

#### 6.X.2.2 Procedure of UE Data Collection from UE to DE-NWDAF

UE

DE-NWDAF

UPF

AMF

1. Determine to collect UE data, based on request from OTT server or internal trigger

4. Namf_Communication_N1N2Transfer (user plane info(DE-NWDAF UP address), [O] Data Collection Request)

5. DL NAS message (user plane info(DE-NWDAF UP address), [O] Data Collection Request)

6. UE establish user plane connection with DE-NWDAF

7. Data Request over User Plane Connection

9. Data Transfer over User Plane Connection

UDM

8. UE contacts LMF to retrieve positioning label data

3. DE-NWDAF check user consent for data collection

LMF

2. Select UE for data collection as described in clause 6.X.2.3

Figure 6.X.2.2-1: Procedure of UE Data Collection from UE to DE-NWDAF

1. DE-NWDAF determines to collect UE data based either on data request from OTT server, or its internal trigger.

2. Optionally, if no UE list is provided by OTT server, DE-NWDAF and AMF determine the UE list for data collection as described in clause 6.X.2.3.

3. DE-NWDAF checks user consent for UEs. It may subscribe for user consent information from UDM. DE-NWDAF skips those SUPIs that don’t grant user consent for UE Data Collection.

NOTE: Details of user consent check for UE data collection for UE-side model training will be discussed with SA WG3.

4. DE-NWDAF invokes Namf_communication_N1N2MessageTransfer service operation to send the user plane information in a NAS container to AMF to indicate UE to utilize user plane for UE data collection. The user plane information includes the user plane address of the DE-NWDAF. DE-NWDAF may optionally send data collection request in another container to AMF, which includes correlation ID and other parameter, such as Data Context ID(s), Time Window, optional Sub case info etc.

5. When AMF receives the user plane information and optional data collection request from DE-NWDAF in step 4, AMF sends it to UE via a DOWNLINK NAS TRANSPORT message which includes the user plane info in the N1 container.

6. If there is no established applicable PDU session for user plane data transfer, UE establishes PDU Session for data transfer and user plane connection with DE-NWDAF: UE uses the URSP which includes specific PDU session parameters, e.g. a dedicated DNN and S-NSSAI to establish the PDU Session for user plane data transfer.

7. Optionally, if DE-NWDAF doesn’t sent data collection request in step 4, DE-NWDAF sends data collection request to UE over the User Plane connection with correlation ID and other parameter, such as Data Context ID(s), Time Window, optional Sub case info etc.

8. If the data request from DE-NWDAF indicates that data collection for AI-based positioning case is needed, then UE may contact LMF for positioning label data. In such case, 5GC-MO-LR procedure, as defined in clause 6.2 of TS 23.273[x], is triggered.

9. UE transfers collected data to DE-NWDAF over the User Plane connection with the correlation ID.

Editor’s Note: Whether and how the network (e.g. DE-NWDAF) can be aware whether the data collected from the UE is the same from the one measured by the UE is FFS.

#### 6.X.2.3 Procedure of UE Selection for Data Collection

Option #2

Option #1

AMF

DE-NWDAF

OTT server

0. trigger of UE data collection

1. Determine to collect UE data

2a. Namf_EventExposure_Subscribe (“Any UE”, Event ID(s) = "UEs in/out area of interest", and optionally "UE Data Collection Capability", "PEI", "UE CM State“, …)

3a. Namf_EventExposure_Subscribe Response (list of SUPIs in the area of interest, [O] UE Data Collection Capability, [O] PEI, [O] UE CM state, …)

4a. Decides the UE list per AMF

3b. Namf_EventExposure_Subscribe (“Any UE”, Event ID(s) = "UEs in/out area of interest", "UE Data Collection Capability", "PEI", "UE CM State“ and corresponding filter(s))

4b. Namf_EventExposure_Subscribe Response (filtered UE list)

5b. Decides the UE list per AMF

2b. Decides the set of event ID and corresponding filter(s)

The DE-NWDAF determines the UE list for data collection based on data collection requirements and UE information (e.g., user consent, PEI, UE status).

Figure 6.X.2.3-1: Procedure of UE Selection for Data Collection

- DE-NWDAF determines to collect UE data based either on data request from OTT server or its internal trigger. If no UE list is included in the data request from OTT server or UE data collection is triggered internally, DE-NWDAF determines the UE list for data collection

Option 1): DE-NWDAF gathers UE information and selects UE list.

2a. DE-NWDAF subscribes to AMF to retrieve the list of UEs located in an area of interest and the UE Data Collection Capability for each UE using Namf_EventExposure_Subscribe Request (Target of Event Reporting = "any UE", set of Event ID including "UEs in/out area of interest", and optionally "UE Data Collection Capability", "PEI", "UE CM State"). When the AoI involves the service areas of multiple AMFs, DE-NWDAF subscribes to each of them.

3a. The AMF sends Namf_EventExposure_Subscribe Response (list of SUPIs in the area of interest, and optionally for each SUPI, UE Data Collection Capability, PEI, UE CM state…) to DE-NWDAF.

4a. DE-NWDAF may decide the number of UEs based on the requirements from OTT server as described in clause 6.X.1. If multiple AMFs are involved, DE-NWDAF may decide the number of UEs per AMF based on, e.g. the proportion of AoIs served by the AMF. Then the DE-NWDAF may decide the UE list per AMF based on Vendor ID, Chipset ID, UE CM-Connected status, user consent, etc.

Option 2): DE-NWDAF sends filters to AMF and gets filtered UE list from AMF.

2b-3b. DE-NWDAF decides the set of event ID including "UEs in/out area of interest", "Data Collection Capability", "PEI ", "UE CM State", "number of UEs", etc., and the corresponding filters associated with each Event ID. Then NWDAF invokes Namf_EventExposure_Subscribe Request towards AMF.

4b. AMF responses the filtered UE list to DE-NWDAF.

5. DE-NWDAF may decide UE list per AMF based on the AMF response and user consent, etc.

Editor’s note: The details of AMF impact are FFS.

### 6.X.3 Impacts on services, entities and interfaces

UE:

-	Register UE Data Collection Capability to AMF.

-	Perform and report new measurement.

-  Establish a new application layer connection with DE-NWDAF.

DE-NWDAF:

-	Register UE Data Exposure Capability in NF profile to NRF.

-  Establish a new application layer connection with UE.

-	Collect UE data from UE for UE-side model training.

-	Provide UE data to OTT server via NEF.

-	Decide UE list for training data collection.

OTT server:

-	Subscribe for UE data from NEF for UE-sided model training.

NEF:

-	Support UE data exposure from DE-NWDAF to OTT server.

NRF:

-	Support DE-NWDAF registration and discovery procedure for UE data exposure.

AMF:

-	Support User Plane connection establishment between UE and DE-NWDAF: transfer DE-NWDAF user plane information to UE in new NAS container.

-  Support UE selection for training data collection: provide UE information to DE-NWDAF, or provide filtered UE list to DE-NWDAF.

PCF：

-	Provide UE with URSP rules which includes specific PDU session parameters for user plane connection between UE and DE-NWDAF.

# * * * * End of changes * * * *