3GPP TSG-SA WG2 Meeting #168	S2-2503144

Gothenburg, Sweden, April 07 – 11, 2025

Source:	NTT DOCOMO

Title:	KI#2: New Sol: Energy-aware data transfer policy negotiation

Document for:	Approval

Agenda Item:	20.4.1

Work Item / Release:	FS_EnergySys_Ph2 / Rel-20

Abstract of the contribution: This contribution proposes a solution for KI#2 where AF can negotiate for an energy-aware data transfer policy.

# 1	Discussion

5GC provides "Planned Data Transfer with QoS Requirement" policy negotiation services that allow AF to negotiate with 5GC to transfer (background) data with QoS requirements; however, these policies do not consider the energy related aspects (e.g., energy consumption). It is proposed to introduce a new policy negotiation services to consider energy as well. The basic principle of the proposed solution is that AF can negotiate with 5GC not only the transfer time but also the QoS requirements to achieve an energy-related objective (such as minimization of energy consumption).

# 2 Proposal

It is proposed to update TR 23.700-67 according to the following text.

********** First Change (All new text) **********

## 6.x	Solution #X: Energy Aware Data Transfer Policy Negotiation

### 6.x.1	Key Issue mapping

The proposed solution maps to the following aspect of KI#2:

WT#2: Whether and how to support service adjustments for the UE to enable energy saving. This may include:

WT#2.1 Whether and how to support taking the energy attributes (e.g., renewable energy information) into account to determine UE traffic handling (e.g., QoS adjustment)

WT#2.2 Whether and how to support policy control based on interaction with EIF

### 6.x.2	Functional Description

Currently, PCF provides "Npcf_PDTQPolicyControl" service that allow AF to negotiate with 5GC for "Planned Data Transfer with QoS requirements" (PDTQ). However, the policy does not consider the energy-related aspects. To support energy related information as service criteria, it is proposed to extend 5GC functionalities to provide a service similar to PDTQ that allows AF to negotiate "Energy Aware Data Transfer" (EADT).

The overall procedure of EADT is similar to that of PDTQ policy negotiation procedures. AF initiates the negotiation by specifying its desired settings (e.g., desired time window and/or desired QoS profile). PCF takes this request and considering the AF preferences and also the status of the network (which can be predicted by NWDAF for time windows in future) creates a number of policies (indicating time window, QoS profile and energy-related information) and shares them with AF; then AF selects a policy and inform the PCF. Later, AF requests the QoS flow in the time window specified in the selected policy.

In the proposed solution, there are two differences in the EADT policy negotiation, in comparison to the PDTQ policy negotiations: 1) the AF can specify its adaptability/flexibility in terms of QoS level and transfer time; and 2) PCF also takes the energy-related aspect of service provision into account and consequently, the derived policies consider energy-related information as service criteria. The details of these two aspects are explained in the following paragraphs.

To consider energy related information as service criteria, AF may adapt the transfer of the data in two aspects:

- QoS requirements Adaptability: On one hand, providing different QoS levels by 5GS may consume different amounts of energy; on the other hand, similar to the "Alternative QoS Profile" concept, different QoS profiles might be acceptable for the AF. Hence, for the energy aspects of the service (e.g., energy consumption, and carbon emission), the AF can indicate its adaptability in terms of QoS requirements either by 1) providing a list of acceptable QoS profiles or 2) providing a desired QoS profile and indicating the level of flexibility in terms of QoS requirement (e.g., "0" implies no flexibility and "100" means full flexibility).

- Transfer time Adaptability: Energy related aspect of a service (e.g., renewable energy consumption ratio) may differ in different time windows, consequently, to consider energy related information as service criteria, similar to the PDTQ policy, AF may adapt the transfer time of the data. AF can specify its adaptability either by 1) providing a list of acceptable transfer time windows or 2) providing a desired transfer time windows and indicating its level of flexibility in terms transfer time with respect the desired window (e.g., "0" implies no flexibility and "100" means full flexibility).

Accordingly, in the proposed solution, in addition to the parameters that existing in the PDTQ policy such as "data volume", "Desired time window", "Desired QoS profile", the AF can also indicate its level of adaptability in terms of QoS requirements and/or transfer time.

PCF is the NF responsible for deriving the policies and responding to the AF. The policies are derived based on the AF desired settings and also considering network status. To support energy related information as service criteria, in deriving the policies, PCF considers not only availability of the network resources but also energy related aspect of service provisioning, that may include:

- Energy related information of RAN segment of the service

- Energy related information of the 5GC segment of the service

- Energy consumption of the service

- Energy efficiency of the service

- Carbon emission of the service

- Renewable energy consumption ratio of the service

For this purpose, at the first step, according to the (desired) transmission time window(s), (desired) QoS profile(s) and the corresponding levels of adaptability provided by AF, PCF derives a number of "Transmission Opportunity" options where each option is composed of (i) transmission time and (ii) QoS level. Each option is time window and QoS level proposal for the AF to transmit the data. In the second step, PCF needs to evaluate each option considering both the network resources and the energy aspects via collecting information from NWDAF, OAM, or EIF.

Once, PCF has the energy related information of a transmission opportunity, it derives the corresponding policy considering resource availability and operator’s policy. Finally, PCF send the list of the policies to the AF. AF evaluates the polices and selects one of them and informs the PCF. Later, AF request the QoS flow at the time specified in the selected policy.

### 6.x.3	Procedures

The procedure depicted in Figure 6.x.3-1 allows AF to negotiate an energy aware time window with specified QoS level to transmit data.

Figure 6.x.3-1: Negotiation for energy aware data transfer

Prior to the transfer of the data, the AF negotiates with the 5G Core for the EADT policies. At first, AF discovers its serving NEF before starting the following procedure.

1a.	The AF invokes the Nnef_EADTPolicyNegotiation_Create Request (ASP Identifier, target UE), and optionally includes (Desired) time window(s), (Desired) QoS Profile(s), Transfer time adaptability, QoS requirements adaptability.

1b-1c. The NEF may authenticate the AF and authorize the EADT request from the AF. If the authentication/authorization of the AF's request has failed, the NEF will respond to the AF's request through the Nnef_EADTPolicyNegotiation_Create Response with a failure result and the following steps are skipped.

2.	Based on an AF request, the NEF may translate the information provided by the AF and invokes the Npcf_EADTPolicyControl_Create with the PCF to authorize the creation of the policy.

3.	PCF queries the UDR to retrieve all existing EADT polices using Nudr_DM_Query service operation.

4.	The UDR provides all the stored EADT policies and corresponding related information to the PCF.

5.	Based on the (desired) QoS profile(s) and (desired) transmission time window(s) and also the corresponding level of adaptabilities provided by the AF, the PCF derive a number of transmission opportunity options. Each option is composed of a time window and QoS profile. Then steps 6-7 are performed for each option.

6.	For the given transmission opportunity option, PCF requests about network status prediction from NWDAF.

7.	For the given transmission opportunity option, PCF requests about energy-related information from OAM (and EIF).

8.	Based the information collected in step 6 and 7, and considering the operator’s policy on energy, the PCF determines one or more EADT policies. Each EADT policy includes a recommended time window, a recommended QoS profile, and corresponding energy related information, for the traffic transfer for each of the AF sessions for each of the UEs involved.

9.	The PCF sends one or more EADT policies to NEF in Npcf_EADTPolicyControl_Create Response including the EADT Reference ID.

10.	The NEF sends a Nnef_EADTPolicyNegotiation_Create response to the AF to provide one or more EADT policies together with the EADT Reference ID.

11.	The AF selects one of the EADT policies and notifies NEF for the selected EADT policy via Nnef_EADTPolicyNegotiation_Update request together with the EADT Reference ID. The AF stores the EADT Reference ID for the future interaction with the PCF.

12-14. The NEF notifies PCF about the selected EADT policy by the AF. The PCF acknowledges NEF. The NEF responds to the AF request with a Nnef_EADTPolicyNegotiation_Update Response.

15-16. The PCF stores the EADT Reference ID together with the new EADT policy in the UDR by invoking Nudr_DM_Update. The UDR sends a response to the PCF as acknowledgement.

### 6.x.4	Impacts on existing services, entities and interfaces

The solution has the following impacts:

NEF:

-	Extending service to negotiate EADT policy.

PCF:

-	Extending service to negotiate EADT policy.

-	Extending functionality to get energy related information of transmission opportunities options

OAM:

-	Extending functionalities to provide energy related information

********** End of Changes **********