SA WG2 Meeting #168	S2-2503161

Goteborg, Sweden, 07 – 11 April 2025	(revision of S2-250xxxx)

Source:	Ofinno

Title:	Update Assistance information for Inventory procedure

Document for:	Approval

Agenda Item:	19.14.2

Work Item / Release:	AmbientIoT / Rel-19

Abstract of the contribution: The paper proposes to update clause 5.4.

# 1. Discussion

At the last meeting, two AIoT service procedures (i.e., Inventory procedure, Command procedure) between the AIoTF and the AIoT RAN were approved. These two procedures can be differentiated based on their message names between the AIoTF and the AIoT RAN. Accordingly, AIoT service type in the assistance information is redundant.

Proposal 1:   It is proposed to delete the AIoT service type from the Assistance information.

As explained in the S2-2503149, for the Inventory procedure, conditional assistance information may be required in the Inventory request message, if the Inventory procedure is performed as part of the Command procedure, to indicate existence of a follow-up command procedure.

Proposal 2:   It is proposed to include a command indicator as a conditional assistance information for Inventory procedure to indicate the follow-up command procedure.

# 2. Proposal

It is proposed to capture the following changes into TS 23.369v0.2.0.

* * * * Start of Changes * * * *

## 5.4	Assistance information provided to AIoT RAN node

The AIOTF provides the assistance information to the AIoT RAN together with the service operation requests. The AIOTF determines the assistance information provided to the AIoT RAN based on the information received from the AF. The Assistance information is used by the AIoT RAN for performing service operations, e.g. radio resource allocation.

The following assistance information may be provided:

-	Conditionally, a command indicator indicating that an Inventory procedure is followed by a Command procedure;

-	Optionally, approximate number of AIoT devices based on AF request;

-	Optionally, approximate D2R message size based on AF request.

Editor's note:	Other assistance information may be added later if necessary.

Editor's note:	How the AIOTF determines the assistance information (e.g. approximate number of AIoT Devices, approximate D2R message size) without input from the AF is FFS.

* * * * End of Changes * * * *