3GPP TSG-SA WG2 Meeting #168	S2-2503165

Goteborg, Sweden, April 07 – 11, 2025

Source:	Samsung

Title:	Key issue for WT#1: Support of IMS voice over GEO NB-IoT NTN connecting to EPC

Document for:	Approval

Agenda Item:	20.1.1

Work Item / Release:	FS_5GSAT_Ph4-ARC / Rel-20

Abstract of the contribution: The contribution proposes the key issue for WT#1 Support of IMS voice over GEO NB-IoT NTN connecting to EPC.

1. Discussion

The SID on Integration of satellite components in the 5G architecture Phase 4 (SP-250400) includes the following WTs:

-	WT-1: IMS voice call over GEO via NB-IoT NTN connecting to EPC

NOTE 1:	both CP and UP based NB IoT NTN solutions will be studied in the WT-1, checkpoint will be set before the normative work to align with RAN scope.

NOTE 2:	Only architecture options with data via SGi will be considered in the study. The option with data via SCEF/T8 will not be considered.

-	WT-1.1 Whether and how to support QoS for IMS voice service over GEO satellite,

-	WT-1.2 whether and how to define IMS enhancement to utilize NB IoT as IP-CAN and potential optimization to comply with the relevant SA1 requirements.

-	WT-1.3 Support of IMS emergency call via NB-IoT NTN.

-	WT-1.4 How to obtain UE Geo-location for IMS emergency call and other regulatory services.

NOTE 3:	Solutions should take into account that CP data (DoNAS) in current pre Rel-20 NB-IoT is regarded as NAS signalling and will have higher QoS than UP data (S1-U).

NOTE 4:	There will be NO mobility enhancement developed for the NB-IoT NTN in this WT.

NOTE 5:	Depending on operators’ policies, IMS enhancements defined in the WT-1 may also be used in 5GS with NR NTN, but no additional study will be conducted in this study item.

Therefore, it is proposed to study the following KI for further enhancement on integration of satellite components in GEO NB IoT NTN connecting to EPC.

2. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-19.

* * * Start of Change * * * *

5.x	Key Issue #x: Support of IMS voice over GEO NB-IoT NTN connecting to EPC

5.x.1	Description

This key issue studies how to enhance architecture and procedures to support IMS voice call over GEO satellite access via NB-IoT NTN connecting to EPC. In particular, this key issue will study:

NOTE 1:	The type of solutions (CP, UP, or both) to be taken as the normative scope will be aligned with RAN scope.

NOTE 2:	Only the SGi PDN connection option will be considered, and any SCEF PDN connection option will not be considered in this study.

-	Whether and how to support QoS for IMS voice service over NB-IoT NTN GEO access.

-	Whether and how to enhance IMS procedures to reduce call set-up time for NB-IoT GEO access over IP-CAN.

-	How to support IMS emergency call over NB-IoT NTN GEO access.

-	How to obtain the precise UE location (e.g. geographical location) for IMS emergency call and other regulator services.

NOTE 3:	Any mobility enhancement for the NB-IoT NTN (e.g. satellite change) will not be considered in this study.

NOTE 4:	Any IMS enhancement for 5GS with NR NTN will not be studied. The outcome of this KI may be used for 5GS based on the operator policies.

* * * End of Change * * * *