3GPP TSG-SA WG2 Meeting #168	S2-2503174

# Goteborg, Sweden, April 07 – 11, 2025               	       (revision of S2-220xxxx)

Source: 	OPPO

Title: 	23.700-14: Key issue X: Discovery and Selection of Sensing Function

Document for: 	Approval

Agenda Item: 	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract of the contribution: The contribution proposes the key issue for WT#3 on Discovery and Selection of Sensing Function.

1. Discussion

The WT-3 of the ISAC study addresses the general requirement to discover and select sensing entities (e.g., transmitters, receivers, sensing function (SF)) based on service requirements triggered by sensing requests and entity capabilities. SA1 mandates support for sensing applications (e.g., intruder detection, collision avoidance, real-time mapping) across commercial, V2X, and public safety use cases, which require coordinated operation of multiple entities. For example, transmitters send sensing signals, receivers generate measurement data, and SF nodes process data to generate results.

While the discovery and selection of all sensing entities (transmitters, receivers, SF) is part of the broader study, ​this Key Issue specifically focuses on SF discovery and selection, as SF are responsible for processing sensing data and generating actionable results. The need to study SF-specific mechanisms is driven by:

- ​SF capability diversity: Different sensing services (e.g., high-accuracy monitoring vs. low-latency collision avoidance) require SF nodes with varying capabilities (e.g., computational power, supported algorithms).

- ​Deployment scenarios: SF nodes may be deployed as centralized network functions or edge nodes, necessitating flexible discovery mechanisms compatible with 3GPP architecture.

- ​Service requirement alignment: SF selection must match service needs (e.g., latency for real-time applications, accuracy for intruder detection) defined in SA1.

Key aspects to investigate for ​SF discovery and selection include:

-	Whether and how to enable discovery and selection of SF entities based on service requirements triggered by sensing service requests.

-	Whether and how to define selection criteria (e.g. centralized, edge deployed, serving area) for SF entities considering sensing service type (e.g., intruder detection, real-time mapping) and use case (commercial, public safety).

-	Whether and how to enhance existing 3GPP procedures or define new mechanisms for SF discovery and selection.

Thus, it is proposed to agree the following changes to 3GPP TR 23.700-14.

2. Proposal

Start of Changes (all new text below)

## 5.1.x	Key Issue #x: Discovery and Selection of Sensing Function

### 5.1.x.1	Description

Integrated sensing and communication (ISAC) expands 3GPP networks into communication-sensing integrated systems, requiring coordinated discovery and selection of sensing function (SF) entities to process sensing data and generate results. This is justified by SA1 requirements to support diverse sensing applications (e.g., intruder detection, monitoring, collision avoidance) across commercial, V2X, public safety, and emergency services use cases. The selection of appropriate SF entities must align with service requirements (e.g., sensing accuracy, latency) and SF capabilities to ensure efficient operation within 5G systems.

This Key Issue aims to study:

- Whether and how to enable discovery and selection of SF entities based on service requirements triggered by sensing service requests.

- Whether and how to define selection criteria (e.g. centralized, edge deployed, serving area) for SF entities considering sensing service type (e.g., intruder detection, real-time mapping) and use case (commercial, public safety).

- Whether and how to enhance existing 3GPP procedures or define new mechanisms for SF discovery and selection.

End of Changes