3GPP TSG-SA WG2#168	S2-2503224

Gothenburg, Sweden, 7-11 April 2025	(was S2-250xxxx)

Source:	Qualcomm Incorporated, Vodafone

Title:	On TAI configuration in AMF via OAM

Document for:	Discussion/Approval

Agenda Item:	19.1.2

Work Item / Release:	5GSAT_Ph3_ARC/Rel-19

Abstract of the contribution: Discusses the RAN3 proposal for TAI configuration in AMF and proposes a way forward.

# 1.	Discussion

SA2#166 sent reply LS S2-2413030 to RAN3 and SA5 on “OAM requirements to support regenerative payload”. The main points of this LS are summarised below:

- the TAIs broadcast by an NTN gNB are earth static, and the TAIs broadcast by the NTN gNB may change with the movement of the satellite;

- the NG Removal procedure is defined to remove the interface between a gNB and an AMF. SA2 believes that, after receiving an NG Removal message, the AMF will release the information of supported TAIs and it is the gNB to determine the time to initiate the NG Removal procedure based on the AMF’s serving area (e.g., the serving PLMN allowed operation area). Therefore, the number of TAIs and related updates will be bounded by those which can be covered by a satellite while using a NTN gateway;

- SA2 discussed whether the OAM solution would have impacts to the existing procedures related to paging and sequencing of OAM updates and RAN3 updates towards the AMF, e.g., in NGAP Setup/configuration update etc.

- It was unclear to SA2 how the OAM solution would work based on the description in the incoming LS to be able to determine whether there are impacts.

- The frequency of updates by OAM was not clear to SA2 and whether the information provided to the AMF is a timetable of which TAIs the gNB will support at what time, or whether the updates are provided frequently as the satellite moves updating the AMF each time a change occurs.

The bulleted list provided by RAN3 in R3-250895 is numbered for better readability:

- In general, satellite orbital movement is periodic and predictable; therefore, any resulting change to node parameters will result in the same information being frequently exchanged over network interfaces at regular intervals.

- In the solution agreed by RAN3, both AMF and gNB share the same OAM configuration on supported TAI list of the on-board gNB, this is considered as an optimization.

- NG Setup procedure including the supported TAs is always expected to take place according to TS38.413.

- The solution agreed by RAN3 avoids the need for periodic and frequent RAN Configuration Update procedures. Any potential discrepancy between OAM configuration and parameters received via NG signalling, if present, is expected to be resolved via implementation.

- With respect to the frequency of updates by OAM, RAN3 believes this is in SA5 scope.

# 2.	Comments to the LS response from RAN3 in R3-250895

We have the following comments on the LS response and the solution in general:

- Frequency of updates by OAM is not clarified and therefore the signalling optimisation is unproven: In point 5 SA2 indicated that the frequency of updates by OAM was not clear. In RAN3’s answer (point A) it was indicated that that the frequency of updates is periodic and predictable but did not define what the expected frequency is. It referred to SA5 to decide the frequency (see point E). During offline RAN3 discussions it was indicated that that the period of updates, e.g., in LEO satellite scenario can be as low as 7min but the authors believe that even with this high frequency of updates saving two NG messages every 7min does not justify the extra complexity introduced. Note that changing TAIs and possibly cell-ids will require “migration” of the UEs that camped on the satellite and will have to be handed over to another satellite, this would incur in “UE associated signalling” several orders of magnitude larger than the “nodal signalling” for migration.

- Coordination across the signalling based and OAM solutions is not clear: In point B it is stated:  “…both AMF and gNB share the same OAM configuration on supported TAI list of the on-board gNB”. And in point D: “Any potential discrepancy between OAM configuration and parameters received via NG signalling, if present, is expected to be resolved via implementation”. It remains unclear how the gNB will know whether the AMF is “correctly configured” via OAM with the TAI list in order to avoid sending the RAN Configuration Update procedure when it crosses the geographic boundary that would trigger changes to new TAI. If a mismatch occurs, e.g., the AMF is not configured or is “unsynchronised”, it will have impacts in the paging procedures. Therefore, at a minimum there has to be a “handshake” to agree on TAI timetable at time of NG setup.

- Violation of “plug n’ play” principles: since Rel.8 in 3GPP it was agreed to follow "plug and play" principles which refer to the ability of a multi-vendor RAN node (eNB/gNB) to connect to the network and automatically configure itself. This concept is supported in TS 32.501 and TS 32.508. In the LS response from RAN3 in point B it is admitted that the AMF and the gNB need to share the same OAM configuration for this concept to work.

- Change of functional split between RAN and CN: currently the CN need have no knowledge of the geographic area covered by the gNB. Coverage aspects are left to RAN and should not be added to the CN.

# 3.	Proposal

Considering the points in clause 2, we think that the functionality of TAI configuration in AMF via OAM does not provide any material signalling optimisation whereas at the same time introduces significant AMF complexity, violates the “plug n’ play” principles for RAN and CN interfaces, and can lead to significant interoperability issues that RAN3 admits can only be resolved via implementation. For these reasons the authors believe that we should not pursue the functionality of TAI-gNB timetable configuration in AMF via OAM.

The above discussion and proposal also apply to the work on IoT NTN regenerative payload with the MME.

The related LS response to R3-250895 & S5-250842 is provided in S2-2503231.