3GPP TSG-SA WG2#168	S2-2503232

Gothenburg, Sweden, 7-11 April 2025	(was S2-250xxxx)

Source:	Qualcomm Incorporated

Title:	WT1:Architecture assumptions for normal IMS voice call over GEO via NB-IoT NTN connecting to EPC

Document for:	Discussion/Approval

Agenda Item:	20.1.1

Work Item / Release:	FS_5GSAT_Ph4-ARC/Rel-19

Abstract of the contribution: Proposes architecture requirements for WT1.

# 1.	Text proposal

It is proposed to agree the following changes vs. TS 23.700-19 for architecture assumptions for “normal” i.e. non-emergency IMS voice call over GEO via NB-IoT NTN connecting to EPC:

# >>>>BEGINNING OF CHANGES<<<<

# 4	Architectural Assumptions

## 4.x	Architectural Assumptions for the support of “normal” IMS voice call over GEO via NB-IoT NTN connecting to EPC

This section contains architecture assumptions for the support of "normal” i.e. non emergency, IMS voice call over GEO via NB-IoT NTN connecting to EPC:

-	Enhancements based on Control Plane CIoT EPS Optimisation, User Plane CIoT EPS Optimisation and S1-U as defined in TS 23.401 [5] will be studied. Checkpoint will be set before the normative work to align with RAN scope.

-	Architecture options with data via SGi interface as defined in TS 23.401 [5] will be considered. The option with data via SCEF/T8 will not be considered.

-	Architecture options using IP or non-IP PDN connection type with data via SGi interface will be considered.

-	Solutions should take into account that CP data (DoNAS) in current pre Rel-20 NB-IoT is regarded as NAS signalling and by default will have higher QoS than UP data (S1-U).

-	Existing mobility procedures used for intra and inter-RAT mobility for NB-IoT NTN defined in TS 23.401 [5]  will be supported. There will be no new mobility enhancements developed for the NB-IoT NTN.

-	The performance evaluation for IMS registration, IMS session setup performance targets and the QoS requirements for IMS signalling and voice packet traffic will be based on the KPIs defined in TS 22.261 [z].

# >>>>NEXT CHANGE<<<<

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G system, Stage 2".

[4]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System".

[5]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

…

[z]	3GPP TS 22.261: "Service requirements for the 5G system"

# >>>>NEXT CHANGE<<<<

# >>>>END OF CHANGES<<<<