SA WG2 Meeting #168	S2-2403320

7th – 11st, April, 2025, Stor-Göteborg, SE

Source:	OPPO

Title:	FS_5GSAT_Ph4_ARC New Key Issue: QoS Support for IMS voice service over GEO satellite

Document for:	Approval

Agenda Item: 	20.1.1

Work Item / Release:	FS_5GSAT_Ph4_ARC/Rel-20

Abstract of the contribution: This Key Issue proposed to address the WT1.1 Whether and how to support QoS for IMS voice service over GEO satellite in objective of Sat-ph4 SID (SP-250400), to comply with the relevant SA1 requirements.

1	Discussion

This Key Issue proposed to address the WT1.1 Whether and how to support QoS for IMS voice service over GEO satellite.

This Key Issue proposed to address the WT1.1 Whether and how to support QoS for IMS voice service over GEO satellite in objective of Sat-ph4 SID (SP-250400), to comply with the relevant SA1 requirements.

SA1 has concluded the following requirements and KPIs, to be further studied by Stage-2.

The 5G system with GEO satellite access shall be able to support IMS voice communication as defined in TS 22.228 [xa].

The 5G system with GEO satellite access shall be able to provide mechanisms to optimize IMS voice (e.g., call setup, transmission overhead) and support a codec for the transfer of the voice considering the transmission data rate, latency and packet size.

Table 7.4.2-1: Performance requirements for satellite access

| Scenario | Experienced data rate (DL) | Experienced data rate (UL) | Area traffic capacity (DL) (note 1) | Area traffic capacity (UL) (note 1) | Overall user density | Activity factor | UE speed | UE type | Others |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| …… |  |  |  |  |  |  |  |  |  |
| IMS voice call using GEO (note 7) | [1-3] kbit/s | [1-3] kbit/s | N/A | N/A | N/A | N/A | TBD | Handheld | Call set up time (note 8) ≤30 s (note 9) |
| …… Note 7: All the values defined for IMS voice call using GEO satellite access are generic for both regular and emergency calls. Note 8: call set up time refers to [xb]; Note 9: 30s is the upper bound that is derived based on the user’s patience suggestions (30s) in [xc]; |  |  |  |  |  |  |  |  |  |

2	Proposal

It is proposed to include the following changes in TR 23.700-xx V0.1.0.

* * * * Start of Changes (All text new) * * * *

5.x	Key Issue #X: QoS Support for IMS voice service over GEO satellite

5.X.1	General description

For normal IMS voice over TN, the KPI of 5Q1=1 and 5QI=5 is defined as below (from TS23.501).

| 5QI Value | Resource Type | Default Priority Level | Packet Delay Budget (NOTE 3) | Packet Error Rate | Default Maximum Data Burst Volume | Default Averaging Window | Example Services |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | GBR | 20 | 100 ms | 10-2 | N/A | 2000 ms | Conversational Voice |
| 5 | Non-GBR | 10 | 100 ms | 10-6 | N/A | N/A | IMS Signalling |

However, due to the long propagation delay for GEO satellite (~270ms), the latency required for normal IMS voice cannot be fulfilled when performing IMS voice over GEO satellite.

The bitrate (BR) is another issue to be resolved. From one aspect, the BR supported via GEO access may not fulfil the requirement for codec rate used by current IMS voice (the lowest BR is 4.75 kbps for AMR-NB). From another aspect, the current Satellite access (incl. GEO) does not support Guaranteed BR.

Besides, the NB-IoT Device has very low number of EPS bearer supported (less than 2), which makes a NB-IoT Device difficult to perform data service and IMS voice together.

The following points are expected to be studied within this key issue:

-  How to support EPS bearer(s) with proper QoS parameters (latency, Bit Rate) for IMS voice over GEO satellite?

-  How to transfer IMS signalling and IMS voice data via the EPS bearer for NB-IoT device?

-  What is the impact to the procedure of CIoT EPS optimizations and/or other EPS procedures?

NOTE 1:	both CP and UP based NB IoT NTN solutions will be studied.

NOTE 2:	Only architecture options with data via SGi will be considered in the study. The option with data via SCEF/T8 will not be considered.

NOTE 3: Coordination with SA1, SA4 or RAN WGs will be performed, if needed.

* * * * End of Changes * * * *