3GPP TSG-SA2 Meeting #168	S2-2503346

Stor-Göteborg, Sweden, 7th Apr 2025 - 11th Apr 2025

Source:	Nokia

Title:	KI #1: New Sol: U-plane-based IMS service via NB-IoT access

Document for:	Approval

Agenda Item:	20.1.1

Work Item / Release:	FS_5GSAT_Ph4_ARC / Rel-20

Abstract of the contribution: The contribution proposes a solution to KI#1: Support of QoS in NB-IoT access for IMS voice over GEO satellite.

# Introduction

This contribution proposes EPC architecture enhancement to support IMS voice service via NB-IoT access.

# Discussion

The proposed architecture is defined by considering the current limitations of NB-IoT access UEs on the creation of a dedicated bearer based on IP-CAN session required for IMS voice call setup. The current NB-IoT access does not support the creation of a GBR bearer. The solution proposes to remove those restrictions to allow user plane-based CIoT optimization to have two bearers, one for IMS signalling and another one for IMS voice.

The proposed solution represents the creation of a dedicated bearer in EPC to achieve IMS voice service via U-plane.

# Proposal

A solution is proposed for KI#1 for incorporation in the FS_5GSAT_Ph4_ARC TR 23.700-19.

*** Start of changes (all new text) ***

## 6.X	Solution #X: U-plane based IMS service via NB-IoT access

### 6.X.1	Key Issue mapping

This solution aims to resolve Key Issue #1, " Support of QoS in NB-IoT access for IMS voice over GEO satellite ".

### 6.X.2	Description

In this proposal, the use of user plane CIoT EPS Optimisation is assumed to provide IMS voice services via 1 default and 1 dedicated bearer.

Figure 1 UE - P GW user plane with E-UTRAN

### 6.X.3	Procedures

#### 6.X.3.1	Impacts on CIoT EPS Optimisations

As per TS 23.401 [x], if the UE indicates support for the User Plane CIoT EPS Optimisation, then it shall also indicate support for the S1-U data transfer.

#### 6.X.3.2	Attach or Tracking area Update procedure impact

The GEO satellite access RAN node needs to broadcast its support for User Plane CIoT EPS Optimisation in system information broadcast to enable the UE to choose right satellite for achieving voice service.

During Attach, the UE need to include the information element "Voice domain preference and UE's usage setting". The voice domain preference for E-UTRAN needs to indicate “IMS PS Voice only” and UE’s usage setting as voice-centric.

The UE also needs to include a Preferred Network Behaviour indication as “User Plane CIoT EPS Optimisation is supported” along with “S1-U data transfer is supported”.

The UE that wants to avail voice service shall indicate the PDN type as IPv4, IPv4/IPv6 or IPv6 in the ESM container and set the APN as IMS.

#### 6.X.3.3	Dedicated bearer activation for IMS signalling/media

If the UE has set "Attach without PDN Connectivity is supported" in the Preferred Network behaviour at attach time and the network has acknowledged its support to the UE or the UE wants to create a dedicated bearer specifically for IMS signalling, then UE can execute UE requested PDN connectivity procedure as described in clause 5.10.2 in 23.401 [x].

The procedure described in clause 5.4.1 in 23.401[x], will also be applicable for UEs accessing via NB-IoT access.

The IP-CAN session can be allowed only for access type (i.e. NB-IoT) and RAT type (i.e. E-UTRAN(GEO)) UEs to allow creation of a dedicated bearer for IMS signalling and IMS media.

### 6.X.4	Impacts on services, entities and interfaces

UE:

- Need to support user plane CIoT optimization with at least one dedicated bearer.

- Need to support IMS APN bearer creation for IMS signaling over NB-IoT access in EPC.

- Need to support dedicated bearer over NB-IoT access for GBR QoS.

MME:

- Need to support additional bearer creation other than default bearer to support IMS voice for UE with NB-IoT access.

PGW:

- Need to support additional bearer creation other than default bearer to support IMS voice for UE with NB-IoT access.

eNB:

- Need to support GBR bearer for NB-IoT access as per QoS need for GEO access.

*** END of changes ***