SA WG2 Meeting #168	S2-2503413

April 7 - 11, 2025, Goteborg, SE

Source:	China Mobile

Title:	Discussion on Architecture Assumption for ISAC study

Document for:	Discussion

Agenda Item: 	20.2.1

Work Item / Release:	FS_ISAC/Rel-20

Abstract of the contribution: This document discusses architecture assumptions for the ISAC study to support sensing service management and sensing data management.

1. Introduction

The enhancement of architecture to support sensing was mentioned during the SID discussion. However, considering the evolution of sensing architecture and the alignment of sensing service logic, the design of functional modules and interfaces for the Sensing Function (SF) requires further discussion.

2. Discussion

In summary, besides architectural enhancements for sensing, functional enhancements related to the ISAC study include two primary aspects:

- Sensing Service Control: Authorization and revocation of sensing services, discovery and selection of sensing entities based on service requirements, and configuration parameter/policy provisioning to support ISAC services.

- Sensing Data Management: Collection and transport mechanisms for sensing data and associated information to enable result calculation, as well as mechanisms for exposing sensing results and contextual information (e.g., one-time, periodic, or event-based reporting).

Communication network evolution follows the principle of Control-User (C-U) or Control-Data (C-D) separation. Sensing result calculation demands significantly more computational resources than signaling handling, while sensing measurement data transmission consumes greater network resources.

Integrating the Sensing Function (SF) instead of splitting it into control and data management modules may lead to scalability issues. Deploying a single SF on physical/virtual servers with limited computational resources would require simultaneous deployment of control-plane and data-processing entities. However, demands for sensing control and data-processing functions do not scale proportionally, resulting in network resource waste. Additionally, these functions should be implemented through distinct interfaces due to differences in signaling characteristics, packet size requirements, and solution approaches.

Potential Issues with Function Splitting:

- Service-Level Message Interaction: Mechanisms for initiating, adjusting, or terminating sensing tasks across different sensing functions.

- Service-Level Reporting: Determining whether sensing results should be exposed via data-processing or control-plane entities.

- Data Transport Protocol: A low-latency, high-efficiency protocol (e.g., UDP) is preferable to TCP for faster data transmission.

3. Conclusion and proposal(s)

Proposal 1: This architecture should enable Sensing Function split to have a control plane entity (e.g., SF-C) for sensing service management and control and a data processing entity (e.g., SF-U) for sensing result calculation.

Proposal 2: It is expected that sensing Tx/Rx(s) need to transmit sensing measurement data to SF-U via a separated user plane interface, e.g., via a node-level GTP-U tunnel if the Rx is a gNB.