TSG SA WG2 Meeting #168	S2-2503544

07 - 11 April, 2025, Goteborg, SE

Source:	Lenovo

Title:	New KI: sensing result exposure

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing-ARC / Rel-20

Abstract of the contribution: Proposes KI description for sensing result exposure

1	Background

According to the SP-250401[1], the study on Integrated Sensing and Communciation targeted at Rel-20 supports 5GS architectural and functional extensions so that sensing service can be supported. One of the objectives that this study attempts to address is:

-	WT-5: Mechanisms for providing sensing associated information and result (including sensing result and contextual information) exposure, for one time, periodic or event based reporting.

In this paper, sensing result exposure is discussed.

2	Discussion

2.1 WT-5 reconsideration

In TS 22.137 [2] , Sensing assistance information and sensing contextual information are defined as follows,

Sensing assistance information: information that is provided to the 5G system from a trusted third-party and can be used to support the derivation of a sensing result. This information does not contain 3GPP sensing data. NOTE 1: Examples of sensing assistance information are map information, area information, a UE ID attached to or in the proximity of the sensing target, UE position information, UE velocity information etc. Sensing contextual information: information that is exposed with the sensing results by 5G system to a trusted third-party which provides context to the conditions under which the sensing results were derived. This information does not contain 3GPP sensing data. NOTE 2: Examples includes map information, area information, time of capture, UE location and ID. This contextual information can be required in scenarios where the sensing result is to be combined with data from other sources outside the 5GS.

That is, sensing assistance information is provided to the 5G system form a trusted third-party and can be used for sensing result calculation. Meanwhile, sensing contextual information is exposed with the sensing result by 5G sytem to a trusted third-party. Based on the examples, we have obtained the following observation,

Observation 1: Sensing assistance information and sensing contextual information defined in TS 22.137 share many parameters, e.g., map information, area information, UE ID and UE location etc.

It is assumed that AF request includes sensing task ID, sensing assistance inforamtion, sensing requirement and sensing reporting configuration etc. For sensing result exposure, 5GC NF (either NEF or Sensing Function) needs to provide both sensing task ID (as the reference ID) and the sensing results. Then the trusted third-party is able to obtain the shared parameters (e.g., map information, area information, UE ID and UE location etc.) if contained in sensing assistance information based on the sensing task ID. However, sensing contextual information includes other parameters (e.g., time of capture) which the trusted third-party is not able to obtain by referring the sensing task ID.

Observation 2: Sensing contextual information contains other parameters (e.g., time of capture) which is not covered by the sensing assistance information.

In WT-5, the objective is to study mechanisms for providing A)  sensing associated information and B) the 5GC exposing the sensing result and optionally contextual information. As defined in TS 22.137 [2], the reason why sensing contextual information may be exposed to the trusted third-party is that it provides context to the conditions under which the sensing results were derived. Besides, as “sensing associated information” and “sensing contextual information” share many parameters, it is not necessary to include A) in WT-5.

Proposal 1: it is proposed to remove the “sensing associated information” in WT-5.

2.2 Sensing result exposure

In current specification, the Network Exposure Function (NEF) supports external exposure of capabilities of network functions. External exposure can be categorized as Monitoring capability, Provisioning capability, Policy/Charging capability, Analytics reporting capability and Member UE selection capability. Besides, user plane direct 5GS information exposure function allows the UPF to report the network information directly to consumer based on the instructions provided by SMF. The user plane direct 5GS information exposure includes QoS monitoring information and TSC Manamgent Information.

For sensing service, it is assumed that a new functionality in the 5GC (e.g., a new Sensing Function (SF)) collects sensing data and sensing assistane information from one or more sensing radio measurement entities, trusted third-party AFs and/or other NFs. It is assumed that NEF needs to authorize the AF request first and it is FFS whehter NEF performs sensing results (including sensing result and sensing contextual information) exposure, or the new NF(e.g., SF) exposes sensing results directly.

Proposal 2: it is proposed to investigate whether and how NEF and/or a new sensing functionality in the 5GC (e.g., SF) performs sensing result exposure.

For QoS monitoring as an example, reporting frequency is defined as follows in clause 6.1.3.21 of TS 23.503 [3].

Reporting frequency (event triggered, periodic): - if the reporting frequency is event triggered: - the corresponding reporting threshold to each QoS Monitoring parameter; - minimum waiting time between subsequent reports; - the reporting period;

That is, it is one time reporting if no reporting frequency is provided. It is periodic reporting if the reporting period is provided. And it is event based reporting if reporting threshold is provided.

Proposal 3: It is proposed that the existing functionalities for event reporting (e.g. reporting frequency ) can be reused whenever possible for the sensing reporting configuration.

3 Reference

[1] SP-250401, New SID on Study on Stage 2 for Integrated Sensing and Communication, Xiaomi

[2] TS 22.137, 3rd Generation Partnership Project; Technical Specification Group TSG SA; Service requirements for Integrated Sensing and Communication; Stage 1

[3] TS 23.503, 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Policy and charging control framework for the 5G System (5GS); Stage 2

4 Text Proposal

It is proposed to capture the following changes to TR 23.700-14.

* * * * Start of Change * * * *

## 5.X	Key Issue X: < Sensing result exposure >

### 5.X.1	Description

This key issue will address sensing result exposure, which includes sensing result and optionally sensing contextual information. Sensing contextual information is exposed with the sensing results by 5G system to a trusted third-party which provides context to the conditions under which the sensing results were derived. The key issue will study the following aspects:

-  Investigate whether and how the 5GC (e.g. NEF and/or a new sensing functionality) performs sensing result (including sensing contextual information) exposure?

-  Whether and how to enhance the existing scheme to support sensing reporting (e.g., one time, periodic or event based reporting)?

NOTE:	Existing functionalities for event reporting (e.g. reporting frequency) can be reused whenever possible for the sensing reporting configuration.

* * * * End of Change * * * *