3GPP TSG-WG SA2 Meeting #168	S2-2503609

Goteborg, Sweden, 07 – 11April 2025

Source:	HONOR

Title:	Update to clause 5.7 Identifiers

Document for:	Approval

Agenda Item:	19.14.2

Work Item / Release:	AmbientIoT-ARC / Rel-19

Abstract: Update to clause 5.7: Identifiers and add the content about the AIoT device temporary identifier.

1. Discussion

SA WG3 concludes to support the requirement on AIoT Device privacy and solution principle is using “Temporary ID” in LS reply S2-2501384 and S3-251048：

The following aspects and principles are agreed for the conclusion on KI#3

-	A mechanism to protect AIoT device ID based on the use of temporary ID shall be supported.

-	Mechanism shall allow unambiguous identification of the AIoT device

-	A mechanism to re-synchronize de-synchronized temporary IDs shall be supported.

Editor’s Note: Additional conclusions on solution are FFS.

In TR 33.713, there are mainly two methods to allocate an AIoT device temporary identifier to the AIOT device:

Method 1: The AIoT device temporary identifier is locally generated both by the CN (e.g. the AIOTF) and the AIoT device according to security mechanism designed by SA WG3.

Method 2: The AIoT device temporary identifier is allocated by the CN (e.g. the AIOTF) according to security mechanism designed by SA WG3.

We consider that which method is applied depends on SA WG3 conclusion and SA WG2 should align with SA WG3.

Hence, SA2 needs to support the AIoT device temporary identifier based solution from architectural perspective in normative phase and is better to have minimal effect on existing architecture and procedure design.

Observation: SA WG2 needs to support the AIoT device temporary identifier based solution from architectural perspective and align with SA WG3 during the normative phase.

Generally, the following principles should be considered in SA WG2:

- The AIoT Device privacy principle concluded by SA3 is that AIoT device ID is protected based on the use of temporary ID. So that SA2 should align with SA3 conclusions and the network should not use the AIoT device permanent identifier to page the AIoT Device during AIoT inventory service operation and AIoT service command operation.

- According to SA3 solutions in TR 33.713, the AIoT device temporary identifier is a privacy preserving identifier containing the concealed the AIoT device permanent identifier so that the AIoT device temporary identifier is used only inside 3GPP system.

- When the AIoT device temporary identifier is available both in the AIoT device side and the network side, based on SA WG3 conclusion, the AIOTF should use the AIoT device temporary identifier(s) instead of the AIoT device permanent identifier to trigger the AIoT Reader to perform AIoT service operations towards the AIoT Device(s). There is implied 1-to-1 relationship between an AIoT device permanent identifier and an AIoT device temporary identifier. Hence the AIOTF should support to store the mapping relationship between the AIoT device temporary identifier and the AIoT device permanent identifier, store the AIoT device temporary identifier in AIoT device context and/or AIoT device profile data.

2. Text Proposal

It is proposed to capture the following changes vs. TS 23.369.

# * * * * First change (all new texts) * * * *

5.7.x	AIoT Device Temporary Identifier

The 5G system supports allocation of the AIoT device temporary identifier in order to support the AIoT device ID privacy protection. The AIoT device temporary identifier is a privacy preserving identifier containing the concealed the AIoT device permanent identifier and has the 1-to-1 relationship with the AIoT device permanent identifier. The AIoT device temporary identifier is used to uniquely identify an AIoT device in the MNO domain and is used only inside 3GPP system.

Editor’s Note:	How the AIoT device permanent identifier is concealed will be determined by SA WG3.

When the AF provides the AIoT device permanent identifier(s) in the AIoT service operation request and the AIoT device temporary identifier(s) has been allocated, the AIOTF should use the AIoT device temporary identifier(s) to trigger the AIoT Reader to perform AIoT service operations towards the AIoT Device(s).

Editor’s Note:	How the AIoT device temporary identifier is allocated/reallocated needs to align with SA WG3.

# * * * * End of changes * * * *