3GPP TSG-SA2 Meeting #168	S2-2503636

Goteborg, Apr 7th – 11th 2025

Source:	Huawei, HiSilicon

Title:	WT#3: NF selection profile and UPF selection

Document for:	Approval

Agenda Item:	20.4.1

Work Item / Release:	FS_EnergySys_Ph2 / Rel-20

Abstract: This document proposed the NF selection as the solution of the KI#3.

# 1.	Introduction

In the approved SID SP-250417, the NF selection is considered as one of the candidate key issues to be resolved for energy study in Rel-20:

- WT#3: Whether and how to support NF (re)selection and/or UP path adjustment based on energy related information

Current NF selection defined in clause 6.3 of TS 23.501 mainly considers criteria such as Location information, Data Network Name, slicing information, capability, local policy, load condition etc. There is no consideration on energy related criteria, such as maximize the energy efficiency of the network, trade the performance with energy consumption following the SLA agreement, etc., and may lead to a sub-optimal result from the energy perspective.

Since in Rel-19, the NF selection has already been discussed well, the resolution for Rel-19 is a good starting point for the discussion for Rel-20. This document uses Rel-19 resolution as the starting point and proposes the associating solution for KI#3, especially on the following aspects:

-	Scheduled switch-off time. Essential new parameters for NF selection. This is the information that whether the NF is subject to energy related execution present or future (e.g., deactivated/turned off because of the energy consumption in the off-peak hours), and it may also include the possibility (or the priority) to be subject to energy related execution, and the interval of the energy related execution.	As such, NF profile can be enhanced to guarantee a flexible NF selection regarding energy. The PDU Session Anchor is not expected to be changed during the lifetime of the PDU Session for SSC mode 1. However, energy related executions may require the NF to be switched to "inactive"/"energy saving" or other state to save energy or improve energy efficiency, which could result in the unavailability of the UPF. Therefore, it may negatively affect the flexibility of switching off the UPFs for energy saving, if more UPFs are managing PDU session with SSC mode1. Those UPFs should be deprioritized to be chosen as the PDU Session Anchor of the PDU session with SSC mode 1.

-	Energy Priority Information. Additional and optional for the selection. It indicates the priority relative to other NFs of the same type considering the energy characteristics of the NF. It is determined based on operator policy and provided to NRF considering also the various energy characteristics of the NF, for example the energy consumption in function of the load, the actual energy consumption, whether is powered with renewable energy, etc. Which is the energy related information and how they are considered in order to determine the Energy Priority Information parameter is based on operator configurations.

This solution proposes to enhance the NF discovery and selection related functionalities for energy aware NF selection.

# 2. Text Proposal

It is proposed to capture the following changes in TR 23.700-67.

# * * * * First change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[X1]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[X2]	3GPP TS 23.502: "Procedures for the 5G system, Stage 2".

# * * * * Second change * * * *

# 6	Solutions

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |
| --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 |
| X |  |  | X |
| Y |  |  | X |

# * * * * Third change (All text new) * * * *

## 6.X	Solution #X: Energy related information in NF profile

This solution is for Key Issue #3: Whether and how to support NF (re)selection based on energy related information

### 6.X.1	Functional Description

This solution proposes to enhance the NF selection related functionalities for energy aware NF selection.

The following solution components are envisaged:

-	Service provider NF registers new energy related information in its NF profile at the NRF and updating such information at the NRF when needed. Or alternatively, such new energy related information can be (re-) configured by OAM.

-	Enhancing the NF Discovery procedure with new energy-related NF discovery query parameters (e.g. energy aware NF discovery policy/criteria) and based on new energy-related policies at the NRF;

-	NF service consumer taking into account the new energy related information from the NF profiles (defined in Table 6.X.1-1) discovered from NRF using discovery procedure (and subsequent updates if subscribed to), for the selection of a target 5GC NF;

NRF maintains also energy related information as part of NF profile for registered NF instance. Such energy related information includes the example in Table 6.X.1-1 described.

Table 6.X.1-1: Energy related information

| Information | Description | Category |
| --- | --- | --- |
| Scheduled switch-off time | The schedule of the energy states, e.g., the time of entering to/exiting from the Energy Saving State regarding energy saving. (NOTE 1) | Optional |
| Energy Priority Information | This parameter indicates the priority relative to other NFs of the same type considering the energy characteristics of the NF. | Optional |
| NOTE 1: Energy states refers to state of a cell, a network element and/or a network function with respect to energy, e.g. (not) energy saving states, which are defined in TS 28.310. This can be used for e.g., UPF selection for PDU Session with SSC mode 1, and SMF may not select the UPF about to be in energySaving state in that case. |  |  |

The Energy Priority Information indicates the priority relative to other NFs of the same type considering the energy characteristics of the NF.

The principles of Energy Priority Information are:

-	If the energy is a criteria for selection of a NF, the Consumer provide such criteria altogether with the other criterias and if more them one NF matches the criteria the Energy Priority Information parameter can be used to select those with higher priority

-	The definition of the priority is determined by the Operator based on their policies and considering also the various energy characteristics of the NF, some possible example (not limited to the list below)

-	The NFs with renewable power supply have higher Energy Priority, to the NF with higher renewable ratio can be assigned higher priority

-	The energy priority can be higher for NF more efficient

-	The energy energy priority can be changed when the NF enters in Energy Saving state

-	Any combination of energy considerations can be used to determine the Energy Priority

The consumer NF of NRF discovery service may select the NF or NF instance considering the energy information in the discovered NF profile.

If the energy is a criterion for the selection of the NF in case of multiple NF satisfying the other criteria, the Energy Priority can be considered for selecting the NF with high priority. If the energy is a criterion for the selection whether the Energy Priority takes precedence on the NF priority is based on operator policy.

NOTE:	The information within NF profile specified in Table 6.X.1-1 is expected not to change frequently, e.g., constantly reflecting changes in energy consumption.

### 6.X.2	Impact

The impacts are :

-	NRF

-	support the NF profile extension

-	extension of SBI to send request of NF based on energy indication support

-	other NFs

-	extension of SBI to send request of NF based on energy indication support

And when SMF selects the UPF, the SMF can also make use of the NF Profiles (i.e., Scheduled switch-off time) by considering the potential usage of the UPF for energy related execution. The UPF with potential usage of energy related execution should be deprioritized to be chosen as the PDU Session Anchor of the PDU session with SSC mode 1.

### 6.X.3	Procedures

#### 6.X.3.1	Selection of the UPF with considering energy related information

The information to be considered for UPF selection are defined in clause 6.3.3 of TS 23.501 [X1]. When SMF selects the UPF, the SMF can also make use of the NF Profiles by considering the potential usage of the UPF for energy related execution. The UPF with potential usage of energy related execution should be deprioritized to be chosen as the PDU Session Anchor of the PDU session with SSC mode 1.

Figure 6.X.3.1-1: Selection of the UPF with considering energy related information.

0.	SMF provisioning of UPF instances using NRF.

1.	The UE registers to the PLMN, as described in clause 4.2.2.2 of TS 23.502 [X2].

2.	The PCF makes use of UE Configuration Update procedure to provide UE Policy as defined in clause 4.2.4.3 of TS 23.502 [X2].

3.	UE initiates the UE Requested PDU Session Establishment procedure with including the necessary information, including the requested SSC mode.

4.	Upon UPF selection, the SMF selects the UPF based on its Additional Energy Information in the NF profile and the additional factors related to Additional Energy Information (Energy Efficiency and Energy saving parameters e.g. NF energy states, analytics, Energy Cost etc.). If the selected SSC mode of the PDU session is SSC mode 1, the SMF selects the UPF that is not subject to energy related execution, or selects the UPF with lower possibility/priority subject to energy related execution.

# * * * * 4th change (All text new) * * * *

## 6.y	Solution #y: Selection of the UPF with considering energy related information

This solution is for Key Issue #3: Whether and how to support UP path adjustment based on energy related information.

When SMF selects the UPF, the SMF can also make use of the NF Profiles (i.e., Scheduled switch-off time) by considering the potential usage of the UPF for energy related execution. The UPF with potential usage of energy related execution should be deprioritized to be chosen as the PDU Session Anchor of the PDU session with SSC mode 1.

### 6.X.3	Procedures

The information to be considered for UPF selection are defined in clause 6.3.3 of TS 23.501 [X1]. When SMF selects the UPF, the SMF can also make use of the NF Profiles by considering the potential usage of the UPF for energy related execution. The UPF with potential usage of energy related execution should be deprioritized to be chosen as the PDU Session Anchor of the PDU session with SSC mode 1.

Figure 6.X.3.1-1: Selection of the UPF with considering energy related information.

0.	SMF provisioning of UPF instances using NRF.

1.	The UE registers to the PLMN, as described in clause 4.2.2.2 of TS 23.502 [X2].

2.	The PCF makes use of UE Configuration Update procedure to provide UE Policy as defined in clause 4.2.4.3 of TS 23.502 [X2].

3.	UE initiates the UE Requested PDU Session Establishment procedure with including the necessary information, including the requested SSC mode.

4.	The SMF selects the UPF interacting with the NRF based on energy information in the NF profile, for example when the UE has a energy subscription or based on SMF configuration related to Network Energy Saving. If the selected SSC mode of the PDU session is SSC mode 1, the SMF may select the UPF that is not subject to energy related execution, or selects the UPF with lower possibility/priority subject to energy related execution.

5-6	These steps are the step in PDU session establishment procedure in TS 23.502.

### 6.X.2	Impact

### X.2	Impact

The impacts are :

-	NRF

-	support the NF profile extension for energy based information

-	extension of SBI to send request of NF based on energy indication support

-	SMF

-	support of UPF selection based on energy information

-	support of SBI extension toward NRF

# * * * * End of change * * * *