3GPP TSG|WG-SA2 Meeting #168	S2-2503695

Goteburg, Sweden, April 07 – 11, 2025	(revision of S2-20xxxx)

Source:	Samsung

Title:	Discussion on AsInfoGet service operation

Agenda Item:	19.32

Document for:	Information

# 1.	Background

For the NG_RTC_Ph2 feature, there is mechanism defined how the DC AS can provide the request to NEF initiate one standalone bootstrap DC session or adding application DC to an existing IMS session etc. In this case NEF queries the HSS to know the IMS AS address serving the user (i.e. IMPU) by using the Nhss_ImsUECM_AsInfoGet service operation.

# 2.	Discussion

As described in the background Nhss_ImsUECM_AsInfoGet service operation is used by NEF to know the IMS AS address serving the user but in the SA2#167 meeting there was a proposal (S2-2501505) to change the design of the existing service operation. The proposed new service operation is Nhss_ImsUECM_Get

The requirement here is NEF to get IMS AS address which serves the user. HSS as NF service producer can not provide IMS AS address only if the proposed solution of Nhss_ImsUECM_Get with NF type set to NEF because IMS AS is information is not only applicable for the NEF. In future DCSF can query the IMS AS address of an user from HSS where the NF type will be DCSF. Moreover even NEF also in future may need some other information from HSS.

Observation 1:  IMS AS address information is not exclusively applicable for NEF, the proposal of NF type based input to retrieve information in IMS would not work.

The intention of the proposal is to make it generic and provide some specific information to the service consumer based on the NF type which is basically similar to the Nudm_UECM_Get service operation from 5GC. In 5GC there are many Network functions and some specific informations are applicable only for relevant Network Functions like Analytics information is applicable for NWDAF, Access Type if NF is AMF, SMS address if NF is SMSF but in IMS there are only few NFs for which this kind of use case is applicable (e.g. The Restoration Info is applicable for S-CSCF only) in IMS.

The Nhss_ImsUECM_RestorationInfoGet service operation is already designed in IMS using which S-CSCF retrieves the stored information from HSS. If the intention is to make generic retrieval instead of some specific information from HSS, the proposed solution should have been considered (i.e. Nhss_ImsUECM_Get with NF type set to S-CSCF) during that time. As the use cases and information associated with network functions are different in IMS and 5GC, it seems the service operation were designed in IMS to retrieve information using specific service operation.

Observation 2: As the use cases are different in IMS and 5GC and moreover the existing service operation design to retrieve restoration information is specific, similar approach would make the system very easy and simple.

If really we want to make it generic like 5GC, then existing service operation Nhss_ImsUECM_RestorationInfoGet will be obsolete and need to be deleted because the new proposal of Nhss_ImsUECM_Get with NF type set to S-CSCF, HSS will provide restoration information.

Observation 3: Deleting the existing service operation from older release to make the new proposal work break the backward compatibility.

In 5GC, the Nudm_UECM_Get service operation is used by NEF to retrieve (for e.g. AMF address of the UE), similarly if we agree to current proposal, the HSS will provide complete information from which NEF need to filter out the IMS AS address. But the design was not to have additional complexity at NEF from IMS side and hence using specific service operation Nhss_ImsUECM_AsInfoGet will give direct the IMS AS address

Observation 4: Agreeing to Nhss_ImsUECM_Get service operation will have additional complexity and support to NEF to filter out the IMS AS address from the entire received information from HSS.

# 3.	Proposal.

Proposal: By looking at the all four observations, it is proposed not to change the existing Nhss_ImsUECM_AsInfoGet service operation