3GPP TSG-SA WG2 Meeting #168	S2-2503764

# 7-11 April, 2025, Goteborg, Sweden	(Revision of S2-250xxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1643 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on UE-sat-UE communication procedures |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3-ARC |  |  |  |  |  | Date: |  |  | 2025-03-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Current UE-sat-UE communication procedure for call setup only describes how P-CSCF does event subscription/notification and media authorization to PCF via N5 interface. How P-CSCF does event subscription/notification and media authorization to PCF via Rx interface is missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add descriptions of using Rx interface for event subscription/notification and media authorization in call setup procedure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | AE.5.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

## AE.5.1	At call setup

When the originating P-CSCF receives the initial INVITE request from the originating UE using regenerative NR satellite access, it selects the IMS-AGW on ground and triggers media negotiation by sending SDP offer to the terminating side indicating that satellite access is used on the originating side. If the terminating P-CSCF activates UE-satellite-UE communication, the originating P-CSCF re-selects an IMS-AGW on satellite. See Figure AE.5.1-1 that describes the procedure. In the procedure, UE A and P-CSCF A represent originating side function or entity, UE B and P-CSCF B represent terminating side function or entity.

NOTE 1:	To support the UE-satellite-UE communication, P-CSCFs (AF) need to support N5 interface for AF influence on traffic routing and user plane management event subscription/notification.

Figure AE.5.1-1: Session establishment procedure with activation of optimizing media routing

1.	UE A sends a SIP INVITE request, containing an initial SDP offer, to P-CSCF A. The access information in the SIP INVITE request indicates that the UE is accessing from NR satellites access as defined in TS 24.229 [10a].

2.	After receiving the SIP INVITE request indicating NR satellite access, if UE A is not roaming and the P-CSCF A supports UE-Satellite-UE communication, the P-CSCF requests Access Network Information, which includes an indication for requesting satellite identifier information, for UE A from the PCF as described in TS 23.503 [95] via Npcf_PolicyAuthorisation_Create service operation or via Rx interface as specified in Annex C of TS 23.503 [95]. The PCF requests Access Network Information, which includes the identifier of UE A's serving satellite, from the SMF via Npcf_SMPolicyControl_UpdateNotify service operation. Then the PCF receives the Access Network Information via Npcf_SMPolicyControl_Update service operation as described in TS 23.502 [94] and TS 23.503 [95]. This satellite identifier is subsequently conveyed to the P-CSCF in the Access Network Information Notification via Npcf_PolicyAuthorization_Notify service operation.

3.	The P-CSCF A allocates an IMS-AGWon the ground as default. P-CSCF A interacts with IMS-AGW on the ground to allocate transport resources based on the mechanism defined in TS 23.334 [74].

Steps 4-20 are performed if the UE-Satellite-UE communication is possible based on step 2 (i.e. UE A is using NR regenerative payload satellite access and is not roaming). Otherwise, procedures for IMS user plane traffic routing with IMS-AGW on ground are performed (details have been omitted from the diagram).

4.	P-CSCF A updates the SIP INVITE request inserting the identifier of the satellite serving UE A and updates SDP offer with the IMS-AGW transport addresses allocated in step 3. P-CSCF A forwards the updated SIP INVITE request to the terminating side via IMS core (details have been omitted from the diagram).

5.	After receiving the SIP INVITE request which contains a satellite identifier indicating that UE A is using regenerative satellite access, if UE B is not roaming and the P-CSCF B supports UE-Satellite-UE communication, the P-CSCF B requests Access Network Information, which includes an indication for requesting satellite identifier information, for UE B from PCF as described in TS 23.503 [95] via Npcf_PolicyAuthorization_Create service operation, or via Rx interface as specified in Annex C of TS 23.503 [95]. The PCF obtains Access Network Information, which may include an identifier of the satellite serving UE B, from the SMF via Npcf_SMPolicyControl_UpdateNotify and Npcf_SMPolicyControl_Update service operations as described in TS 23.502 [94] and TS 23.503 [95]. This satellite identifier, if received, is subsequently conveyed to the P-CSCF in the Access Network Information Notification via Npcf_PolicyAuthorization_Notify service operation.

NOTE 2:	P-CSCF B can decide to include an indication for requesting satellite identifier information based on e.g. access information in the SIP REGISTER request (or other SIP messages) which indicates that UE B is accessing from NR satellites access as defined in TS 24.229 [10a].

6.	Based on the Access Network Information received at step 5, if UE B is using regenerative payload satellite access and if the two satellites identified in step 4 and 5 are the same or connected with ISL(s), P-CSCF B determines to activate the UE-Satellite-UE communication in IMS.

and	If UE-Satellite-UE communications in IMS is not activated, P-CSCF B selects an IMS-AGW on ground for UE B; otherwise, P-CSCF B selects an IMS-AGW on satellite for UE B.

NOTE 3:	How the terminating P-CSCF uses the satellite identifiers to derive whether the two satellites are connected with ISL(s) and are equipped with UPF and IMS-AGW is based on implementation or operators' policy e.g. by querying transport network system or based on node level information reported by the IMS-AGW deployed on the satellites.

6a-6b.	Based on the decision in step 6, P-CSCF B interacts with IMS-AGW onboard the satellite or IMS-AGW on ground to allocate transport resources based on the mechanism defined in TS 23.334 [74].

Steps 7-20 are performed if the UE-Satellite-UE communication is activated. Otherwise, SIP message routing procedures with IMS-AGW on the ground should be performed after P-CSCF B selecting a ground IMS-AGW for usage (details have been omitted from the diagram).

7-9.	P-CSCF B forwards the updated SIP INVITE request to UE B. UE B returns an 18X response with SDP Answer as normal SIP message routing procedures.

9a-9b.	After receiving the SDP Answer, P-CSCF B interacts with the IMS-AGW onboard the satellite to allocate transport resources based on the mechanism defined in TS 23.334 [74].

10.	P-CSCF B instructs the PCF to authorize the resources necessary to establish a QoS flow for media via Npcf_PolicyAuthorization_Update service operation or via Rx interface as specified in Y.2.3.1 of this specification. To indicate the SMF to insert a UL CL/BP and L-PSA UPF for IMS PDU session to enable UE-Satellite-UE communication, the P-CSCF B should send following parameters via the request to PCF based on Application Function influence on traffic routing mechanism described in clause 5.6.7 of TS 23.501 [93] and clause 4.3.6 of TS 23.502 [94]:

-	A flow description information which contains the IP address of the IMS-AGW onboard the satellite;

-	A DNAI associated with the IMS-AGW onboard the satellite and optionally corresponding N6 traffic routing information.

NOTE 4:	DNAI value can be derived from the satellite identifier as per operator policy and implementation.

10a.	To be notified of satellite change and perform IMS-AGW relocation at change of satellite as described in clause AE.5.2, P-CSCF B subscribes to notification of User Plane management events (i.e. UP path change) via Npcf_PolicyAuthorization_Update service operation to the PCF, or via Rx interface as specified in Annex C of TS 23.503 [95], which in turn subscribes to the events from SMF via Npcf_SMPolicyControl_UpdateNotify service operation and receives the event notifications from the SMF via Nsmf_EventExposure_Notify service operation as specified in clause 4.3.6 of TS 23.502 [94]. The P-CSCF B inserts the following parameters in its subscription request (i.e. Npcf_PolicyAuthorization_Update service operation) as specified in clause 5.2.5.3.3 of TS 23.502 [94]:

-	Early and late notifications about UP path management events.

NOTE 5:	If Rx is used by P-CSCF, implicit subscription is assumed as part of PCC rules setting.

11.	P-CSCF B updates the 18X response to carry the identifier of the satellite serving UE B to indicate that UE-Satellite-UE communication is activated. Then, P-CSCF B returns the updated 18X response to originating side via IMS core (details have been omitted from the diagram).

12.	Based on the identifier of the satellite serving UE B in step 11, P-CSCF A determines to perform UE-satellite-UE communication and select an IMS-AGW onboard the satellite.

12a-12c.	P-CSCF A interacts with IMS-AGW onboard the satellite to allocate transport resources for both IMS access and IMS Core network sides. P-CSCF A also releases the IMS-AGW on ground.

13.	P-CSCF A instructs the PCF to authorize the resources necessary to establish a QoS flow for media via Npcf_PolicyAuthorization_Update service operation or via Rx interface as specified in Y.2.3.1 of this specification. To indicate to the SMF to insert a UL CL/BP and L-PSA UPF for the IMS PDU session to enable UE-Satellite-UE communication in IMS, P-CSCF B should send following parameters via the request to PCF based on Application Function influence on traffic routing mechanism described in clause 5.6.7 of TS 23.501 [93] and clause 4.3.6 of TS 23.502 [94]:

-	A flow description information which contains the IP address of the IMS-AGW onboard satellite;

-	A DNAI associated with the IMS-AGW onboard satellite and optionally corresponding N6 traffic routing information.

NOTE 6:	DNAI value can be derived from the satellite identifier as per operator policy and implementation.

13a.	To be notified of satellite change and perform IMS-AGW relocation at change of satellite as described in clause AE.5.2, P-CSCF A subscribes to notification of user plane management events (i.e. UP path change) via Npcf_PolicyAuthorization_Update service operation to the PCF, or via Rx interface as specified in Annex C of TS 23.503 [95], which in turn subscribes to the events from SMF via Npcf_SMPolicyControl_UpdateNotify service operation and receives the event notifications from the SMF via Nsmf_EventExposure_Notify service operation as specified in clause 4.3.6 of TS 23.502 [94]. The P-CSCF A inserts the following parameters in its subscription request (i.e. Npcf_PolicyAuthorization_Update service operation) as specified in clause 5.2.5.3.3 of TS 23.502 [94]:

-	Early and late notifications about UP path management events.

NOTE 7:	If Rx is used by P-CSCF, implicit subscription is assumed as part of PCC rules setting.

14.	P-CSCF A updates the SDP answer in the 18X response with the IMS-AGW transport addresses allocated in step 12 and forwards the updated 18X response to UE A.

15.	UE A acknowledges the 18X Response and sends the Response Confirmation to P-CSCF A.

16-17.	P-CSCF A forwards the Response Confirmation to P-CSCF B. The Response Confirmation includes SDP offer modified/generated by P-CSCF A which contains the IMS-AGW transport address obtained in step 12. Upon receiving the SDP offer, P-CSCF B updates the allocated IMS-AGW on the satellite for UE B with the transport address in the SDP offer.

18.	P-CSCF B sends the Response Confirmation to UE B.

19.	Procedure continues to setup the call as defined in clauses 5.6 and 5.7.

# * * * End of changes * * * *