3GPP TSG-WG SA2 Meeting #168	S2-2503840

Goteborg, SE, 7th April – 11th April, 2025	(revision of S2-250xxxx)

Source:	Huawei, HiSilicon

Title:	pCR for TS 23.369: Address ENs for AF authorization

Document for:	Approval

Agenda Item:	19.14.2

Work Item / Release:	AmbientIoT-ARC / Rel-19

Abstract: address the EN for AF authorization

1. Introduction/Discussion

Editor's note:	Whether to perform AF authorization only in NEF or in both NEF and AIOTF (including which parameter to use in Table 5.6-1), and whether a trusted AF requires authorization is FFS.

Editor's note:	Whether AF authorization data is stored in ADM is FFS.

There are many NEF exposure services, including Event Exposure, PFD Management, Parameter Provision, Traffic Influence, Service Parameter etc, see the full list of NEF services in clause 5.2.6.1 of TS 23.502. The NEF authorization of AF request shall be generally applicable for all NEF services, including Ambient IoT service, so the NEF authorization function shall be common enough for all NEF services (e.g., check legality of the requested AF and the permission to consume a given API), thus it would be better that the NEF doesn’t look into the specific service parameters for a certain AF service request.

In the context of AIoT service, the AIOTF is defined to exclusively support the Ambient, the AIOTF is deeply involved for AIoT service handling procedure, it will interpret the Ambient service request by decoding the AIoT service parameters (e.g., requested target area, service operations or target devices etc.) in the AF request. Among the AIoT service parameters, service operations or target devices are specially defined for AIoT, the AIOTF is essential to be involved to check such AIoT service special parameters.

To allow certain-level flexibility for NF implementation, AF authorization data can either be configured in ADM or NEF, but not both.

For an AF in trusted domain, the operator can trust the AF in this certain environment and network allows bypassing the AF identity checking for the AF in trusted domain. However, service level check for the request originated by this AF is still required, otherwise unexpected system errors may occur.

Proposal: NEF performs AIoT AF-level authorization, AIOTF performs AIoT service-level authorization for both trusted AF and untrusted AF. To allow certain-level flexibility for implementation, AF authorization data can either be configured in ADM or NEF.

2. Text Proposal

It is proposed to capture the following changes vs. TS23.369.

# * * * * First change * * * *

4.5.3	AIOTF

The AIOTF supports the following functions:

-	Termination of AIoT NAS towards AIoT Device.

-	Connectivity either directly with an AIOT RAN or indirectly with the AIoT RAN via an AMF.

-	Support of AIoT service operations towards the AIoT Devices(s):

-	Providing an interface to the AF (or via NEF) for AIoT services and authorizing the AF's AIoT service operation request.

-	Triggering the AIoT RAN to perform AIoT service operations towards the AIoT Device(s), and optionally determining and providing assistance information to the AIoT RAN .

-	Report the service operation results to AF (or via NEF) based on the local configuration or the AF request.

-	AIoT RAN selection and optionally a list of AIoT RAN Reader selection for AIoT service operations.

-	AMF selection based on target area information in case of indirect connectivity between AIOTF and AIoT RAN via an AMF.

-	Correlation ID allocation corresponding to the AF service operation request.

-	Retrieving and updating AIoT device profile data from ADM.

-	Retrieving AF authorization data from ADM.

-	Optionally AIoT device context management.

# * * * * Second change * * * *

4.5.4	NEF

In addition to the functions defined in TS 23.501 [3], the NEF performs the following functions:

-	Providing a service exposure API to AFs for AIoT services and interacting with AFs.

-	Selection of AIOTF for AIoT services and interacting with the selected AIOTF.

-	Authorization of the AF’s AIoT service operation request.

# * * * * Third change * * * *

5.6	AF authorization to the AIoT Services

The information needed to support the authorization of the AF for performing the AIoT service is stored as the authorization data for 3rd party AF in the ADM, or locally configured in the NEF.

Table 5.6-1 below describes items stored as AF authorization data for the AIoT.

Table 5.6-1: AF Authorization Data for AIoT

| AF Authorization Data | Description |
| --- | --- |
| AF ID | Identifier used to identify the AF. |
| Allowed area | Indicate the allowed area for the indicated AF to perform the AIoT services operations. |
| Allowed service operations | Indicate the allowed service operation (s) for the indicated AF, e.g. inventory, read, write, permanent disable. |
| Allowed target AIoT devices | Indicate the allowed AIoT Device(s) for the indicated AF. |

The authorization to the AF for the AIoT mainly includes two parts:

-	NEF performs AIoT AF request authorization based on the service level agreement (SLA) between the 3rd party AF and the 5GS of the mobile network operator, the operator policy and local configuration as in TS 33.501 [8].

-	AIOTF may perform AIoT service level authorization of AIoT service request from the AF in either trusted or untrusted domain, by retrieving the AF authorization data stored in the ADM as described in above Table 5.6-1.

# * * * * Fourth change * * * *

7.5.2	Nadm_DM_Query service operation

Service operation name: Nadm_DM_Query

Description: NF service consumer may request the AIoT device profile data or the AF authorization data from the ADM.

Inputs, Required: AIoT Device Permanent ID, or AF ID.

Input, Optional: None.

Outputs, Required: the AIoT device profile data or the AF authorization data.

Output, Optional: None.

# * * * * End of changes * * * *