3GPP TSG SA WG2#168	            S2-2503850

Goteborg, Sweden, 07 – 11 April 2025	(revision of)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 5477 | rev | - | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarifications on I-SMF insertion for PDU Session Mobility |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eEDGE_5GC_Ph3 |  |  |  |  |  | Date: |  |  | 2025-03-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As described in clause 6.10 of TS 23.548, if Local Offloading Management is allowed as per the subscription data and the UE is within Local Offloading Management service area, the AMF inserts an I-SMF. AMF should consider I-SMF insertion for Local Offloading Management during PDU Session mobility from EPS to 5G as well as from non-3GPP to 3GPP access. These aspects are addressed by Stage 3 (C4-250255) but are missing in Stage 2 specifications. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarifies I-SMF insertion for PDU Session Mobility from EPS to 5G and non-3GPP to 3GPP access |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specification related to Local Offloading Management |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.23.12.1, 4.23.13.1, 4.23.15 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

#### 4.23.12.1	General

When UE moves from EPS to 5GS, for each PDU Session, the AMF determines whether I-SMF needs to be inserted based on UE location and service area of the SMF+PGW-C. Additionally, if Local Offloading Management is allowed as per the subscription data and the UE is within Local Offloading Management service area, the AMF selects an I-SMF as described in clause 6.10 of TS 23.548 [74].

This clause describes the N26 based Interworking Procedures with I-SMF insertion when UE move from EPS to 5GS and removal when UE moves from 5GS to EPS.

SECOND CHANGE

#### 4.23.13.1	General

When UE moves a PDU Session from EPS to 5GS, the AMF determines whether I-SMF needs to be inserted based on the UE location and the service area of the SMF+PGW-C. Additionally, if Local Offloading Management is allowed as per the subscription data and the UE is within Local Offloading Management service area, the AMF selects an I-SMF as described in clause 6.10 of TS 23.548 [74].

This clause describes the non N26 based Interworking Procedures with I-SMF insertion when UE move a PDU Session from EPS to 5GS and I-SMF removal when UE moves a PDU Session from 5GS to EPS.

This may happen e.g. when the UE moves a PDU Session between an ePDG (over EPC) and 3GPP access over 5GC.

THIRD CHANGE

### 4.23.15	PDU Session mobility between 3GPP and non-3GPP access

In this Release of the specification, deployments topologies with specific SMF Service Areas apply only for 3GPP access.

At PDU Session mobility from 3GPP to non-3GPP access, an I-SMF may need to be removed because an I-SMF had been inserted when the PDU Session was served over 3GPP access.

At PDU Session mobility from non-3GPP to 3GPP access, the AMF determines whether I-SMF needs to be inserted based on the UE location and the service area of the SMF. Additionally, if Local Offloading Management is allowed as per the subscription data and the UE is within Local Offloading Management service area, the AMF selects an I-SMF as described in clause 6.10 of TS 23.548 [74].

End of CHANGES