3GPP TSG-WG SA2 Meeting #168	            S2-2503903

Gotenburg, Sweden, April 7-11, 2025                                                    (was S2-2503325)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 6205 | rev | 1 | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Fix PDU Set based Handling description |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM |  |  |  |  |  | Date: |  |  | 2025-03-28 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | PDU Set handling is used mixedly with PDU Set based handling. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - PDU Set handling/PDU Set QoS handling are replaced with PDU Set based handling or PDU Set based QoS handling in accordance with their intention. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misunderstood the PDU Set based handling feature. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.7.2.4.1, 5.7.2.4.1a, 5.8.2.7, 5.37.5.1, 5.37.5.2, 5.37.5.3, 6.2.3, 6.2.4, 6.2.5.0, 6.2.10 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | In r1, changes on clause 5.8.2.7 and 5.37.5.2 are added(i.e. PDU Set QoS handling PDU Set based handling, PDU Set based QoS handling PDU Set based handling). |  |  |  |  |  |  |  |  |

********** 1st Change **********

5.7.2.4	Notification control

5.7.2.4.1	General

The QoS Parameter Notification control indicates to the NG-RAN that notifications of "GFBR can no longer (or can again) be guaranteed" are requested when the NG-RAN determines that the GFBR, the PDB or the PER of the QoS profile cannot be fulfilled (or can be fulfilled again) for a QoS Flow (during the lifetime of the QoS Flow) and that the QoS Flow should be kept while the NG-RAN is not fulfilling the requested QoS profile. Notification control may be used for a GBR QoS Flow if the application traffic is able to adapt to the change in the QoS (e.g. if the AF is capable to trigger rate adaptation).

The NG-RAN determination whether the GFBR, the PDB and the PER can be fulfilled or not, is done under consideration of the QoS parameters Averaging Window and MDBV (for a GBR QoS Flow using the Delay-critical resource type), which are either provided explicitly as part of the QoS profile or implicitly via the standardized 5QI QoS characteristics defined in clause 5.7.4.

The SMF shall only enable Notification control when the QoS Notification Control parameter is set in the PCC rule (received from the PCF) that is bound to the QoS Flow. The Notification control parameter is signalled to the NG-RAN as part of the QoS profile.

If the NG-RAN has received the PDU Set QoS parameters PSDB and PSER for UL and/or DL (as described in clause 5.7.7) and PDU Set based QoS handling is applied in the respective direction(s), the NG-RAN uses the PSDB and PSER instead of the PDB and PER in the respective direction(s) for the actions described in all (sub) clauses of clause 5.7.2.4. If the PDU Set based QoS handling, PSER and PSDB are applied in one direction only, the NG-RAN still uses the PDB and PER in the other direction for the actions described in all (sub) clauses of clause 5.7.2.4.

5.7.2.4.1a	Notification Control without Alternative QoS Profiles

If, for a given GBR QoS Flow, Notification control is enabled and the NG-RAN determines that the GFBR, the PDB or the PER of the QoS profile cannot be fulfilled, NG-RAN shall send a notification towards SMF that the "GFBR can no longer be guaranteed". Furthermore, the NG-RAN shall keep the QoS Flow (i.e. while the NG-RAN is not fulfilling the requested QoS profile for this QoS Flow), unless specific conditions at the NG-RAN require the release of the NG-RAN resources for this GBR QoS Flow, e.g. due to Radio link failure or RAN internal congestion. The NG-RAN should try to fulfil the GFBR, the PDB and the PER of the QoS profile again.

NOTE 1:	NG-RAN can decide that the "GFBR can no longer be guaranteed" based on, e.g. measurements like queuing delay or system load.

Upon receiving a notification from the NG-RAN that the "GFBR can no longer be guaranteed", the SMF may forward the notification to the PCF, see TS 23.503 [45].

When the NG-RAN determines that the GFBR, the PDB and the PER of the QoS profile can be fulfilled again for a QoS Flow (for which a notification that the "GFBR can no longer be guaranteed" has been sent), the NG-RAN shall send a notification, informing the SMF that the "GFBR can again be guaranteed" and the SMF may forward the notification to the PCF, see TS 23.503 [45]. The NG-RAN shall send a subsequent notification that the "GFBR can no longer be guaranteed" whenever necessary.

NOTE 2:	It is assumed that NG-RAN implementation will apply some hysteresis before determining that the "GFBR can again be guaranteed" and therefore a frequent signalling of "GFBR can again be guaranteed" followed by "GFBR can no longer be guaranteed" is not expected.

NOTE 3:	If the QoS Flow is modified, the NG-RAN restarts the check whether the "GFBR can no longer be guaranteed" according to the updated QoS profile. If the Notification control parameter is not included in the updated QoS profile, the Notification control is disabled.

During a handover, the Source NG-RAN does not inform the Target NG-RAN about whether the Source NG-RAN has sent a notification for a QoS Flow that the "GFBR can no longer be guaranteed". The Target NG-RAN performs admission control rejecting any QoS Flows for which resources cannot be permanently allocated. The accepted QoS Flows are included in the N2 Path Switch Request or N2 Handover Request Acknowledge message from the NG-RAN to the AMF. The SMF shall interpret the fact that a QoS Flow is listed as transferred QoS Flow in the Nsmf_PDUSession_UpdateSMContext Request received from the AMF as a notification that "GFBR can again be guaranteed" for this QoS Flow unless the SMF is also receiving a reference to an Alternative QoS Profile for this QoS Flow (which is described in clause 5.7.2.4.2). After the handover is successfully completed, the Target NG-RAN shall send a subsequent notification that the "GFBR can no longer be guaranteed" for such a QoS Flow whenever necessary. If the SMF has previously notified the PCF that the "GFBR can no longer be guaranteed" and the SMF does not receive an explicit notification that the "GFBR can no longer be guaranteed" for that QoS Flow from the Target NG-RAN within a configured time, the SMF shall notify the PCF that the "GFBR can again be guaranteed".

If the PDU Set based QoS handling is applied in UL and/or DL, the NG-RAN uses the PDU Set QoS parameters PSDB and PSER for UL and/or DL instead of PDB and PER in the respective direction(s) to determine whether "GFBR can no longer be guaranteed" or "GFBR can again be guaranteed".

If the NG-RAN determines that only in one direction the GFBR, the PDB or the PER cannot be fulfilled, the NG-RAN shall indicate that "GFBR can no longer be guaranteed" together with the corresponding direction.

********** 2nd Change **********

5.8.2.7	PDU Session and QoS Flow Policing

ARP is used for admission control (i.e. retention and pre-emption of the new QoS Flow). The value of ARP is not required to be provided to the UPF.

For every QoS Flow, the SMF shall determine the transport level packet marking value (e.g. the DSCP in the outer IP header) based on the 5QI, the Priority Level (if explicitly signalled) and optionally, the ARP priority level and provide the transport level packet marking value to the UPF.

For QoS Flows that are configured for PDU Set based handling, the SMF may additionally take into account the PDU Set Importance when determining the transport level packet marking values. In this case, the SMF provides a list of transport level packet marking values for the downlink direction to the UPF in a FAR, each of the transport level packet marking values corresponding to one or more PDU Set Importance values. When an I-SMF/I-UPF is inserted in the PDU Session, the I-SMF may instruct the I-UPF to derive the transport level packet marking of the outgoing N3 downlink packet based on the transport level packet marking of the incoming N9 downlink packet.

NOTE 1:	It is recommended that the PDU Set Importance based transport level packet marking values applied by the UPF only vary the drop precedence between PDUs in the transport network nodes (e.g. IP routers) on the N3/N9 interfaces. This is needed to avoid causing packet reordering in the transport network when different transport level DSCP marking values are used for a single QoS Flow.

NOTE 2:	The transport level packet marking values are provided on per-QoS Flow basis and it is up to operator deployments to enforce consistency of transport level marking in the transport network.

The SMF shall provide the Session-AMBR values of the PDU Session to the UPF so that the UPF can enforce the Session-AMBR of the PDU Session across all Non-GBR QoS Flows of the PDU Session.

SMF shall provide the GFBR and MFBR value for each GBR QoS Flow of the PDU Session to the UPF. SMF may also provide the Averaging window to the UPF, if Averaging window is not configured at the UPF or if it is different from the default value configured at the UPF.

In the case of 3GPP access, the SMF may decide to activate ECN marking for L4S by PSA UPF for the QoS Flow (see clause 5.37). In this case, the SMF shall send an ECN marking for L4S indicator to PSA UPF.

********** 3rd Change **********

5.37.5	PDU Set based Handling

5.37.5.1	General

A PDU Set is comprised of one or more PDUs carrying an application layer payload such as a video frame or video slice. The PDU Set based QoS handling by the 5G-AN is determined by PDU Set QoS Parameters in the QoS profile of the QoS Flow (specified in clause 5.7.7) and PDU Set Information provided by the PSA UPF via N3/N9 interface as described in clause 5.37.5.2. The PDU Set based Handling can be applied for GBR and non-GBR QoS Flows. The AF should provide PDU Set related assistance information for dynamic PCC control. One or more of the following PDU Set related assistance information may be provided to the NEF/PCF using the AF session with required QoS procedures in clauses 4.15.6.6 and 4.15.6.6a of TS 23.502 [3].

-	PDU Set QoS Parameters as described in clause 5.7.7

-	Protocol Description: Indicates the transport protocol used by the service data flow (e.g. RTP, SRTP) and information, e.g. the following:

-	RTP [185] or SRTP [186];

-	RTP or SRTP with RTP Header Extensions, including:

-	RTP Header Extensions for PDU Set Marking as defined in TS 26.522 [179];

-	Other RTP Header Extensions as defined RFC 8285 [189], RFC 9143 [207];

-	RTP or SRTP without RTP Header Extensions, but together with RTP Payload Format (e.g. H.264 [187] or H.265 [188]);

-	RTP or SRTP with RTP Header Extensions for PDU Set Marking as defined in TS 26.522 [179] and together with RTP Payload Format (e.g. H.264 [187] or H.265 [188]);

-	RTP or SRTP with other RTP Header Extensions following RFC 8285 [189] and together with RTP Payload Format (e.g. H.264 [187] or H.265 [188]).

NOTE 1:	With the Protocol Description options combining SRTP together with RTP Payload Format the UPF can still obtain some of the PDU Set information from the RTP Header (refer to Annex A of TS 26.522 [179]).

-	Media over QUIC (MoQ) Transport, IETF draft-ietf-moq-transport [201] (as further described in clause 5.37.9.2).

When RTP Header Extensions for PDU Set Marking (as defined in TS 26.522 [179] or other RTP header extensions as defined in RFC 8285 [189] is included, the differentiation between different RTP Header Extension Types should be supported.

When RTP Payload Format is included, the differentiation between different RTP Payload Formats should be supported.

NOTE 2:	Multiplexing of different transport protocols and different media traffic for differentiated PDU Set based handling is not supported in the current Release.

The Protocol Description can be UL only, DL only or UL and DL. The Protocol Description for UL and DL traffic may be different.

For end-to-end encrypted traffic, PDU Set Information is received as media related information from the Application Server, see clause 5.37.9.

NOTE 3:	Identification of PDU Set information for end-to-end encrypted traffic is supported in the DL direction only.

AF provided PDU Set QoS Parameters and UL and/or DL Protocol Description may be used in determining the PCC Rule by the PCF as defined in clause 6.1.3.27.4 of TS 23.503 [45] and the DL Protocol Description may be used for identifying the PDU Set Information and PDU Set Information marking by the PSA UPF.

When the SMF receives the PCC rule, the SMF performs binding of the PCC rule to one QoS Flow as described in clause 6.1.3.2.4 of TS 23.503 [45]. At least one of the following shall be included in the PCC rule to enable PDU Set based QoS handling: 1) a PSIHI and/or 2) both PSDB and PSER. Based on the PCC rule, the SMF adds the PDU Set QoS Parameters to the QoS Profile of the QoS Flow as described in clause 6.2.2.4 of TS 23.503 [45]. Alternatively, the SMF may be configured to support PDU Set based Handling without receiving PCC rules from a PCF.

If no DL PDU Set QoS parameters are included in the PCC rule, but a DL Protocol Description is included in the PCC rule received by the SMF and the PSA UPF supports PDU Set based handling (i.e. supports to perform PDU Set Information marking), the SMF may, based on the local operator policy, send to the NG-RAN, a DL PDU Set Information Marking Support Indication indicating that the DL PDU Set Information marking is supported.

For the downlink direction, the PSA UPF identifies PDUs that belong to PDU Sets and marks them accordingly as described in clause 5.37.5.2. If the PSA UPF receives a PDU that does not belong to a PDU Set based on Protocol Description for PDU Set identification, then the PSA UPF still maps it to a PDU Set and determines the PDU Set Information as described in clause 5.37.5.2.

NOTE 4:	If the PSA UPF receives a PDU that does not belong to a PDU Set, then it is assumed that the UPF determines the PDU Set Importance value based on pre-configuration.

For the uplink direction, the UE may identify PDU Sets and how this is done is left up to UE implementation. The SMF may send the UL Protocol Description associated with the QoS rule to UE.

NOTE 5:	Using the Protocol Description or not is left to UE implementation. The use of Protocol Description does not impact QoS Flow Mapping in the UE.

In this Release, the PDU Set based handling is supported in 5GS for a UE registered in 3GPP access for single access PDU Session with IP PDU Session Type, for a UE registered in untrusted or trusted non-3GPP accesses for single access PDU Session with IP PDU Session Type, and for a 5G-RG registered in W-5GAN for single access PDU Session with IP PDU Session Type. The support of PDU Set based handling in 5G-RG is specified in TS 23.316 [84].

5.37.5.2	PDU Set Information and Identification

To support PDU Set based handling, the PSA UPF identifies PDUs that belong to a PDU Set and determines the below PDU Set Information and sends it to the 5G-AN in the GTP-U header. The PDU Set Information is used by the 5G-AN for PDU Set based handling as described above.

The PDU Set Information comprises:

-	PDU Set Sequence Number.

-	Indication of End PDU of the PDU Set.

-	PDU Sequence Number within a PDU Set.

-	PDU Set Size in bytes.

-	PDU Set Importance, which identifies the relative importance of a PDU Set compared to other PDU Sets within a QoS Flow.

The 5G-AN may use the Priority Level (see clause 5.7.3.3) across QoS Flows and PDU Set Importance within a QoS Flow for PDU Set level packet discarding in presence of congestion.

NOTE 1:	In addition to considering the PDU Set Importance within a QoS Flow, 5G-AN could also consider the relative PDU Set Importance across QoS Flows of the same Priority Level when determining which PDU Set needs to be discarded, which is up to implementation and configuration of operator.

NOTE 2:	The PDU Set Information can be different for different PDU Sets within a QoS Flow.

If the 5G-AN has provided a PDU Set based handling support Indication indicating that PDU Set based handling is supported and a DL Protocol Description together with 1) a PSIHI and/or 2) PSDB and PSER is included in the PCC rule, the SMF instructs PSA UPF to perform PDU Set marking and may provide the PSA UPF the DL Protocol Description used by the service data flow. The DL Protocol Description may be received in the PCC rule, based on information provided by the AF or by PCF local policies as described in clause 5.37.5.1.

If the DL PDU Set Information Marking Support Indication has been sent to the 5G-AN (as described in clause 5.37.5.1) and the 5G-AN has provided a PDU Set based handling support Indication indicating that PDU Set based handling is supported, the SMF instructs PSA UPF to perform PDU Set marking and provides the PSA UPF the DL Protocol Description used by the service data flow.

PSA UPF can identify the PDU Set Information using the DL Protocol Description and the received transport protocol headers and payload or using implementation specific means. The details of the RTP/SRTP headers, header extensions and/or payloads used to identify PDU Set Information are defined in TS 26.522 [179].

For end-to-end encrypted traffic, PDU Set Information is received as media related information from the Application Server, see clause 5.37.9.

For each DL PDU received on N6 for which PDU Set based handling is indicated from the SMF, the PSA UPF applies the rules for PDU Set identification and provides the available PDU Set Information to the 5G-AN in the GTP-U header.

NOTE 3:	The PSA UPF is expected to assign a unique PDU Set Sequence Number in the GTP-U header to each PDU Set of the QoS Flow.

5.37.5.3	Non-homogenous support of PDU set based handling in NG-RAN

The SMF, by sending PDU Set QoS parameters (as described in clause 5.7.7.1) or a DL PDU Set Information Marking Support Indication (as described in clause 5.37.5.1) to the NG-RAN, requests the NG-RAN to activate PDU Set based handling for a given QoS Flow and the NG-RAN provides the SMF with a PDU Set Based Handling Support Indication if the PDU Set based handling is supported. Based on this indication, SMF may activate the PDU Set identification and marking in the PSA UPF as described in clause 5.37.5.2.

During mobility procedures that result in the change of NG-RAN, the target NG-RAN provides to the SMF a PDU Set Based Handling Support Indication if it supports PDU Set based handling, as specified in TS 38.413 [34]. Based on the target NG-RAN indication, the SMF may, upon completion of the mobility procedure, initiate the PDU Session modification procedure to provide PDU Set QoS parameters or the DL PDU Set Information Marking Support Indication to NG-RAN and may configure the PSA UPF to activate the PDU Set identification and marking. If the PDU Set Based Handling Support Indication is not received from the target NG-RAN and PDU Set identification and marking is active in the PSA UPF, the SMF may deactivate it.

In the case where the PSA UPF identifies and marks PDUs with PDU Set information in GTP-U header, it shall start doing so from a complete PDU Set.

NOTE:	Based on the PDU Set Based Handling Support Indication from the target NG-RAN, the SMF can update the QoS profile and, if applicable, Alternative QoS Profiles as defined in clauses 5.7.1.2 and 5.7.1.2a to include the PDU Set QoS parameters respectively.

********** 4th Change **********

6.2.3	UPF

The User plane function (UPF) includes the following functionality. Some or all of the UPF functionalities may be supported in a single instance of a UPF:

-	Anchor point for Intra-/Inter-RAT mobility (when applicable).

-	Allocation of UE IP address/prefix (if supported) in response to SMF request.

-	External PDU Session point of interconnect to Data Network.

-	Packet routing & forwarding (e.g. support of Uplink classifier to route traffic flows to an instance of a data network, support of Branching point to support multi-homed PDU Session, support of traffic forwarding within a 5G VN group (UPF local switching, via N6, via N19)).

-	Packet inspection (e.g. Application detection based on service data flow template and the optional PFDs received from the SMF in addition, IP or MAC filter-based packet detection functionality).

-	User Plane part of policy rule enforcement, e.g. Gating, Redirection, Traffic steering).

-	Lawful intercept (UP collection).

-	Traffic usage reporting.

-	QoS handling for user plane, e.g. UL/DL rate enforcement, Reflective QoS marking in DL.

-	Uplink Traffic verification (SDF to QoS Flow mapping).

-	Transport level packet marking in the uplink and downlink.

-	Downlink packet buffering and downlink data notification triggering.

-	Sending and forwarding of one or more "end marker" to the source NG-RAN node.

-	Functionality to respond to Address Resolution Protocol (ARP) requests and / or IPv6 Neighbour Solicitation requests based on local cache information for the Ethernet PDUs. The UPF responds to the ARP and / or the IPv6 Neighbour Solicitation Request by providing the MAC address corresponding to the IP address sent in the request.

-	Packet duplication in downlink direction and elimination in uplink direction in GTP-U layer.

-	NW-TT functionality.

-	High latency communication, see clause 5.31.8.

-	ATSSS Steering functionality to steer the MA PDU Session traffic, refer to clause 5.32.6.

NOTE:	Not all of the UPF functionalities are required to be supported in an instance of user plane function of a Network Slice.

-	Inter PLMN UP Security (IPUPS) functionality, specified in clause 5.8.2.14.

-	Event exposure, including exposure of network information, i.e. the QoS monitoring information, as specified in clause 5.8.2.18, events as specified in clause 5.2.26.2 of TS 23.502 [3], exposure of data collected for analytics, as specified in clause 5.2.26.2 of TS 23.502 [3] and exposure of the TSC management information as specified in clause 5.8.5.14.

-	Exposure of the UE information, e.g. UE IP address translation information as specified in clause 5.2.26.3 of TS 23.502 [3] and clause 4.15.10 of TS 23.502 [3] if Network address translation (i.e. NAT) functionality of the UE IP address is deployed within UPF.

-	Support of PDU Set based handling as defined in clause 5.37.5.

-	Support MoQ relay functionality as defined in clause 5.37.9.2.

-	NAT information exposure functionality (if NAT is deployed within UPF) as described in clause 5.8.2.17.

-	Features indicated by the means of operator configurable UPF capabilities, as described in clause 5.8.2.21.

-	Handling of Payload Headers as defined in clause 5.8.2.22.

-	Support N6 delay measurement and reporting to SMF, as defined in clause 5.8.2.23.

6.2.4	PCF

The Policy Control Function (PCF) includes the following functionality:

-	Supports unified policy framework to govern network behaviour.

-	Provides policy rules to Control Plane function(s) to enforce them.

-	Accesses subscription information relevant for policy decisions in a Unified Data Repository (UDR).

-	Support PDU Set based handling as defined in clause 5.37.5.

NOTE:	The PCF accesses the UDR located in the same PLMN as the PCF.

The details of the PCF functionality are defined in clause 6.2.1 of TS 23.503 [45].

6.2.5	NEF

6.2.5.0	NEF functionality

The Network Exposure Function (NEF) supports the following independent functionality:

-	Exposure of capabilities and events:

NF capabilities and events may be securely exposed by NEF for e.g. 3rd party, Application Functions, Edge Computing as described in clause 5.13.

NEF stores/retrieves information as structured data using a standardized interface (Nudr) to the Unified Data Repository (UDR).

-	Secure provision of information from external application to 3GPP network:

It provides a means for the Application Functions to securely provide information to 3GPP network, e.g. Expected UE Behaviour, 5G-VN group information, CAG information, time synchronization service information, PDU Set based handling service specific information and Non-3GPP Device Identifier Information. In that case the NEF may authenticate and authorize and assist in throttling the Application Functions.

-	Translation of internal-external information:

It translates between information exchanged with the AF and information exchanged with the internal network function. For example, it translates between an AF-Service-Identifier and internal 5G Core information such as DNN, S-NSSAI, as described in clause 5.6.7.

In particular, NEF handles masking of network and user sensitive information to external AF's according to the network policy.

-	Redirecting the AF to a more suitable NEF/L-NEF e.g. when serving an AF request for local information exposure and detecting there is a more appropriate NEF instance to serve the AF's request.

-	The Network Exposure Function receives information from other network functions (based on exposed capabilities of other network functions). NEF stores the received information as structured data using a standardized interface to a Unified Data Repository (UDR). The stored information can be accessed and "re-exposed" by the NEF to other network functions and Application Functions and used for other purposes such as analytics.

-	A NEF may also support a PFD Function: The PFD Function in the NEF may store and retrieve PFD(s) in the UDR and shall provide PFD(s) to the SMF on the request of SMF (pull mode) or on the request of PFD management from NEF (push mode), as described in TS 23.503 [45].

-	A NEF may also support a 5G-VN Group Management Function: The 5G-VN Group Management Function in the NEF may store the 5G-VN group information in the UDR via UDM as described in TS 23.502 [3].

-	Support management of ECS Address Information.

-	Support management of relationship between DNAI and EAS Address Information.

-	Exposure of analytics:

NWDAF analytics may be securely exposed by NEF for external party, as specified in TS 23.288 [86].

-	Retrieval of data from external party by NWDAF:

Data provided by the external party may be collected by NWDAF via NEF for analytics generation purpose. NEF handles and forwards requests and notifications between NWDAF and AF, as specified in TS 23.288 [86].

-	Support of Non-IP Data Delivery:

NEF provides a means for management of NIDD configuration and delivery of MO/MT unstructured data by exposing the NIDD APIs as described in TS 23.502 [3] on the N33/Nnef reference point. See clause 5.31.5.

-	Charging data collection and support of charging interfaces.

-	Support of Member UE selection assistance functionality:

-	NEF may provide one or more list(s) of candidate UE(s) (among the list of target member UE(s) provided by the AF) and additional information to the AF based on the parameters contained in the request from the AF as described in clause 5.46.2. NEF supports the translation of the member UE selection filtering criteria parameters received from the AF to the corresponding event or analytics filters that can be understood by the 5GC NFs for events or analytics related data collection. NEF interacts with 5GC NFs using existing services in order to collect the corresponding data and then derive the list(s) of candidate UE(s) and other assistance information as described in clause 4.15.13 of TS 23.502 [3].

-	Support of Multi-member AF session with required QoS for a set of UEs identified by a list of UE addresses:

-	Details are specified in clause 4.15.6.13 of TS 23.502 [3].

-	Support of UAS NF functionality:

Details are defined in TS 23.256 [136].

-	Support of EAS deployment functionality:

Details are defined in TS 23.548 [130].

-	Support of SBI-based MO SM transmit for MSISDN-less MO SMS:

Details are defined in TS 23.540 [142].

-	Support PDU Set based handling as defined in clause 5.37.5.

-	Support management of common EAS and common DNAI:

Details are defined in TS 23.548 [130].

-	Support of Vertical Federated Learning (VFL):

Details are defined in TS 23.288 [86].

-	Support request and exposure of energy related information as defined in clause 5.51.

A specific NEF instance may support one or more of the functionalities described above and consequently an individual NEF may support a subset of the APIs specified for capability exposure.

NOTE:	The NEF can access the UDR located in the same PLMN as the NEF.

The services provided by the NEF are specified in clause 7.2.8.

For external exposure of services related to specific UE(s), the NEF resides in the HPLMN. Depending on operator agreements, the NEF in the HPLMN may have interface(s) with NF(s) in the VPLMN.

When a UE is capable of switching between EPC and 5GC, an SCEF+NEF is used for service exposure. See clause 5.17.5 for a description of the SCEF+NEF.

********** 5th Change **********

6.2.10	AF

The Application Function (AF) interacts with the 3GPP Core Network in order to provide services, for example to support the following:

-	Application Function influence on traffic routing (see clause 5.6.7);

-	Application Function influence on Service Function Chaining (see clause 5.6.16.2);

-	Accessing Network Exposure Function (see clause 5.20);

-	Interacting with the Policy and charging control framework (see clause 5.14);

-	Time synchronization service (see clause 5.27.1.8);

-	IMS interactions with 5GC (see clause 5.16).

-	Support PDU Set based handling as defined in clause 5.37.5.

-	Support Vertical Federated Learning (VFL) as VFL Server or VFL Client (see clause 5.4 of TS 23.288 [50]).

Based on operator deployment, Application Functions considered to be trusted by the operator can be allowed to interact directly with relevant Network Functions.

Application Functions not allowed by the operator to access directly the Network Functions shall use the external exposure framework (see clause 7.3) via the NEF to interact with relevant Network Functions.

The functionality and purpose of Application Functions are only defined in this specification with respect to their interaction with the 3GPP Core Network.

********** End of Changes **********