3GPP TSG-SA2 Meeting #168	S2-2503949

07 - 11 April 2025, Goteborg, SE                                       (revision of S2-2503744)

Source:	CATT, vivo

Title:	FS_5GSAT_Ph4_ARC Key issues for WT#1

Document for:	Approval

Agenda Item:	20.1.1

Work Item / Release:	FS_5GSAT_Ph4_ARC / Rel-20

Abstract of the contribution: This paper is to propose new key issue for FS_5GSAT_Ph4_ARC.

1	Discussion

1.1	Introduction

The Work Task#1 of the FS_5GSAT_Ph4_ARC SID reads as follows:

-	WT-1: IMS voice call over GEO via NB-IoT NTN connecting to EPC

NOTE 1:	Both CP and UP based NB-IoT NTN solutions will be studied in the WT-1, checkpoint will be set before the normative work to align with RAN scope.

NOTE 2:	Only architecture options with data via SGi will be considered in the study. The option with data via SCEF/T8 will not be considered.

-	WT-1.1 Whether and how to support QoS for IMS voice service over GEO satellite,

-	WT-1.2 whether and how to define IMS enhancement to utilize NB-IoT as IP-CAN and potential optimization to comply with the relevant SA1 requirements.

-	WT-1.3 Support of IMS emergency call via NB-IoT NTN.

-	WT-1.4 How to obtain UE Geo-location for IMS emergency call and other regulatory services.

NOTE 3:	Solutions should take into account that CP data (DoNAS) in current pre Rel-20 NB-IoT is regarded as NAS signalling and will have higher QoS than UP data (S1-U).

NOTE 4:	There will be NO mobility enhancement developed for the NB-IoT NTN in this WT.

NOTE 5:	Depending on operators’ policies, IMS enhancements defined in the WT-1 may also be used in 5GS with NR NTN, but no additional study will be conducted in this study item.

Based on the WT, we have to study how to support IMS voice call over GEO via NB-IoT NTN connecting to EPC.

1.2	Rationale for the key issues formulation

The following key issues are proposed to be added to the technical report.

-	Key Issue #X: Support of IMS voice service over GEO satellite via NB-IoT NTN
Currently, IMS voice service can not be supported via NB-IoT NTN. Moreover, due to long transmission latency and limited data rate provided by NB-IoT NTN over GEO satellite, current QoS requirements for IMS voice service defined in TS 23.401 can not be satisfied.

-	Key Issue #Y: IMS enhancement for GEO NB- IoT NTN access
As described in TS 22.261 [a], for a IMS voice call over GEO satellite, the Experienced data rate for UL/DL is [1-3] kbit/s, and the Call set up time is no more than 30s. Considering the transmission data rate, latency and packet size supported by NB-IoT NTN over GEO satellite, it is proposed to study how the requirements can be met.

-	Key Issue #Z: Support of IMS emergency call via NB-IoT NTN
NB-IoT NTN currently doesn’t support emergency call, however this is required in some jurisdictions to be able to offer the service. Moreover, it should be noticed that the SA1 requirements defined for IMS voice call using GEO satellite access shown in the preceding paragraph are also applicable for emergency calls.

-	Key Issue #W: Location service for IMS emergency call and regulatory services over NB-IoT NTN
Provision of emergency calls and other regulatory services requires the network to obtain the UE geo-location (e.g. to correctly route the call to the most appropriate PSAP). This capability must be extended to IMS emergency call over GEO via NB-IoT NTN connecting to EPC

2 Proposal

It is proposed to agree to add the following key issues for WT#1 into TR 23.700-19.

* First change *

5.X	Key Issue #X: Support of IMS voice call over GEO via NB-IoT NTN connecting to EPC

To support IMS voice call over GEO via NB-IoT NTN connecting to EPC, the following issues are proposed to be studied:

-	5G System enhancements to support IMS voice service over GEO satellite via NB-IoT NTN, including:

-	Whether and how to support QoS for IMS voice service over GEO satellite.

5.Y	Key Issue #Y: IMS enhancement for GEO NB- IoT NTN access

To meet the requirements on experienced data rate and call setup time documented in TS 22.261 [a], this key issue will examine:

-	Whether and how to define IMS enhancement to utilize NB-IoT as IP-CAN;

-	Potential IMS optimization.

5.Z	Key Issue #Z: Support of IMS emergency call via NB-IoT NTN

This key issue will study:

-	How to support IMS emergency call over GEO via NB-IoT NTN connecting to EPC to comply with the relevant requirements defined in TS 22.261 [a].

## 5.W	Key Issue #W: Location service for IMS emergency call and regulatory services over NB-IoT NTN

This key issue aims to study whether existing location services can be used to obtain the UE geo-location when the UE establishes an IMS emergency call over GEO via NB-IoT NTN connecting to EPC or other services for which regulatory requirements for obtaining network provided location information exist.

* End of changes *