SA WG2 Meeting #168	S2-2503957

7 - 11 April 2025, Goteborg, Sweden

Source:	ZTE

Title:	UP building for UE training data collection

Document for:	Approval

Agenda Item:	20.3.1

Work Item / Release:	FS_AIML_CN_Ph2 / Rel-20

Abstract: This contribution proposes a way for UP building between an NF and UE.

# 1. Discussion

3GPP SA2 Release 20 has approved the study item "AI/ML Core Network Enhancements Phase 2", which includes the research task of "whether and how to support standardized user plane transmission for AI/ML-related UE data collection." To enable the establishment of user plane connections from the UE to an NF, this paper proposes a way for UP building.

# 2. Proposal

This paper proposes to add a solution for Key Issue #1 to TR 23.700-04 as follows.

---------- Start of change ----------

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |
| --- | --- | --- |
| Solutions | <Key Issue 1> | <Key Issue #y> |
| #1 | X |  |
| #2 |  |  |

---------- Next change (all new text) ----------

6.x	Solution #X: UP Building for UE data collection

6.x.1	Functional Description

This solution proposes a user plane connection between an NF and UE, with some enhancements in the UP building request. The NF received the request from AF and trigger UP building. After UE training data collection finished, the NF checks the training data and sends them to NEF/AF. This solution is based on the user plane positioning procedure as described in clause 6.18.1 of TS 23.273[X].

6.x.2	Procedures

UEr

The UP building procedure is shown below:

NF in core

AF

NEF

UPF

SMF

AMF

1. UP building request

2. URSP matching

3. PDU session establishment

4.N4 session establishment

5. PDU session establishment accept

6. UP binding request

7. UP binding response

8. PDU session establishment complete

9. UE training data transfer

10. UE training data check and confirm

11. UE training data transfer

12. UE training data transfer

Figure 6.x.2-1: User plane building between an NF and UE

- NOTE 1: The user consent check is not addressed of this solution.

- Editor’s note: Whether and how to separate UE data collection traffic over user plane from end user generated user plane traffic is FFS.

- Editor’s note: In this solution, which steps are applicable for UE data collection and which steps are applicable UE data transfer is FFS.

- Editor’s note: The service operations will be updated.

- Editor’s note:  How to select UE for data collection is FFS.

- 0. An NF has received UE training data collection request from AF.

- Editor’s note: The trigger process between AF and an NF for UE data collection is FFS.

- 1. The NF send UP building request to UE via AMF and gNB for AI/ML data collection, the message may contain an NF address information, UP binding ID (i.e. to associate the user plane connection to be established with the target UE), purpose of building user plane (e.g. AI/ML user plane data collection), training data type (e.g. CSI, SSB, RSRP), sample number, sample quality threshold (i.e. only samples that quality greater than the threshold can be sent to an NF).

Editor's note:	It is FFS whether training data type and sample quality threshold are included in the message.

Editor's note:	It is FFS which entity in CN will perform UE data collection.

- 2. The UE performs URSP rule matching based on the received parameters. For example, based on the address information of the NF, the UE matches the corresponding DNN and S-NSSAI in the RSD. Then, it requests the establishment of a PDU session on the matched DNN and S-NSSAI.

Editor's note:	It is FFS whether to use AI/ML user plane data collection indication to perform URSP matching.

- 3. The UE sends a PDU session establishment request to the SMF, and the request message includes the DNN and S-NSSAI.

- 4. The SMF selects the appropriate UPF based on the S-NSSAI and DNN and initiates the N4 session establishment.

- 5. The SMF replies with a PDU session establishment acceptance message to the UE, and the PDU connection service between the UE and the NF is established.

- 6. The UE initiates the user plane connection binding process. It sends a user plane connection binding request to the NF, and the request message includes the user plane binding ID received in step 1.

- 7. The an NF replies with a user plane connection binding acceptance message to the UE.

Editor's note:	It is FFS whether and how to binding UP.

- 8. The UE sends a user plane connection establishment complete message to the AMF. The AMF then forwards the user plane connection establishment complete message to the SMF.

- 9. The UE sends UE training data for AI/ML through the user plane to the NF.

- 10.  The NF check whether the collected UE training data align with the requirements from AF.

Editor's note:	It is FFS whether and how the NF perform checking is FFS.

- 11. The NF sends the AI/ML-related data collected from the UE to the NEF. This message may include one or more data types requested by the AF along with the corresponding data.

- 12. The NEF maps the information received from the NF into external information (e.g. mapping SUPI to GPSI) and sends the training data to the AF.

Editor's note: How to support visibility of standardized data is FFS.

6.X.3	Impacts on services, entities and interface

The main enhancement for this solution is one NF in core build a user plane with UE. Also, the NF may provide a purpose of building user plane (e.g. AI/ML user plane data collection) and data types/sample information to UE in the UP building request message. Lastly, the NF needs to check and confirm the collected training data are aligned with the requirements before it sends to NEF/AF.

---------- End of changes ----------