3GPP TSG SA WG 2 Meeting SA2#168	                                                                  TDoc S2-2504026

Gothenburg, Sweden, 2025-04-07 – 2025-04-11

Title:	DRAFT - LS reply on support for multiple non-3GPP device identifiers

Response to:	LS C1-250822 on LS on support for multiple non-3GPP device identifiers from CT1

Release:	Rel-19

Work Item:	UIA_ARC

Source:	 SA2

To:	CT1

Cc:	CT3

Contact person:	Magnus Hallenstål

magnus.l.hallenstal@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	CR on TS 23.502, S2-250xxxx, CR#nnnn, and S2-250yyyy, CR#nnnn

CR on TS 23.503, S2-250zzzz, CR#nnnn

# 1	Overall description

In LS on support for multiple non-3GPP device identifiers from CT1 the following questions were asked:

Q1: Whether the multiple non-3GPP device identifiers are allowed in a single PDU SESSION MODIFICATION REQUEST message?

Q2: For the Non-3GPP device identifier not available scenario, whether the SMF rejects the Non-3GPP device identifier or rejects the whole PDU SESSION MODIFICATION REQUEST message?

Q3: For the scenario of removal of more than one non-3GPP device identifier from the subscription in the UDR, whether a single or multiple non-3GPP device identifiers should be permitted in a single PDU SESSION MODIFICATION COMMAND message?

Answers:

Q1: SA2 recognise the possibility that allowing only a single non-3GPP Device Identity in a PDU Session Modification may increase the signalling overhead. To rectify this issue, SA2 has agreed CR S2-250xxxx and S2-250zzzz, which allows the PDU Session Modification to have multiple non-3GPP Device Identifiers.

Q2: After discussions, SA2 has come the following conclusion: If any Device ID in the PDU session modification request is not available in the UDR, the SMF will reject the PDU session without any indication about which Device ID(s) that caused the rejection. It is up to UE how to resolve the issue. E.g. the UE could interact with an AF to get a correct list of defined Device IDs or it could start doing session modification for single Device ID. Please see CR S2-250xxxx and S2-250zzzz.

Q3: SA2 has discussed this and come to the following conclusion. Removal of one device ID in UDR will result in removing the corresponding QoS rules. Removing multiple device IDs will consequently mean removing all related QoS rules related to the removed device IDs.. See CR S2-250yyyy and S2-250zzzz.

# 2	Actions

To CT1

ACTION: Based on the answers above, SA2 asks CT1 to take any needed action

# 3	Dates of next TSG SA WG 2 meetings

SA2#169		2025-05-19 – 2025-05-23 Fukuoka City, Japan

SA2#170		2025-08-25 – 2025-08-29 Gothenburg, Sweden