3GPP TSG-WG SA2 Meeting #168	S2-2504060

Goteborg, SE, 7th April – 11th April, 2025	(revision of S2-2503845)

Source:	Huawei, HiSilicon

Title:	pCR for TS 23.369: Update of Device ID for NID

Document for:	Approval

Agenda Item:	19.14.2

Work Item / Release:	AmbientIoT-ARC / Rel-19

Abstract: The AIoT Device ID can contain an optional NID, but there is no indication it is present.

# 1. Introduction/Discussion

The AIoT Device Permanent Identifier can contain Domain Information. In the description of the Domain Information it can contain zero, one or more of a network identifier (i.e., MNC and MMC), a NID and a third party ID (i.e. it can contain 0, 1, 2, or 3 fields) and the ID Type field then indicates whether a network identifier or a third party ID is present in the Domain Information.

The existing text describing a network identifier says it can contain an MCC and MNC, and/or a NID.

TS 23.003 does not have such a combined structure defined, it has separate definitions for a PLMN Identifier (clause 12.1) and a NID (clauses 12.7.1 and 12.7.2).

The PLMN Identifier is “MCC || MNC” and the NID has 2 slightly different formats (both of which are self-describing and both are 11 hexadecimal digits long).

TS 23.501 Clause 5.40.2.1 (for example) describes that a the combination of PLMN ID and Network identifier (NID) are used.

When an AIOTF, for example, receives a AIoT Device Permanent Identifier it may need to parse it and determine which (fixed length) fields it contains to, for example, extract the Identification Information, and from this it can then construct, for example if needed, a realm using the MNC, MCC and if present NAI, as described in 23.003 clause 28.7.3:

-	"5gc.mnc<MNC>.mcc<MCC>.3gppnetwork.org"

-	"5gc.nid<NID>.mnc<MNC>.mcc<MCC>.3gppnetwork.org"

The device does not need to provide all the fixed parts (e.g. "5gc.mnc", ".mcc", ".3gppnetwork.org"), only the parts which are variable (i.e. NID, MNC, MCC).

With this in mind, the AIoT Device Permanent Identifier can therefore have fixed length fields for PLMN Identifier and NID, but in order to make it parseable, the AIoT Device Permanent Identifier needs to describe which of the fields are present.

Therefore, the AIoT Device Permanent Identifier ID Type field is updated to include whether a NID is present indication.

The flip side is that if the NID presence is not indicated, then the Network Identifier is variable length which will cause problems.

The terminology is also updated (e.g., AIoT Device Permanent Identifier is used etc) and aligned with TS 23.003 (e.g., PLMN Identifier), so it is clear what fields are being reference, along with other editorial clean-ups.

# 2. Text Proposal

It is proposed to capture the following changes vs. TS 23.369.

# * * * * First change * * * *

### 5.7.2	AIoT Device Permanent Identifier

In order to support the AIoT feature in 5G System, a globally unique AIoT Device Permanent Identifier shall be allocated to each AIoT device. An AIoT Device Permanent Identifier is assigned either by an operator or by a third party. The AIoT Device Permanent Identifier is used to identify an AIoT Device and locate the entity where the AIoT Device related information is stored.

NOTE 1:	How to configure an AIoT Device with the AIoT Device Permanent Identifier is out of scope of this specification.

The AIoT Device Permanent Identifier includes the following components:

-	The ID Type, including:

-	Information indicating whether a PLMN ID is included.

-	Information indicating whether a NID is included.

-	Information indicating whether a third party identifier is included.

-	Identification Information Type, indicating whether the Identification Information contains  an EPC or unstructured information.

-	The Domain Information includes none, one or more of the following:

-	A PLMN Identifier (i.e.,MCC and MNC) as specified in TS 23.003 [6] when the information in the ID type indicates it is included.

-	A Network Identifier (NID) as specified in TS 23.003 [6] when the information in the ID type indicates it is included.

-	A third party identifier used to identify a third party when the information in the ID type indicates it is included.

-	The Identification Information is used to distinguish different AIoT Devices within the scope identified by Domain Information (if available) and can contain either:

-	An EPC, as defined in clause 14 of [7];

-	Unstructured information, where the contents is defined by the allocator.

Figure 5.7.2-1: AIoT Device Permanent Identifier Structure

An operator allocated AIoT Device Permanent Identifier should include the identifier of the network for the operator. The identifier of the network is present as either a PLMN Identifier, NID or both in the AIoT Device Permanent Identifier.

A third party allocated AIoT Device Permanent Identifier may include none of the following information or include any combination of at least one kind of the following information: a PLMN Identifier, NID or third party identifier..

NOTE 3:	The length of ID Type, PLMN Identifier (if present), NID (if present) and the third party identifier (if present) components is fixed. The length of the Identification Information is variable. The details are specified in TS 23.003 [6] and TS 29.xxx [xx].

Editor’s note:	The reference in NOTE 3 needs to be updated, when the appropriate stage 3 document is identified.

The following lengths are supported for the Identification Information in an AIoT Device Permanent Identifier: 96 bits, and 128 bits.

NOTE X: The encoding for the length of the Identification Information enables additional shorter or longer fixed lengths to be supported in the future.

# * * * * End of changes * * * *