3GPP TSG-SA WG2 Meeting #168	S2-2504065 (was 3380)

Goteborg, Sweden, April 07 – 11, 2025

Source:	Samsung, Futurewei

Title:	Command Type AIoT data

Document for:	Approval

Agenda Item:	19.14.2

Work Item / Release:	AmbientIoT-ARC / Rel-19

Abstract of the contribution: The contribution proposes normative text for Command Type AIoT data.

1. Discussion

Command Procedure needs to be updated in TS 23.369.

For the following EN, AIoT data need to be structured per Command Type.

Editor's note:	It is FFS whether and how to structure the AIoT data if the Command Type is Read, Write or Disable.

For Command Type Read, the following data fields are needed.

-	Start Address: Address to start reading

-	End Address: Address to stop reading

For Command Type Write, the following data fields are needed.

-	Start Address: Address to start writing

-	Writing Entity: Contents to write

For Command Type Disable, the following data fields are needed.

-	Token Value: Token value for the Device to check if the Disable Command is valid

-	Disable Type: Permanently (In this release, only Permanently Disable is supported.)

2. Proposal

It is proposed to agree the following changes to 3GPP TS 23.369.

* * * Start of Change * * * *

6.2.3	Procedure for Command

Editor's note:	Additional information in the steps, parameters, and their naming throughout the procedures requires alignment with other clauses and references adding as required.

Editor's note:	Alignment is required for how to document and describe Direct Path and Indirect Path options in the procedures.

Figure 6.2.3-1 depicts the command procedure.

Figure 6.2.3-1: Command Procedure

1.	The AF sends the Nnef_AIoT_Command Request message (AF ID, Command Type, [Target area information for AIoT service operation], [Target AIoT device information], [approximate number of AIoT devices], [approximate D2R message size], (AIoT data)) to NEF.

Table 6.2.3-1: AIoT data structure

| Command Type | Field | Description |
| --- | --- | --- |
| Read | Start Address | Address to start reading |
|  | End Address | Address to stop reading |
| Write | Start Address | Address to start writing |
|  | Content | Contents to write |
| Disable | Address dedicated for Disable | The address to activate "disable" for that specific device. The address dedicated to mean "disable" may be different per device. The Token means |
|  | Disable Type | Permanently (In this release, only Permanently Disable is supported. Permanently, Temporariliy, Enabled) |

NOTE X:	The name and detailed meaning of each filed may be changed depending on of AIoT application, which is out of scope of this specification. The table potentially can be expanded in future releases.

The Target AIoT device information may include one or more AIoT Device ID(s) or the filtering information which is used to associate with multiple AIoT devices.

Editor's note:	It is FFS how to define the filtering information.

2.	The NEF selects AIOTF(s) taking into account the target area information for AIoT service operation and Target AIoT device information in the step 1. If no AIOTF can be selected, the NEF rejects the AIoT Command request with an appropriate cause code.

3.	The NEF sends Naiotf_AIoT_Command Request message (AF ID, Command Type, [Target area information for AIoT service operation], [Target AIoT device information], [approximate number of AIoT devices], [approximate D2R message size], AIoT data) message to the selected AIOTF.

4.	The AIOTF receives the AIoT service operation request and checks the parameters included in the request. If the AIoT service operation request cannot be processed, the AIOTF rejects the AIoT Command request with an appropriate cause code. The AIOTF performs AIoT RAN and optionally AIoT RAN Reader selection as specified in clause 5.3. If no AIoT RAN or AIoT RAN Reader can be selected, the AIOTF rejects the AIoT Command request with an appropriate cause code.

Editor's note:	The authorisation split between the NEF and AIOTF needs alignment with the other clauses in this TS.

5.	AIOTF sends the Naiotf_AIoT_Command Response message (accept or reject result for the AIoT Command service operation request) to the NEF.

6.	NEF sends the Nnef_AIoT_Command Response message (accept or reject result for the AIoT Read/Write/PermanentDisable service operation request) to the AF.

Step 7 to step11 of procedure for Inventory specified in clause 6.2.2 are performed.

Editor's note:	It is FFS whether the AIoT RAN performs aggregation for Inventory Report during the Command procedure.

Editor's note:	It is FFS how to handle the scenarios that the Inventory Response received for Command Request sent for many individual devices are in parallel.

Editor's note:	Further clarification and enhancement on step 7 to step 11 of procedure for Inventory specified in clause 6.2.2 is FFS.

For Direct path as shown in clause 4.2.2.2, step 12a is performed.

12a.	AIOTF sends Command Request message (correlation identifier, NAS Command Request) to the AIoT RAN. The correlation identifier is corresponding to this AF service operation request, and is used for the AIOTF to correlate the service operation responses to the request.

For Indirect path via AMF as shown in clause 4.2.2.3, step 12b and 12c are performed.

12b.	The AIOTF sends Namf_AIoT_MessageDelivery (AIoT RAN ID, Command Request message (correlation identifier, NAS Command Request)) to AMF. The correlation identifier is corresponding to this AF service operation request, and is used for the AIOTF to correlate the service operation responses to the request..

Editor's note:	Additional information included in the NAS Command Request for security will be determined and aligned with SA WG3.

12c.	The AMF sends the Command Request message (correlation identifier, NAS Command Request)) to AIoT RAN.

13.	The AIoT RAN sends the AS R2D message (NAS Command Request) to the AIoT device.

14.	The AIoT device sends the AS D2R message (NAS Command Response) to the AIoT RAN.

Editor's note:	The AS R2D message and AS D2R message will be aligned with RAN WG's specification.

Editor's note:	Additional information included in the NAS Command Response for security will be determined and aligned with SA WG3.

For Direct path as shown in clause 4.2.2.2, step 15a is performed.

15a.	The AIoT RAN responds an Command Response message (correlation identifier, Reader ID, NAS Command Response) to the AIOTF.

For Indirect path via AMF as shown in clause 4.2.2.3, step 15b and step 15c are performed

15b.	The AIoT RAN responds an Command Response message (correlation identifier, Reader ID, NAS Command Response) to the AMF.

15c.	The AMF responds an Naiotf_AIoT_Command Notify message (AMF ID, correlation identifier, Command Response (correlation identifier, Reader ID, NAS Command Response)) to the AIOTF.

16.	The AIOTF reports the result of the AIoT Command request to the NEF by sending the Nnef_AIoT_Command Notify message (AIoT Device ID(s), AF ID, AIoT data).

17.	The NEF informs the AF of the result of the AIoT_Command request by sending the Nnef_AIoT_Command Notify message (AIoT Device ID(s), AF ID, AIoT data).

Editor's note:	It is FFS whether a subscribed service is required before triggering Nnef_AIoT_Command Notify.

* * * End of Change * * * *