3GPP TSG-SA WG2#168	S2-2504076

Gothenburg, Sweden, 7-11 April 2025	(was S2-2503713)

Source:	Qualcomm Incorporated

Title:	New Solution to WT#2: Enabling dynamic QoS handling for energy efficiency lterby use of Energy Saving QoS Profiles

Document for:	Discussion/Approval

Agenda Item:	20.4.1

Work Item / Release:	FS_EnergySys_Ph2 / Rel-20

Abstract of the contribution: This contribution proposes to enable dynamic QoS handling based on whether energy efficiency mechanisms are being used, by defining an Energy Efficiency QoS profile.

# 1.	Discussion

This contribution proposes to enable dynamic QoS handling by define Energy Saving QoS profiles.

# 2.	Text proposal

It is proposed to agree the following changes vs. TS 23.700-67:

# >>>>BEGINNING OF CHANGES (all new text) <<<<

## Y.X	New Solution to WT#2: Enabling dynamic QoS handling for energy efficiency by use of Energy Saving QoS Profiles

## Y.X.1	Description

This solution addresses WT#2.

This solution proposes that PCF generates the Energy Saving QoS profile for the service flow. AF may provide the Energy Saving Service Requirements in addition to the QoS Reference, QoS parameters or Alternative Service Requirements for a service flow. The Energy Saving Service Requirements indicates the service requirement(s) for the AF session if energy saving is applied by the 5GS. PCF authorizes the service requirement(s) from the AF and generates the corresponding PCC rules which include the list of Energy Saving QoS profile(s) and the priorities. The AF uses the procedureas described in clause 6.1.3.22 in TS 23.503 to provide the Energy Saving Service Requirements to PCF. If AF does not provide the Energy Saving Service Requirements, PCF may generate the Energy Saving QoS profile and the priority based on operator policy. PCF does not need to collect any energy related information from other 5GC entities to generate the Energy Saving QoS profile.

The Energy Saving QoS profile represents a combination of QoS parameters (e.g., 5QI, ARP, PDB etc.), in the same format as the existing QoS profile as described in clause 5.7.2 in TS 23.501 but with different values for each parameter. The Energy Saving QoS profile(s) can be applied to both GBR and non-GBR QoS flows, and is only used for the specific QoS flow when the NG-RAN actives the energy saving.

The PCF shall enable QoS Notification Control and include the derived Energy Saving QoS profile with other QoS profile in the PCC rule sent to the SMF. SMF shall provide the Energy Saving QoS profile together with other QoS profile to the NG-RAN.

NG-RAN may decide to  apply energy saving for the QoS flow , for example, the notification from OAM that the energy consumption of the NG-RAN node exceeds a pre-configured threshold, or NG-RAN’s decision. If the Energy Saving QoS profile is received from SMF for the QoS flow, the NG-RAN selects the Energy Saving QoS profile to apply and notify the SMF that the Energy Saving QoS profile is applied. SMF may indicate the used Energy Saving QoS profile to PCF and also forwards to UPF for charging purpose.

## Y.X.2	Procedures

The solution is proposed with the following principles:

AF provides the Energy Saving Service Requirements to PCF by reusing the setting up an AF session with required QoS procedure as described in clause 4.15.6.6 in TS 23.502, with the following addition:

- AF may provide Energy Saving Service Requirements to PCF.

- PCF generates a list of Energy Saving QoS profile(s) and the priority list for the QoS flow based on AF’s request and operator policy. the Energy Saving QoS profile can be applied to both GBR and non-GBR QoS flows. The Energy Saving QoS profile represents a combination of QoS parameters (e.g., 5QI, ARP, PDB etc.), in the same format as the existing QoS profile as described in clause 5.7.2 in TS 23.501 but with different values for each parameter.

PCF provides the Energy Saving QoS profile to SMF and NG-RAN by reusing the PDU session establishment as described in clause 4.3.2 in TS 23.502 and PDU session modification in clause 4.3.3 in TS 23.502 with the following addition:

- PCF provides the Energy Saving QoS profile(s) and the priority list along with other QoS profile to SMF.

- SMF provides the Energy Saving QoS profile(s) and the priority list along with other QoS profile to NG-RAN.

NG-RAN determines to active the energy saving due to implementation, e.g. the notification from OAM that the energy consumption of the NG-RAN node exceeds a pre-configured threshold, or NG-RAN’s decision. NG-RAN sends the notification control with the selected Energy Saving QoS profile to SMF as described in clause 4.3.3.2 and clause 4.3.3.3 in TS 23.502 with the addition:

- NG-RAN sends the notification with the selected Energy Saving QoS profile

- SMF sends the selected Energy Saving QoS profile to PCF.

- SMF sends the selected Energy Saving QoS profile to UPF for charging purpose.

## Y.X.3	Impacts

PCF

-	Generates Energy Saving QoS profile which used when the energy saving is activated in NG-RAN

SMF

- Indicate Energy Saving QoS Profile to NG-RAN

- Receive notification of selected Energy Saving QoS Profile from NG-RAN, and propagating that information to PCF and UPF.

NG-RAN

-	Use of Energy Saving QoS Profile for energy efficiency if Energy saving is applied to the QoS flow.

-	Notification of use of Energy Saving QoS Profile to SMF.

UPF

- Indication of Energy Saving QoS Profile usage in CDR.

# >>>>END CHANGE<<<<