3GPP TSG-WG SA2 Meeting #168	S2- 2504078

# Goteborg, Sweden, 07 – 11April 2025                      (revision of S2-2503392)

Source: 	vivo

Title: 	KI#2, new sol: Service adjustments for network energy saving based on AF request and energy information

Document for: 	Approval

Agenda Item: 	20.4.1

Work Item / Release:	FS_EnergySys_Ph2 / Rel-20

Abstract of the contribution: Propose the new solution for KI#2 for TR 23.700-67.

1. Introduction

This contribution proposes a new solution for WT#2 for the r20 FS_EnergySys_Ph2 study.

2. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-67.

FIRST CHANGE

6.0	Mapping of Solutions to Key Issues

Editor's note:	The number of columns of Key Issues will be updated according to clause 5.

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| Solution# | 1 | 2 | 3 |
| x |  | x |  |

SECOND CHANGE

6.x	Solution # x: Service adjustments for network energy saving based on AF request and energy related information

6.x.1	Description

This solution applies to Key Issue #2.

In the release 19 features, the AF could subscribe the energy consumption information exposure from the network at specific granularity, e.g, UE level, service data flow level, PDU session level. The AF may also provide the reporting information, e.g., reporting period, reporting frequency, or reporting threshold.

Besides the energy information exposure, certain application providers may have the following considerations:

- Certain application providers may be highly concerned with the energy consumption for its applications, and would like to aware the energy consumption information of its application. If the energy consumption per UE per application is too high, it could accept certain network energy saving behaviours to minimize the energy consumption for its application.

- Some other application providers, e.g., some factories, may have its mobile terminals accessing the local non-pubic 5G network. These application providers are considered with the network energy consumption of its mobile terminals. If the energy consumption per UE is too high, it could accept certain network energy saving behaviours to minimize the energy consumption.

The network energy saving behaviours are provided by the network based on the operators’ decisions and may be configured in the PCFs of the PLMN. The network energy saving behaviours may be AM policy update, SM policy update, UE policy update, etc. The network may perform network energy saving behaviours for the UE due to certain energy related information.

In this solution, it is proposed that the network could consider the AF’s preference for the network energy saving behaviours for the UE. So the AF may indicate to the network the expected network energy saving behaviours for its applications or its UEs. The AF may also provide the network energy consumption threshold at certain granularity (e.g., per UE per application, per UE) to inform the network when to perform the network energy saving behaviours. Then, the network energy saving behaviours, e.g., QoS adjustment, are negotiated between the AF and operators. In this case, the network energy saving behaviours are considered as preferred or acceptable from the application point of view, if the energy consumption information at certain granularity is above the energy consumption threshold.

6.x.2	Procedures

Figure 6.x.3-1: Network energy saving behaviours negotiation between the AF and network

- The AF may provide the network with the indication on the preferred or expected network energy saving behaviours for its application’s service data flows or its UEs. The AF sends Nnef_NetworkEnergySaving_Creat Request to the NEF to provide the indication on the expected network energy saving behaviours, e.g., QoS adjustment. The AF may provide the energy consumption threshold at certain granularity (e.g., per UE per application, per UE) to inform the network when to perform the network energy saving behaviours.

NOTE 1:	The operators could provide several network energy saving behaviours, e.g., QoS adjustment, which may be configured in the PCFs of the PLMN. The AF may prefer certain network energy saving behaviours provided by the operators, so the AF indicates its preferred network energy saving behaviours to the NEF and negotiate the network energy saving behaviours with the operators.

NOTE 2: The AF may provide the preferred network energy saving behaviours for its own applications on the service data flows. The AF may provide the preferred network energy saving behaviours for the UEs only if the UEs are owned or managed by this AF.

NOTE 3: The existing NEF service may be reused for the interaction between the NEF and AF in this step. Whether to define the new NEF service or reuse the existing NEF service is FFS.

2.	The NEF authorize the AF requested service based on the SLA between the AF and the operators. If the request from the AF is accepted, the NEF will respond to the AF to indicate the result. If the network can not accept certain network energy saving behaviours, the NEF may respond to the AF about the result.

3.	If the NEF accepts certain network energy saving behaviours, the NEF sends the information to the UDR via Nudr_DM_Create including the expected network energy saving behaviours and the related energy consumption value for applying the behaviour.

4.  The UDR further sends the information to the PCF via Nudr_DM_Notify.

5.  Based on the UE’s subscription for network energy saving and the AF request, the PCF may subscribe the energy consumption information notify from EIF via Neif_EventExposure_Subscribe/Notify.

6.	The PCF makes the policy decisions based on the UE subscription (Energy saving indicator), EIF notification and AF request.

6.x.3	Impacts on existing services, entities and interfaces

AF:

-	provide the indication on the preferred or expected network energy saving behaviours.

-  may also provide the corresponding energy consumption threshold for certain granularity (e.g., per UE per application, per UE) to inform the network when to perform the network energy saving behaviours;

NEF:

-	support the inaction with AF about the preferred or expected network energy saving behaviours.

PCF:

-	make the policy decisions for the network energy saving behaviours based on the AF request and energy related information.

UDR:

-	New parameters addition related with network energy saving behaviours from AF.

End of CHANGES