3GPP SA WG2 Meeting #168	S2-2504102

# Gothenburg, Sweden, 7-11 April 2025												(was S2-2503872)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 6270 | rev | 1 | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on MWAB URSP processing |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | VMR_Ph2 |  |  |  |  |  | Date: |  |  | 2025-03-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Clause 5.49.2.2 have below text: “The OAM server of the MWAB Broadcasted PLMN/SNPN provides (re-)configuration parameters ……………… to assist the MWAB-gNB providing information used by MWAB-UE for the BH PDU Session(s) management via URSP processing” SA5 is requesting to clarify which exact parameters in this has to be configured by OAM. When applying URSP rules, the MWAB-UE makes use of the information provided by the MWAB-gNB to match the corresponding traffic descriptors. Therefore, the OAM server should configure the MWAB-gNB on the traffic descriptor related information. The OAM server is not required to understand the URSP rules. However, the OAM server should coordinate with the with HPLMN/subscribed SNPN of MWAB-UE to ensure that the traffic descriptor information is configured properly. . |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify the information provided by MWAB-gNB to MWAB-UE for URSP processing is the traffic descriptor related information and the configuration needs to be coordinated with the HPLMN/SNPN of the MWAB-UE . |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear specification for stage 3 development. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.49.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*************************** Start of Changes ***************************

#### 5.49.2.2	Configuration of the MWAB-gNB

The MWAB-gNB's configuration is managed by the OAM of the MWAB Broadcasted PLMN/SNPN and the address (or FQDN) of the OAM server per MWAB Broadcasted PLMN/SNPN may be configured at the MWAB-gNB. The OAM server of the MWAB Broadcasted PLMN/SNPN provides (re-)configuration parameters considering location of MWAB as specified in TS 38.401 [42], e.g. for the purpose including the access stratum operation of the MWAB-gNB, the N2, N3 and Xn interface management, activating/deactivating the MWAB-gNB operation and to assist the MWAB-gNB providing traffic descriptor information(see clause 5.49.1.2) used by MWAB-UE for the BH PDU Session(s) management via URSP processing.

NOTE 1:	It is expected the OAM of the MWAB Broadcasted PLMN/SNPN is aware of location in which the MWAB operation can be authorized and also can obtain MWAB's current location. Only when the MWAB is authorized successfully, the configuration information can be provisioned to the MWAB-gNB. Alternatively, the OAM can also provision configuration parameters related to different areas, as defined in TS 38.401 [42].

The MWAB-gNB shall be configured with the supported S-NSSAI(s) in the MWAB Broadcasted PLMN/SNPN.

NOTE 2:	A MWAB-gNB is not expected to be configured to support any dedicated S-NSSAI for BH PDU Session(s) used by MWAB-UEs for MWAB operation in the MWAB Broadcasted PLMN/SNPN as specified in TS 38.401 [42]. The support at a MWAB-gNB of S-NSSAIs dedicated to MWAB-UE BH PDU Session(s) in the MWAB Broadcasted PLMN/SNPN would mean that the MWAB-gNB could support serving the MWAB-UEs BH-PDU Session(s) in the broadcasted PLMN/SNPN.

NOTE 3:		The OAM of the MWAB Broadcasted PLMN/SNPN is expected to co-ordinate with HPLMN/subscribed SNPN of the MWAB-UE to configure the traffic descriptors related information in the MWAB-gNB if it is needed. The co-ordination between OAM and HPLMN/subscribed SNPN is outside the scope of 3GPP.

*************************** End of Changes ****************************