3GPP SA WG2 Meeting #168	S2-2504128

# April 7 - 11, 2025, Goteborg, SE     				             			   (revision of S2-2503673)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.256 | CR | 0199 | rev | 1 | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | The indication on altitude thresholds and reporting periodicity for the aerial UE's altitude reporting |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UAS_Ph3 |  |  |  |  |  | Date: |  |  | 2025-3-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Based on R3-250876, this propose aims to clarify how the CN indicates altitude thresholds and reporting periodicity for the aerial UE's altitude reporting to RAN via re-using the Location Report Control mechanism as define in clause 8.12 in TS 38.413. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarification is made on indicating altitude thresholds and reporting periodicity for the aerial UE's altitude reporting to RAN. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | How to indicate altitude thresholds and reporting periodicity for the aerial UE's altitude reporting to RAN is unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.16.2,5.16.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

>>>> Start of Changes <<<<

5.16.2	Instructing aerial UEs to perform altitude reporting

Procedure shown in Figure 5.16.2-1 specifies how the 5GS instructs the aerial UEs (UAVs) to perform altitude reporting, considering requests from a USS/UTM for assistance with UAV's flight. This procedure can be used in conjunction with the NEF-assisted pre-flight planning, in-flight monitoring and USS changeover as specified in clauses 5.12.2, 5.12.3 and 5.13, respectively.

Figure 5.16.2-1: Procedure for instructing aerial UEs for altitude reporting

1.	UAS NF/NEF assists the USS/UTM with pre-flight planning, in-flight monitoring and/or USS changeover based on the received request, which is either a Nnef_RetrieveInfoUAVFlight_Get or a Nnef_UAVFlightAssistance_Create, as specified in clauses 5.12 or 5.13.

2.	Based on the flight path information included in step 1 and other information obtained as a result of either of the procedures (i.e. NEF-assisted pre-flight planning specified in clause 5.12.2, NEF-assisted in-flight monitoring specified in clause 5.12.3, or USS changeover specified in clause 5.13.2), the UAS NF/NEF determines minimum and maximum values of altitudes (i.e. altitude range) for UAV's flight across each of the determined tracking areas (TAs) from the starting point to the destination point.

The determined minimum and maximum altitude values for each of the UAV UE is used by the UAS NF/NEF to derive altitude thresholds for altitude reporting events.

NOTE 1:	Event H1 and H2 for altitude-based reporting specified in clause 5.5.4.21 and clause 5.5.4.22 of TS 38.331 [23] can be used for altitude thresholds and event-based reporting.

Additionally, the UAS NF/NEF determines whether to request the aerial UE to report its altitude information periodically, threshold-based (e.g. altitude becomes a higher/lower than the provided threshold, multiple threshold values can be provided) or event-based (e.g. change of a tracking area/NG-RAN node).

3.	Delivering the UAS NF/NEF-derived altitude thresholds and, optionally, the indication about reporting periodicity (i.e. threshold-based, event-based, or periodic) for UE's altitude reporting to the aerial UE(s).

Option A: Application layer.

3a.	The UAS NF/NEF sends the derived altitude threshold values and provides indication about the required periodicity of the reporting (on demand, event-triggered, periodic, or per change of the altitude, or change of a specific geographical area) to the serving USS/UTM either in a Nnef_RetrieveInfoUAVFlight_Get response message or in a Nnef_UAVFlightAssistance_Notify message depending on the procedure that has triggered the UAS NF/NEF to derive the altitude thresholds for the aerial UE.

4a.	The USS/UTM sends the altitude reporting thresholds and, optionally, indication about reporting periodicity (and any additional information required to perform the reporting, e.g. a list of TA(s)/NG-RAN node identifier(s) for event-based reporting, periodicity value for periodic reporting) to the UAV over the application layer for the UAV's consumption.

NOTE 2:	Application layer reporting itself is outside of 3GPP scope.

Option B: 3GPP-defined signalling.

Editor's note:	This requires RAN to provide further enhancement of the procedure under RAN responsibility.

3b.	The UAS NF/NEF may send an Namf_EventExposure_Subscribe request to the AMF to get notified about events of UAV's deviation from the assigned trajectory (i.e. Event ID = "Assigned Trajectory") as specified in clause 5.2.2.3.1 of TS 23.502 [3]. Inside the subscription request, the UAS NF/NEF may include the derived altitude thresholds and indication about reporting periodicity for the UE's altitude reporting.

Additionally, the UAS NF/NEF may also include an indication to perform UE's altitude reporting for all aerial UEs in a specific TA(s) or NG-RAN node(s).

4b.	The AMF sends the corresponding N2 messages (LOCATION REPORTING CONTROL message, see clause 8.12.1.2 of TS 38.413 [24]) to all applicable NG-RAN node(s), including the derived altitude thresholds and/or reporting periodicity.

5b.	The NG-RAN node(s) instructs the relevant UE(s) to perform altitude reporting as requested by the UAS NF/NEF (via AMF).

NOTE 3:	Aspects related to AMF informing NG-RAN regarding instructing UEs to perform altitude reporting are specified in TS 38.413 [24].

5.16.3	Reporting UE's altitude information

A procedure for reporting aerial UE's altitude information to the core network is shown in Figure 5.16.3-1. The reporting can be done by the aerial UE based on the received altitude reporting thresholds and indication about reporting periodicity as specified in clause 5.16.2.

Figure 5.16.3-1: Procedure for reporting of UE's altitude information

0.	An aerial UE receives request to perform its altitude information reporting as specified in clause 5.16.2. The request may include altitude reporting thresholds and, optionally, indication about reporting periodicity.

1.	UE's flight altitude meets the previously provided altitude reporting conditions (i.e. altitude thresholds and, optionally, reporting periodicity).

Option A: Application layer.

2a.	The UE may report the altitude information over the application layer to the serving USS/UTM in case the USS/UTM has provided altitude reporting threshold and, optionally, indication about reporting periodicity in step 4a in clause 5.16.2.

NOTE 1:	Application layer reporting itself is outside of 3GPP scope.

3a.	The USS may invoke the Nnef_UAVFlightAssistance_Update to inform the UAS NF/NEF about UE's altitude and time stamp of the altitude.

Option B: 3GPP-defined signalling.

1b.	The UE provides a report with the altitude information to the NG-RAN.

NOTE 2:	Aspects related to a UE providing altitude information to an NG-RAN node are specified by RAN in TS 38.331 [23].

2b. After receiving a report with the altitude information from the UE, the NG-RAN node should, for instance, add a time stamp indicating when the altitude information has been received from the UE; the NG-RAN sends the UE's altitude information report in an N2 message (LOCATION REPORT message, see clause 8.12.3.2 of TS 38.413 [24]) to the AMF.

3b.	The AMF provides the UAS NF/NEF the altitude information report inside a Namf_EventExposure_Notify request.

4.	The UAS NF/NEF together with the USS determines the impact on UAV's flight path based on the received altitude information report; this aspect is outside the scope of the 3GPP.

>>>> End of Change <<<<