3GPP TSG-WG SA2 Meeting #168	S2-2504270

Goteborg, SE, 7th April – 11th April, 2025	was S2-2504015, S2-2503475

Source:	China Mobile, ZTE, Futurewei, Samsung

Title:	pCR for TS 23.369: AIoT Reader Selection

Document for:	Approval

Agenda Item:	19.14.2

Work Item / Release:	AmbientIoT-ARC / Rel-19

Abstract: address ENs about AIoT Reader Selection

# 1. Introduction/Discussion

LS from RAN3 (R3-250905):

RAN3 would like to inform SA2 and SA5 about the following RAN3 progress related to Architecture and OAM aspects:

- A-IoT RAN node is an aggregated gNB. And this gNB may or may not only provide the A-IoT radio.

- Support both the direct and indirect communication options between the gNB and the AIOTF in Topology 1.

- Working assumption: Including AIOTF information containers in NGAP, rather than introducing new protocol carried by NGAP.

- It is FFS whether to define a new A-IoT Area, or use Tracking Area dedicated for A-IoT, to support e.g., the selection of gNBs by the AIOTF.

- AIOTF obtains the A-IoT RAN information (supported Area, served reader ID list) via OAM configuration, FFS on RAN3 signalling for the A-IoT RAN information.

- AIOTF may also be aware of the other A-IoT RAN information (location of reader) via OAM configuration, FFS on RAN3 signalling for the other A-IoT RAN information...

- In Inventory Request, gNB receives the requested Service Area Information (encoded as Area and/or reader ID list) from AIOTF.

- The gNB always sends the A-IoT device’s location at reader ID granularity to the AIOTF in Inventory Report.

- “Global gNB ID + Reader index" is used to uniquely identify the Reader globally.

As RAN3 concluded, AIOTF obtains the A-IoT RAN information (supported Area, served reader ID list) via OAM mechanism. The Inventory Request includes the requested supported area Information (encoded as Area and/or reader ID list).

The following 2 ENs can be resolved.

Editor’s note:	Whether and how the AIOTF chooses the RAN reader(s) is FFS.

Editor’s note:	AIoT RAN and RAN reader information needs to coordinate with the RAN WG(s). Details are pending RAN WG feedback.

Proposal 1: The AIOTF selects the AIoT RAN node(s) and, optionally, AIoT RAN’s requested area information ( Area and/or AIoT reader ID list) based on the internal area information in the service request and its local configurations via OAM.

The operator and AF need to coordinate to be aware of the AIoT RAN’s supported area granularity information. AF uses the external target area information to represent one or multiple the AIoT RAN’s supported service areas. The NEF needs to convert this information into internal area information (e.g., RAN’s supported area) based on match between external target area information and its local configurations via OAM. Local configuration has the mapping relationship between target area information and RAN’s supported area.

AF may use the physical Area (e.g., Geo-location --civic address or polygon with coordinates) or Area ID to represent external target area information. However, the target area information provided by the AF has to be completely match one or more AIoT RAN’s supported areas. It can’t be partial match of the one AIoT RAN’s supported area. If it is a partial match of the one AIoT RAN’s supported area, the core network can’t instruct the AIoT RAN reader to inventory only designated physical area. From the AF side, it expects the AIoT system to inventory the designated physical area only, but the RAN readers just can inventory one or more complete supported areas. Then, it means that the product items, not in designated physical area requested by AF, will be reported to AF unexpectedly.

The relationship between the target area information (e.g., physical area or Area ID) and the internal area information (e.g., RAN’s supported area) is established collaboratively by the operator and AF. This is based on factors such as the supported area(s) from AIoT RAN,and corresponding external target area from AF.

Proposal 2: AF may provide target area information in the format of location identifier or Physical Area. NEF converts the target area information into internal area information (e.g., RAN’s supported service area) based on local configuration.

NOTE:   The target area information provided by the AF needs to completely match one or more of the AIoT RAN’s supported areas. It cannot be a partial match of a single AIoT RAN’s supported area.

NOTE:	The mapping relationship between the target area information and the internal area information (e.g., RAN’s supported service area) is determined collaboratively by the operator and AF, based on factors such as the AIoT RAN’s supported areas and external target area information.

Then, the following 2 ENs can be resolved.

Editor’s note:	It is FFS how to handle the case that the AF provides other information than target area information, for the purpose of reader selection. In such case how the NEF and/or AIOTF use it to select the RAN reader.

Editor’s note:	How the NEF converts target area information to internal area information and the format of the internal area information is FFS.

# 2. Text Proposal

It is proposed to capture the following changes in TS 23.369 v0.2.0.

# * * * * First change * * * *

## 5.3.x	NG-RAN Node and RAN Reader Selection

The AIOTF selects NG-RAN node(s) and optionally RAN reader(s).

The AIOTF obtains the NG-RAN information (supported Area, RAN reader ID list, and, optionally, the location of each served RAN reader) via OAM. The AIOTF receives an AIoT service request including the Target Area information from the NEF or trusted AF. Based on the received Target Area information and the NG-RAN information configured by OAM, the AIOTF selects the NG-RAN node(s) and optionally RAN reader(s).

NOTE:  The Target Area information can span the supported Area of multiple NG-RAN node(s) or can be a subset of the supported Area of a NG-RAN node.

The AIOTF sends the AIoT service request to the selected NG-RAN node(s), optionally including the RAN Area information derived from mapping the Target Area information to the supported Area and/or RAN reader ID list, if selected, to assist RAN reader selection by NG-RAN node, either directly or through the selected AMF. If multiple NG-RAN nodes are selected, the AIOTF sends the AIoT service request to each selected NG-RAN node along with its corresponding RAN Area information and/or the RAN reader ID list, if selected.

If the AIOTF does not provide the RAN Area information or the RAN reader ID list in the AIoT service request to the NG-RAN node, then the NG-RAN node may use all available RAN readers.

Editor’s note:	AIoT RAN and RAN reader information needs to coordinate with the RAN WG(s). Details are pending RAN WG feedback.

Editor’s note:	The AIOTF or AMF configuration of AIoT RAN and RAN reader information over NGAP needs to coordinate with RAN WG(s).

Editor’s note:	It is FFS how AIoT RAN transfers and updates its information to the AIOTF in indirect path.

NOTE:	RAN reader ID is not exposed to the AF.

If an AIoT service request includes AIoT device Identifier(s), the AIOTF may consider the last known serving RAN reader(s) from the context data to determine targeted RAN reader(s) directly.

NOTE:  From the selected NG-RAN node, the AIOTF receives the Inventory Report including the RAN reader ID that represents the AIoT device’s location at reader ID granularity. The AIOTF uses the RAN reader ID to update the last known serving RAN reader information in the local AIoT device context.

# * * * * End of changes * * * *