3GPP TSG-WG SA2 Meeting #168	S2-2504358

# Goteborg, SE, 7th April – 11th April, 2025		(revision of S2-2503431)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.503 | CR | 1537 | rev | 1 | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Terminology alignment for steering functionality(ies) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | MASSS |  |  |  |  |  | Date: |  |  | 2025-03-28 |
| Category: | D |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Based on previous meeting discussion, it is agreed to use the terms of MPQUIC-UDP functionality, MPQUIC-IP functionality and MPQUIC-IE functionality to present the different steering functionality. Therefore, this CR would like to further propose editorial corrections to keep terminologies aligned. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Terminology alignment for clause 6.1.3.20 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Terminology is not aligned which is misleading. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.3.20 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

6.1.3.20	Access Traffic Steering, Switching and Splitting

As specified in TS 23.501 [2], the Access Traffic Steering, Switching and Splitting (ATSSS) feature is an optional feature that may be supported by the UE and the 5GC network.

The ATSSS feature enables a multi-access PDU Connectivity Service, which can exchange PDUs between the UE and a data network by simultaneously using one 3GPP access network and one non-3GPP access network (both connected to 5GC) when both accesses are allowed for the same S-NSSAI. The multi-access PDU Connectivity Service also supports the exchange of PDUs between the UE and a data network by simultaneously using one 3GPP access network in EPC and one non-3GPP access network in 5GC, or one 3GPP access network in 5GC and one non-3GPP access network in EPC, as described in TS 23.501 [2]. This enables a scenario where a MA PDU Session can simultaneously be associated with user-plane resources on 3GPP access network connected to 5GC or EPC and non-3GPP access connected to 5GC, or with user-plane resources on 3GPP access network connected to 5GC and non-3GPP access connected to EPC.

The PCF is informed of the ATSSS capabilities of a MA PDU Session by the SMF, as defined in clause 5.32.2 of TS 23.501 [2]. The ATSSS capabilities are both the Steering Mode and the Steering Functionality.

The PCF control of Access Traffic Steering, Switching and Splitting for a detected service data flow (SDF) is enabled by including Multi-Access PDU (MA PDU) Session Control information in the PCC rule. This allows the PCF to control:

-	The Steering Mode that is used to steer/switch/split the detected SDF. The available Steering Modes are defined in TS 23.501 [2].

-	The Steering Functionality that is used for the detected SDF, e.g. the MPTCP functionality, the ATSSS-LL functionality, the MPQUIC-UDP functionality, the MPQUIC-IP functionality, or the MPQUIC-E functionality defined in TS 23.501 [2].

-	The Steering Mode Indicator authorized for the detected SDF.

-	The Threshold values for RTT and Packet Loss Rate authorized for the detected SDF.

-	The Charging information depending on what Access Type is used for a detected SDF.

-	The Usage Monitoring information depending on what Access Type is used for a detected SDF.

-	Transport Mode that is used for the detected SDF. The available Transport Modes are defined in TS 23.501 [2].

The rest of the information in the PCC Rule apply to the SDF as such and are not dependent on what Access Type is used for a packet.

The MA PDU Session Control information in the PCC rules is used by the SMF in order to create applicable N4 rules for the UPF and ATSSS rules for the UE, as described in TS 23.501 [2]. The ATSSS rules are sent to UE via NAS when the MA PDU Session is created or updated by the SMF/PCF, as described in TS 23.501 [2] and TS 23.502 [3].

When MA PDU Session Control Information is provided to the SMF within a PCC Rule, the (H-)PCF provides both the Service Data Flow templates to identify a Service Data Flow in the UPF and if the Service Data Flow template includes an application identifier, then the corresponding Application descriptors to identify the application traffic in the UE is also included.

The (H-) PCF may use the OSid stored in the UDR as DataSet "Policy Data" and Data Subset "UE context policy control data" to determine the OSAppId supported by the OSid. The (H-)PCF may also provide multiple Application descriptors to identify application traffic in the UE, this is determined by the (H-)PCF local policies that indicates e.g. the operating system supported by the UE. If no OSid is available in the UDR, the (H-)PCF may use the PEI to determine the OSid supported by the UE.

NOTE 1:	If the (H-)PCF does not take into account the received PEI and/or OSId then the (H-)PCF can send PCC rules containing application traffic descriptors associated to multiple operating systems.

The Traffic Descriptor in the ATSSS rule is generated by the SMF from the SDF template of the PCC rule. If the SDF template contains SDF filters, the SMF uses the UL SDF filters for the generation of the IP descriptors or Non-IP descriptors, respectively. If the SDF template contains an application identifier, the SMF includes the Application descriptors received from the PCF as part of the MA PDU Session information in the PCC Rule within the Traffic Descriptors in the ATSSS rule.

For the Load-Balancing steering mode with fixed split percentages (i.e. without the Autonomous load-balance indicator or UE-assistance indicator), the PCF may provide one or more threshold values together with the split percentages. For the Priority-based steering mode, the PCF may provide one or more threshold values together with the priority of the accesses. For the Redundant steering mode, the PCF may provide one threshold value and/or a primary access. For the Load-Balancing and the Priority-based steering mode, one threshold value for the Round Trip Time (RTT) and/or one threshold value for the Packet Loss Rate (PLR) may be included in a PCC Rule. For the Redundant steering mode, either one threshold value for the Round Trip Time (RTT) or one threshold value for the Packet Loss Rate (PLR) may be included in a PCC Rule based on local configuration or QoS requirements received from AF as specified in clause 4.15.6.6 or clause 4.15.6.6a of TS 23.502 [3]. The threshold values are not dependent on what Access Type is used for a packet, i.e. a given threshold value is applicable to both accesses. The threshold values are applied by the UE and UPF as described in TS 23.501 [2].

NOTE 2:	The Round Trip Time (RTT) threshold value can be determined based on the PDB of the 5QI authorized for the SDF, and the Packet Loss Rate (PLR) threshold value can be determined based on the PER of the 5QI authorized for the SDF.

The MA PDU Session Control information in a PCC rule may contain only one of the following Steering Mode Indicators:

-	Autonomous load-balance indicator: This indicator may be included only when the Steering Mode is Load-Balancing and indicates whether autonomous load-balance operation is allowed. Further details are specified in clause 5.32.8 of TS 23.501 [2].

-	UE-assistance indicator: It indicates that the UE can decide how to distribute the UL traffic based on its internal state (e.g. battery level), and that the UE can request from UPF to apply the same distribution for the DL traffic. Further details are specified in clause 5.32.8 of TS 23.501 [2].

The PCF may also provide URSP rules to the UE for instructing the UE to establish a MA PDU Session, as described in clause 6.6.2.

The PCF control of PDU Session level Usage Monitoring depending on what access type is used to carry the traffic is enabled by providing Usage Monitoring control related information per access in the PDU Session related policy control information (as described in clause 6.4).

The Transport Mode may be included in a PCC rule that is used for the detected SDF only when the Steering functionality is the MPQUIC-UDP functionality, the MPQUIC-IP functionality, or the MPQUIC-E functionality.

NOTE 3:	The MPQUIC-UDP functionality, the MPQUIC-IP functionality and the MPTCP functionality are only applied for IP MA PDU Session.

NOTE 4:	The MPQUIC-E functionality is only applied for Ethernet MA PDU Session.

If the MA PDU Session is capable of supporting one of the following:

-	MPTCP and ATSSS-LL with any Steering Mode (i.e. any Steering Mode allowed for ATSSS-LL) in the downlink and MPTCP and ATSSS-LL with Active-Standby in the uplink;

-	MPQUIC-UDP and ATSSS-LL with any Steering Mode (i.e. any Steering Mode allowed for ATSSS-LL) in the downlink and MPQUIC-UDP and ATSSS-LL with Active-Standby in the uplink;

-	MPTCP, MPQUIC-UDP and ATSSS-LL with any Steering Mode (i.e. any Steering Mode allowed for ATSSS-LL) in the downlink and MPTCP, MPQUIC-UDP and ATSSS-LL with Active-Standby in the uplink;

then the PCF shall provide a PCC Rule for "match all" traffic. This PCC Rule contains a "match all" SDF template, the lowest precedence, the Steering Functionality set to "ATSSS-LL" and the Steering Mode set to "Active-Standby" for the uplink direction, and the Steering Functionality set to "ATSSS-LL" and the Steering Mode set to any supported steering mode (i.e. any Steering Mode allowed for ATSSS-LL) for the downlink direction.

If the MA PDU Session is capable of supporting one of the following:

-	MPTCP with any Steering Mode in the downlink, ATSSS-LL with any steering mode except Smallest Delay steering mode and Redundant steering mode in the downlink, and MPTCP and ATSSS-LL with Active-Standby in the uplink;

-	MPQUIC-UDP with any Steering Mode in the downlink, ATSSS-LL with any steering mode except Smallest Delay steering mode and Redundant steering mode in the downlink, and MPQUIC-UDP and ATSSS-LL with Active-Standby in the uplink;

-	Both MPTCP and MPQUIC-UDP with any Steering Mode in the downlink, ATSSS-LL with any steering mode except Smallest Delay steering mode and Redundant steering mode in the downlink, and MPTCP, MPQUIC-UDP and ATSSS-LL with Active-Standby in the uplink;

then the PCF shall provide a PCC Rule for "match all" traffic. This PCC Rule contains a "match all" SDF template, the lowest precedence, the Steering Functionality set to "ATSSS-LL" and the Steering Mode set to "Active-Standby" for the uplink direction, and the Steering Functionality set to "ATSSS-LL" and the Steering Mode set to any supported steering mode except Smallest Delay steering mode and Redundant steering mode for the downlink direction.

If the MA PDU Session is capable of supporting one of the following:

-	MPTCP and ATSSS-LL with Active-Standby in the uplink and downlink;

-	MPQUIC-UDP and ATSSS-LL with Active-Standby in the uplink and downlink;

-	MPTCP, MPQUIC-UDP and ATSSS-LL with Active-Standby in the uplink and downlink;

then the PCF shall provide a PCC Rule for "match all" traffic. This PCC Rule contains a "match all" SDF template, the lowest precedence, the Steering Functionality set to "ATSSS-LL" and the Steering Mode set to "Active-Standby" for the uplink direction and the downlink direction.

If the MA PDU Session is capable of supporting one of the following:

-	MPTCP and ATSSS-LL with any Steering Mode (i.e. any Steering Mode allowed for ATSSS-LL) in the uplink and downlink;

-	MPQUIC-UDP and ATSSS-LL with any Steering Mode (i.e. any Steering Mode allowed for ATSSS-LL) in the uplink and downlink;

-	MPTCP, MPQUIC-UDP and ATSSS-LL with any Steering Mode (i.e. any Steering Mode allowed for ATSSS-LL) in the uplink and downlink;

then the PCF shall provide a PCC Rule for "match all" traffic. This PCC Rule may contain a "match all" SDF template, the lowest precedence, the Steering Functionality set to "ATSSS-LL" and the Steering Mode set to any supported steering mode for the uplink direction and for the downlink direction (i.e. any Steering Mode allowed for ATSSS-LL).

If MA PDU Session is capable of supporting both MPQUIC-E and ATSSS-LL functionalities in the uplink and downlink; then the PCF either sets the Steering Functionality to MPQUIC-E for all the PCC rules, or sets the Steering Functionality to ATSSS-LL for all the PCC rules, but not both.

For other cases, the PCF shall provide a PCC rule for "match all" traffic, as follows:

-	If the MA PDU Session is capable of supporting MPQUIC-IP functionality (and zero or more other steering functionalities), the PCC Rule for "match all" traffic may contain a "match all" SDF template, the lowest precedence and the Steering Functionality set to "MPQUIC-IP" and the Steering Mode set to any supported steering mode for the uplink direction and for the downlink direction.

-	If the MA PDU Session is capable of supporting MPQUIC-E functionality (and zero or more other steering functionalities), the PCC Rule for "match all" traffic may contain a "match all" SDF template, the lowest precedence and the Steering Functionality set to "MPQUIC-E" and the Steering Mode set to any supported steering mode for the uplink direction and for the downlink direction.

-	If the MA PDU Session is capable of supporting ATSSS-LL functionality with any steering mode (and zero or more other steering functionalities), the PCF PCC Rule for "match all" traffic may contain a "match all" SDF template, the lowest precedence and the Steering Functionality set to "ATSSS-LL" and Steering Mode set to any supported steering mode (i.e. any Steering Mode allowed for ATSSS-LL) for the uplink direction and for the downlink direction.

-	If the MA PDU Session is capable of supporting ATSSS-LL functionality with Active Standby steering mode, the PCC Rule for "match all" traffic may contain a "match all" SDF template, the lowest precedence and the Steering Functionality set to "ATSSS-LL" and the Steering Mode set to "Active-Standby".

The Steering functionality "ATSSS-LL" shall not be provided together with Steering Mode "Redundant".

These PCC Rules are used by the SMF to generate an ATSSS rule for the UE and an N4 rule for the UPF to route the "match all" traffic of the MA PDU Session in the uplink and downlink direction respectively.

NOTE 5:	The PCF can also use the ATSSS capability of the MA PDU Session to provide PCC Rules containing SDF template for some specific non-MPTCP traffic or non-MPQUIC traffic other than the PCC Rule containing a "match all" SDF template. This allows the operator to apply different policies e.g. charging key to non-MPTCP traffic or non-MPQUIC traffic other than the non-MPTCP traffic or non-MPQUIC matching the "match all" PCC Rule.

When the Redundant steering mode is used and resources are not available in one access, the SMF will notify the PCF about the resource allocation failure and indicate the respective Access Type (as described in clause 6.1.3.5). The PCF can then determine by implementation specific means how to proceed with such a GBR SDF for which redundant traffic transmission is no longer possible.

NOTE 6:	The PCF can e.g. provide the PCC rule again in order to trigger another resource allocation, modify the MA PDU Session Control information in the PCC rule or remove the PCC rule (and inform the AF accordingly).

# * * * * End of changes * * * *