3GPP SA WG2 Meeting #168	S2-2502911

# 7-11 April 2025, Goteborg, Sweden							                 (was S2-2502435)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1603 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#8: Corrections to Avatar procedures |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_Ph2 |  |  |  |  |  | Date: |  |  | 2025-03-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Corrections to UE centric and network centric Avatar procedures |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that the associated URL is used to download Avatar representation of an Avatar ID. Correct the applicability of option 1 and 2 for network centric procedure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | AC.11.3.2.1, AC.11.3.2.2, AC.11.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# >>>>BEGINNING OF CHANGES <<<<

#### AC.11.3.2.1	Sending UE centric procedure

Figure AC.11.3.2.1-1: Procedures of Sending UE centric IMS Avatar communication

Figure AC.11.3.2.1-1 depicts a typical call flow procedure of sending UE centric IMS Avatar communication. The main steps in the call flow are as follows:

1.	IMS audio/video session and bootstrap data channel have been established. UE1 downloads avatar communication application. The avatar ID list is downloaded as described in clause AC.11.1. Then UE1 chooses an avatar ID and runs the avatar communication application. If the avatar representation is locally available in UE1, steps 2-6 are skipped.

2.	P2A application data channel(s) are established for avatar representation transmission. The rendering mode is set to 'UE centric' by the DC AS. The DCSF instructs the MF to establish MDC2 connection either to the BAR or the DC AS.

NOTE 1:	In this step, the UE fetches the associated URL associated with the requested Avatar ID. The associated URL enables the UE to download the Avatar representation associated with the Avatar ID.

3.	UE1 requests downloading Avatar representation from BAR via MF using the associated URL and UE1's identity (i.e. MSISDN) through the established application data channel. The MF, based on the instruction received in step 2 fetches the Avatar represention from the BAR or DC AS using the resource URL for that purpose.

4-5.	The BAR authorizes the request of downloading Avatar representation and responses with avatar representation data to the MF.

NOTE 2:	The MF downloads UE1's avatar representation from BAR directly or indirectly via DC AS based on the MDC2 connection established in step 2.

6.	The MF sends the avatar representation data to UE1.

7.	UE1 performs avatar animation with the selected Avatar representation.

8.	UE1 transmits the animated video stream over RTP to UE2.

# >>>>NEXT CHANGE<<<<

#### AC.11.3.2.2	Receiving UE centric procedure

Figure AC.11.3.2.2-1: Procedures of Receiving UE centric IMS Avatar communication

Figure AC.11.3.2.2-1 depicts a typical call flow procedure of receiving UE centric IMS Avatar communication. The main steps in the call flow are as follows:

1.	IMS session and bootstrap data channel have been established. UE1 downloads avatar communication application. The avatar ID list is downloaded as described in clause AC.11.1. Then UE1 chooses an avatar ID and runs the avatar communication application.

2.	P2A2P application data channel(s) are established between UE1/UE2 and DC AS. The rendering mode is set to 'UE centric' by the DC AS. The DCSF instructs the MF to establish MDC2 connection either to the BAR or the DC AS.

NOTE 1:	In this step, the UE fetches the associated URL associated with the requested Avatar ID. The associated URL enables the UE to download the Avatar representation associated with the Avatar ID.

3.	UE1 decides to request peer UE to perform avatar animation based on its status such as power, signal, computing power, internal storage, etc.

4.	UE1 performs avatar animation negotiation with the DC AS and UE2. The negotiation includes usage of the UE1's avatar ID and the animation data types (e.g. text and/or facial expression) supported by UE1.

Editor's note:	The negotiation procedure needs to be further defined in SA WG4, and cooperation with SA WG4 is needed.

5.	UE2 requests downloading avatar representation from BAR via MF using the associated URL received from UE1 through the established application data channel in step 2. The MF, based on the instruction received in step 2 fetches the Avatar represention from the BAR or DC AS using the Resource URL for that purpose.

6-7.	The BAR authenticates and authorizes the request of downloading avatar representation using UE1's avatar ID and responses with UE1's avatar representation data. The DC AS should authorize the UE2 download request based on the negotiation result from step 4.

NOTE 2:		The MF downloads UE1's avatar representation from BAR directly or indirectly via DC AS based on MDC2 connection established in step 2.

8.	The MF sends UE1's Avatar representation data to UE2.

9.	Based on the avatar animation negotiation result in step 4, UE1 may send metadata (e.g. facial expression) to UE2 through the established application data channel or audio/video through RTP channel between UE1 and UE2.

10.	UE2 performs avatar animation with the UE1's avatar representation based on the received metadata or audio/video from UE1.

# >>>>NEXT CHANGE<<<<

### AC.11.3.3	Network centric procedure

Figure AC.11.3.3-1: Procedures of network centric IMS Avatar communication

Figure AC.11.3.3-1 depicts a typical call flow procedure of network centric IMS avatar communication. The main steps in the call flow are as follows:

1.	IMS session and bootstrap data channel have been established. UE1 downloads avatar communication application. The avatar ID list is downloaded as described in clause AC.11.1. Then UE1 chooses an avatar ID and runs the avatar communication application.

2.	UE1 decides to request network to perform avatar animation based on its status such as power, signal, computing power, internal storage, etc.

3.	UE1 initiates application data channel establishment procedure. Application data channel(s) are established between UE1 and DC AS, and UE2 and DC AS.

4.	The avatar animation negotiation is finished between the DC AS and UE1 via the application data channel established in step 3.

Editor's note:	The avatar animation negotiation procedure and parameters need to be further defined in SA WG4, and cooperation with SA WG4 is needed.

5.	If the negotiation result in step 4 is that the network will perform the rendering, the UE1 initiates new P2A2P or P2P application data channels, which are specifically used for avatar rendering-related data transmission between the UE1/UE2 and the network.

NOTE 1:	The  DC AS decides what type of application data channel would be needed for network centric procedure.

6-7.	During the application data channel establishment procedure, the IMS AS notifies the DCSF via Nimsas_SessionEventControl_Notify of the media change request event. The DCSF checks that the app id in the ADC establishment request belongs to Avatar communication DC app, subsequently, the DCSF fetches DC AS URL from DCSF service specific data or configured locally in DCSF.

8-9.	The DCSF retrieves the avatar media specification data from DC AS using DC AS URL. The DCSF also receives additional instruction on what type of application data channel should be used and what type of rendering mode (e.g. network centric with MF rendering, network centric with DC AS rendering) applies for network-centric rendering initiated in step 5, as well as the Resource URL.

NOTE 2:	The Service operation of step 8-9 is not specified in this release.

10-11.	The DCSF instructs the MF via IMS AS how to establish the data channel and corresponding avatar media processing specification, via Nimsas_MediaControl_MediaInstruction with the parameters set according to the decisions in step 8-9. If the rendering mode is network centric with MF rendering, IMS AS selects a MF that support network centric with MF rendering based on local configuration or via NRF.

12.	The IMS AS sends the MediaControl Instruction response to the DCSF.

13-18.	Steps 11-23 of clause AC.7.2.3, or steps 7-17 of clause AC.7.2.1 applies, depends on type of application data channel decided in step 5.

19.	The application data channel(s) are established. In the network centric with MF rendering mode, the application data channel terminated at MF. In the network centric with DC AS rendering mode, the MF proxies the media transparently to the DC AS.

20.	The IMS AS initiates the media re-negotiation between UE1, UE2 and MF, which anchors UE1 and UE2's audio/video to MF.

Based on the type of network rendering mode decided in step 9, either option 1 (MF rendering mode, steps 21a-26a) or option 2 (DC AS rendering mode, steps 21b-26b) is performed in steps 22-26.

21.	The MF or DC AS downloads UE1's avatar representation using the Resource URL in the avatar media specification data sent from DCSF in step 10.

22.	 The UE1 sends avatar metadata (e.g. facial expression) through the ADC established in step 19 and/or audio/video through RTP channels established in step 20.

23.	Optionally, the UE2 may send avatar metadata (e.g. facial expression) through the ADC established in step 19 and/or audio/video through RTP channels established in step 20.

24.	MF or DC AS perform avatar animation based on the avatar metadata received from application data channel and/or audio/video. MF should support different media processing according to the avatar type, e.g., animation with no rendering for 2D avatar, animation and rendering for 3D avatar.

25-26.	MF sends the animated video stream to UE2, and sends the animated video stream to UE1 if required.

The above procedures also apply for the transition from audio/video communication between two users to avatar communication. In an ongoing audio/video communication, one of UEs decides to switch the communication to avatar mode, it will interact with the IMS Core and peer UE to establish application data channel for the avatar communication. Then two UEs will exchange avatar media; and the original video media between two UEs may be stopped.

# >>>>END OF CHANGES<<<<