3GPP TSG-WG SA2 Meeting #168	S2-2503005

# April 7th-11th, 2025, Goteborg, Sweden	            	                             (revision of S2-2XXXXX)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 6173 | rev | - | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | 23.501 Support of uplink rate control for QoS flows at RAN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Lenovo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM_Ph2 |  |  |  |  |  | Date: |  |  | 2025-03-25 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Based on the latest LS in from RAN2 (S2-2502820/R2-2501347), RAN 2 assumes that the gNB receives the QoS flow information from the CN that specifying which QoS flows are subject to uplink rate control. Since not all the QoS flows of all types of XR applications will have the capability of adjusting their codec bit rate and encoding parameters, AF providing an uplink rate control indication can make NG-RAN be aware of which QoS flows are applicable for adjustment and therefore act upon, e.g., providing a recommented bit rate. Based on this, it is essential to provide an explicit indication associated with the QoS flows to the NG-RAN, rather than based on RAN own implementation to adjust or decide. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the descriptions of provisioning the uplink rate control indication associated with the QoS flows from CN to RAN. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The latest agreement of the uplink rate control for XR cannot be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.37.1, 5.37.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * 1st change * * * *

5.37	Support for high data rate low latency services, eXtended Reality (XR) and interactive media services

### 5.37.1	General

This clause provides an overview of 5GS functionalities for support of XR services (AR/VR applications) and interactive media services that require high data rate and low latency communication, e.g. cloud gaming and tactile/multi-modal communication services according to service requirements documented in TS 22.261 [2]. The standardized 5QI characteristics for such interactive services are provided in Table 5.7.4-1 and TSCAI is used to describe the related traffic characteristics as defined in clause 5.27.2. Further enhancements for these interactive media services are as follows:

-	The 5GS may support QoS policy control for multi-modal traffic, see clause 5.37.2.

-	The 5GS may support network information exposure which can be based on ECN markings for L4S, see clause 5.37.3 or 5GS exposure API, see clause 5.37.4.

-	The 5GS may support PDU Set based handling including PDU Set identification and marking, see clause 5.37.5.

-	The 5GS may ensure that the UL and DL packets together meet the requested round trip delay and also update the delay for UL and DL considering QoS monitoring results, see clause 5.37.6.

-	The 5GS may perform per-flow Packet Delay Variation (PDV) monitoring and policy control according to AF provided requirements, see clause 5.37.7.

-	The 5GC may provide traffic assistance information to the NG-RAN to enable Connected mode DRX power saving, see clause 5.37.8.

-	The 5GS may consider dynamically changed traffic characteristics for better resource management, see clause 5.37.10.

-	The 5GS may support traffic identification for multiplexed media flows in the same transport layer connection, see clause 5.37.11.

-	The 5GC may perform PDU Set Importance based transport level packet marking, see clause 5.8.2.7.

-	The 5GC may provide the Multi-modal Service ID to NG-RAN, see clause 5.37.2.

-     The 5GC may provide the uplink rate control support indication to NG-RAN, see clause 5.37.X.

NOTE:	Home-routed roaming deployments cannot always support the low latency communication requirements of XR services or interactive media services.

# * * * * 2nd change * * * *

### 5.37.X	Support of uplink rate control at NG-RAN

When congestion happens at NG-RAN for uplink XR traffic sent from UE, NG-RAN can perform the data rate control by providing a recommendated reduced data rate to the UE, for the corresponding QoS Flows in the uplink direction, based on the awareness at NG-RAN that the QoS flows are applicable for uplink rate control, e.g., adjusting the data rate.

To support the uplink rate control of XR service at the NG-RAN for handling the congestion of the uplink traffic, AF may explicitly send an uplink rate control indication for the data flows that are applicable for the data rate adjustment to the 5GS, via the Nnef_AFsessionwithQoS service. Then, the PCF may include it into the PCC rules, either based on local operator configuration, or based on the received information from AF, and send it to SMF, as described in the clause 6.1.3.27.X of TS 23.503 [45]. SMF will send the uplink rate control indication associated with each QoS flow to the NG-RAN via N2 SM message when establishing and/or updating the corresponding QoS Flows.

After receiving the indication from SMF, NG-RAN can apply the uplink data rate adjustment for the applicable QoS Flows as indicated by the AF.

During the handover procedure, the Uplink Rate Control Indication is provided to the Target NG-RAN per QoS Flow.

# * * * * End of changes * * * *