SA WG2 Meeting #168	S2-2503012

# Goteborg, Sweden, 7-11 April, 2025                            (revision of S2-25xxxxx)

Source: 	CATT

Title: 	AIoT Reader Selection update

Document for: 	Approval

Agenda Item: 	19.14.2

Work Item / Release:	AmbientIoT / Rel-19

Abstract of the contribution: This paper proposes update on AIoT Reader Selection to TS 23.369.

1	Proposal

RAN3 informed SA2 about their AIoT progress on Reader Selection (yellow highlighted) in R3-250905 as followings.

RAN3 would like to inform SA2 and SA5 about the following RAN3 progress related to Architecture and OAM aspects:

- A-IoT RAN node is an aggregated gNB. And this gNB may or may not only provide the A-IoT radio.

- Support both the direct and indirect communication options between the gNB and the AIOTF in Topology 1.

- Working assumption: Including AIOTF information containers in NGAP, rather than introducing new protocol carried by NGAP.

- It is FFS whether to define a new A-IoT Area, or use Tracking Area dedicated for A-IoT, to support e.g., the selection of gNBs by the AIOTF.

- AIOTF obtains the A-IoT RAN information (supported Area, served reader ID list) via OAM configuration, FFS on RAN3 signalling for the A-IoT RAN information.

- AIOTF may also be aware of the other A-IoT RAN information (location of reader) via OAM configuration, FFS on RAN3 signalling for the other A-IoT RAN information...

- In Inventory Request, gNB receives the requested Service Area Information (encoded as Area and/or reader ID list) from AIOTF.

- The gNB always sends the A-IoT device’s location at reader ID granularity to the AIOTF in Inventory Report.

- “Global gNB ID + Reader index" is used to uniquely identify the Reader globally.

It is proposed to update the AIoT RAN Reader Selection based on the above RAN3 agreements.

# * * * * First change* * * *

5.3	AIoT RAN Reader Selection

AIoT RAN Reader Selection functionality is supported by AIOTF.

Editor’s note:	How AIoT RAN Reader Selection functionality is supported by AIoT RAN node is to be defined by RAN3.

The AIOTF selects AIoT RAN Reader based on the AIoT RAN information (supported AIoT Area, served Reader ID list) obtained via OAM and the target area information (e.g. a TA list) received from NEF/AF.

NOTE x:	"Global gNB ID + Reader index" is used to uniquely identify the Reader globally.

For Direct Connectivity Architecture, the AIOTF may provide Reader ID list and/or AIoT Area to the AIoT RAN node for AIoT RAN Reader selection. For Indirect Connectivity Architecture, the AITOF may provide AIoT RAN Node ID, Reader ID list and/or AIoT Area to the AIoT RAN node via the selected AMF for AIoT RAN Reader selection

Editor’s note:	Whether to define a new AIoT Area, or use Tracking Area dedicated for A-IoT is to be decided by RAN3.

NOTE:	AIoT RAN information is not exposed to the AF.

The AIOTF may also use the last serving Reader to assist with determining which Readers to use for an AF request targeting for a specific AIoT Device.

# * * * * End of Changes * * * *