3GPP TSG-SA WG2 Meeting #168	S2-2503102

Goteborg, Sweden, April 07 – 11, 2025

Source:	OPPO

Title:	Key issue for WT#1: Data collection UE authorization and availability verification

Document for:	Approval

Agenda Item:	20.3.1

Work Item / Release:	FS_AIML_CN_Ph2 / Rel-20

Abstract of the contribution: The contribution proposes the key issue for WT#1 on how a UE is authorised by the network and how the network gets verification from the UE on its availability to perform UE data collection over UP.

- Discussion

In the approved FS_AIML_CN_Ph2 SID, WT#1 is defined as follows:

WT#1: Study whether and how to support the standardized transfer of standardized data over UP for UE data collection to meet requirements for AI/ML for NR air interface operation with UE-side model training.

Note 1: The WT#1 scope is based on UP Data Collection for Option2 documented in RAN LSs RP-243316 and RP-242389.

Note 2: The requirements are elaborated in LSs RP-242389 and R2-2411152 which will be considered during the study phase.

Note 3: WT#1 impact on user consent and privacy will be addressed in cooperation with SA3.

To obtain measurement data for AI/ML model training, the network may select one or multiple UEs for RRC measurements. And then the network collects data from these UEs. However, not all the UEs capable for data collection are trusted by the network to peform data collection. Restricting data collection to network-trusted and authorized UEs is critical for ensuring security, reliability, and compliance; otherwise, the following problems are identified:

- Untrusted UEs could inject false or manipulated data into the network, poisoning AI/ML models and degrading network performance (e.g., falsifying beam measurements to disrupt beamforming).

- Unauthorized UEs might intercept sensitive data (e.g., user locations, network configurations) during collection, exposing vulnerabilities.

- Trusted UEs are validated to ensure their sensors/hardware meet accuracy standards. Untrusted UEs may provide noisy or inconsistent data.

- Authorized UEs are selected based on predefined criteria (e.g., geographic distribution, mobility states), ensuring balanced datasets. Unauthorized UEs could skew data (e.g., over-representing static devices).

- Untrusted UEs might flood the network with unnecessary data, congesting the user plane (UP) or control plane (CP).

Observation 1: Authorizing only trusted UEs for data collection safeguards security, privacy, and data quality, ensuring AI/ML models are trained on reliable inputs.

Additionally, requiring explicit verification from a UE (or its user) before enabling data collection is crucial for ethical, legal, and operational reasons:

- Collected data (e.g., location, mobility patterns, device usage) may reveal personal information. Consent ensures compliance with privacy laws

- Explicit consent is mandatory under many jurisdictions for collecting, processing, or sharing user/device data.

- Users have the right to know what data is collected, how it will be used, and who can access it.

- Mandatory consent fosters trust between users and operators, reducing backlash or reputational harm.

- Data collection (e.g., frequent measurements) drains battery and consumes bandwidth. Consent allows users to avoid unintended resource costs.

Observation 2: Mandating UE availability verification aligns with ethical AI practices, legal obligations, and user-centric service design. It balances the network’s need for high-quality data with individual rights to privacy and control, ensuring sustainable and responsible AI/ML deployments.

This paper defines the key issue for WT#1 on how a UE is authurised by the network and how the network get concent from the UE to perform UE data collection over UP.

- Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-04.

* * * Start of Change (all new) * * * *

5.x	Key Issue #x: Data collection UE authorization and availability verification

5.x.1	Description

To acquire measurement data for AI/ML model training, the network may select one or multiple UEs to perform RRC measurements, after which it aggregates the collected data. However, not all UEs capable of data collection are deemed trustworthy by the network for this task. Restricting participation to network-authorized and verified UEs is essential to ensure security, data integrity, and data quality.

To align with ethical AI practices, legal obligations, and user-centric service design and balances the network’s need for high-quality data, ensuring sustainable and responsible AI/ML deployments, a mechanism for data collection UE availability verification is required to be developed.

The following aspects need to be studied for the potential solutions:

- Study the principle, and identify the necessary and additional information for data collection authorisation and configuration/provisioning to the UE

- Study the procedures related to data collection authorisation and policy/parameter configuration/provisioning to a UE

- Identify the the necessary and additional information for data collection availability verification

- Study the procedures related to data collection availability verification

* * * End of Change * * * *