3GPP TSG-WG SA2 Meeting #168	S2-2503107

Goteborg, SE, 7th April – 11th April, 2025	(revision of S2-250xxxx)

Source:	NEC

Title:	Discussion on Rel-20 AIoT

Document for:	Approval

Agenda Item:	30.6

Work Item / Release:	Rel-20

Abstract: Discuss the scope and time plan of Rel-20 AIoT.

1. Introduction

In SA#107, the Rel-19 Ambient IoT(AIoT) WID (SP-250377) was approved with the objective as follows:

# 4 Objective This work item is to specify the architecture support for Ambient IoT Device and Ambient IoT Services, as per conclusions documented in TR 23.700-13 (clause 8), focusing on Device 1 in Topology 1. General scope of this work item is aligned with RAN WID (RP-243326), wherever applicable. Specifically, the work item objectives are: - WT#1 Architecture to support Ambient IoT: - Architecture to Support Device 1 in Topology 1 (Direct interface option and Indirect interface option). - WT#2 Ambient IoT Device management: - Identifier of Ambient IoT Device (i.e. permanent device ID, temporary device ID). - AIoT Device Data Profile Management - WT#3 Ambient IoT Service enabler for Inventory and Command: - Exposure for AIOT services to AF. - Data transfer between AIoT Device/RAN reader and AIOTF. NOTE: Alignment with SA3 on security aspects is expected.

Rel-19 AIoT WID only focuses on Device 1 in Topology 1, the other device type and topology are expected to be included in Rel-20 AIoT.

2. Discussion

2.1 Device Type 2

Device 2 is not included in Rel-19 AIoT WI, which contains two sub-types: device 2a and device 2b.

- Device 2a can be regarded as an enhancement of device 1 with ability to reflect carrier wave with amplification in the device.

- Device 2b has the ability to generate independent signals without requirement of carrier wave. Device 2b is expected to have larger coverage than device 2a and device 1, and to benefit outdoor use case.

Proposal 1: Device 2(2a&2b) should be in scope of Rel-20 AIoT, and device 2b should be prioritized.

2.2 Topology 2

The topology 2 is shown by Fig.1, which has been studied in Rel-19 AIoT study phase in SA2 TR 23.700-13.

Figure 1: Topology 2 structure

Due to limited time, topology 2 is excluded from Rel-19 AIoT WI, and is expected in Rel-20 AIoT WI. One benefit of topology 2 is extended coverage by the help of intermediate node. Since AIoT device is cost-sensitive and power-limited, the link between AIoT device to reader can sustain a maximum distance around tens of meters. However, for outdoor usage, the coverage requires in units of km. Therefore, an intermediate node is beneficial to extend the coverage distance from tens of meters to several kms.

Proposal 2: Topology 2 should be supported in Rel-20 AIoT.

2.3 Expected time scale

SA2 has studied Topology 2 and Device 2a/2b in Rel-19 TR23.700-13, although they are not specified in Rel-19, the study can be continued and concluded for the Rel-20 normative work. Therefore, a short study phase with 6 months and a following normative phase are expected.

Proposal 3: SA2 allocates 6 months for Rel-20 AIoT SI on Topology 2, Device 2a/2b, and potential enhancements of Device 1 in Topology 1. The time for a following-up normative phase can be 6 or 9 months.

3. Proposal

Proposal 1: Device 2(2a&2b) should be in scope of Rel-20 AIoT, and device 2b should be prioritized.

Proposal 2: Topology 2 should be supported in Rel-20 AIoT.

Proposal 3: SA2 allocates 6 months for Rel-20 AIoT SI on Topology 2, Device 2a/2b, and potential enhancements of Device 1 in Topology 1. The time for a following-up normative phase can be 6 or 9 months.

4. Reference

[1] SP-250377, New WID on Architecture support of Ambient power-enabled Internet of Things, Rel-19 SA2 WID

[2] TR 23.700-13, Study on Architecture support of Ambient power-enabled Internet of Things, SA2

[3] TR 38.848, Study on Ambient IoT (Internet of Things), RAN Plenary

[4] TR 38.769, Study on solutions for ambient IoT (Internet of Things), RAN1/2/3/4