3GPP TSG-SA WG2 Meeting #168	S2-2503166

Goteborg, Sweden, April 07 – 11, 2025

Source:	Samsung

Title:	Key issue for WT#2: UE-SAT-UE communication enhancement for NR NTN NGSO constellation with feeder link available

Document for:	Approval

Agenda Item:	20.1.1

Work Item / Release:	FS_5GSAT_Ph4-ARC / Rel-20

Abstract of the contribution: The contribution proposes the key issue for UE-SAT-UE communication enhancement for supporting non-IMS services.

1. Discussion

The SID on Integration of satellite components in the 5G architecture Phase 4 (SP-250400) includes the following WTs:

-	WT-2: UE-SAT-UE communication enhancement for 5GS under NR NTN NGSO constellation with feeder link available.

-	WT-2.1 Support of UE-SAT-UE communication for roaming case considering the requirements for lawful interception.

NOTE 6:	The scenario will be restricted to two roaming UEs served by same VPLMN and HPLMN.

NOTE 7:	GSMA should be consulted with regard to the roaming requirement for UE-satellite-UE communication, and then further clarification may be needed to specify the detailed roaming scenario..

-	WT-2.2 UE-SAT-UE communication via UPF only onboard satellite for non-IMS services, e.g. a 5G-LAN service with IP and Ethernet PDU Session type.

NOTE 8: Whether existing mechanism can be reused to support WT-2 needs to be checked.

NOTE 9: No UE impact is expected for WT-2.

Therefore, it is proposed to study the following KI for further enhancement on integration of satellite components in GEO NB IoT NTN connecting to EPC.

2. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-19.

* * * Start of Change * * * *

5.x	Key Issue #x: UE-SAT-UE communication enhancement for supporting non-IMS services

5.x.1	Description

-	How to support UE-SAT-UE communication via UPF only onboard satellite for non-IMS services (e.g. 5G-LAN services with IP and Ethernet PDU Session type).

NOTE 1:	Whether existing mechanism can be resued will be considered in this KI.

NOTE 2:	Any UE impact will not be considered in this KI.

* * * End of Change * * * *